| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0527 |
| --- | --- |

| Title: | AHG12: Comments on Tiles and Flexible Tile Partitioning |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wade Wan Minhua Zhou Tim Hellman Brian Heng Peisong Chen | Email: | wade.wan@broadcom.com minhua.zhou@broadcom.com tim.hellman@broadcom.com brian.heng@broadcom.com peisong.chen@broadcom.com |
| Source: | Broadcom Inc. |  |  |

_____________________________

# Abstract

The following document provides comments on tiles and more specifically, the flexible tile partitioning feature discussed in recent meetings.  The authors assert that the idea of generalizing tile partitioning to allow for tiles being split into units smaller than a CTU may sound simple conceptually but has a significant impact to implementations and wish to express significant concern with the implications on frame compression, virtual pipeline data unit processing rate, in-loop filters and on-chip memory access overhead.  Therefore, it is suggested that the topic be more carefully studied, discuss whether tile functionality without splitting into smaller CTUs is already sufficient for applications such as 360 and frame packing video which may not be as prevalent as traditional 2D video and/or consider alternative techniques rather than flexible tile partitioning.

# Introduction

A more flexible form of tile partitioning that allows tiles to be split into units smaller than a CTU has been presented in JVET-K0155 [1] and JVET-L0359 [2].  It was asserted in JVET-L0359 that an important motivation for this feature was load balancing but it was unclear whether that assertion was generally accepted or interesting to the group.  It was also asserted that this feature was useful for 360 video and/or frame packing video to accommodate different rectangular face sizes. These use cases seem narrowly focused and not likely as prevalent as traditional 2D video, so perhaps other techniques such as defining active region(s) and/or cropping window(s) could facilitate the same functionality and it is asserted they would which have less implementation impact.  The remainder of this document will focus on the implications of this flexible tile partitioning approach that were not as apparent to the authors during the meeting but became more apparent after some internal analysis and discussion with our design teams.

# Review of HEVC Tile Constraints

The tile concept was discussed extensively during HEVC standardization with concerns expressed about its impact to overall throughput and testing which the authors can confirm did impact its HEVC product development and implementation.

This document notes that several constraints were imposed to HEVC tiles to specifically try to alleviate the implementation concerns.  They are discussed below to revisit why those constraints were included and demonstrate how generalizing tile partitioning to units smaller than CTUs has the potential to not only make the same issues discussed before worse but also create new issues that will be significant for VVC implementations.

The following properties were defined for HEVC tiles:

- HEVC Tiles were defined on CTU boundaries as tiles were defined to be an integer number of CTUs.

- The size of HEVC CTUs were chosen by the encoder and allowed to range from 64x64 to 16x16 based on the log2_diff_max_min_luma_coding_block_size element.

- Note that level constraints do not allow 16x16 CTBs for large pictures

  - In A.4.1 clause (d) of [3]: For level 5 and higher levels, the value of CtbSizeY shall be equal to 32 or 64.

Since VVC standardization is still ongoing, it is currently unclear or at least not widely discussed what the range of VVC CTUs permitted will be but currently it appears to be leaning towards a maximum of 128x128 and (perhaps?) a minimum of 16x16.  The authors believe that the idea of flexible partitioning is perhaps partially motivated by the fact that VVC CTUs can go up to 128x128 but it should be noted that the smaller 64x64 CTU size can be chosen by the encoder to get the same tile resolution as in HEVC and therefore there is no loss in functionality with VVC.

On a side note, we list HEVC tiles property #3 above which was adopted because of the concern that (small) 16x16 CTBs would have on throughput for larger picture sizes because at a high level, it forces a decoder to handle the 16x16 CTB one at a time rather than process a larger unit.  The authors can confirm that this affected our present decoder implementation, will affect our planned future VVC implementation and can imagine that this concern would apply in general to other implementations.  Therefore, a similar constraint is needed for VVC.

# CU Tile Boundaries

The authors assert that there are several problems with allowing tile boundaries at CU boundaries rather than CTU boundaries which include issues of availability (location and timing) causing the associated processing to become problematic and introducing many corner case edge conditions.

Figure 1 shows an example of a flexible tile partitioning to aid the discussion in this document.

Figure 1: Flexible Tile Partitioning Example

Figure 2 shows a zoomed in view of the upper right CTU of the example in Figure 1 where this CTU is composed of data from four different tiles.

Figure 2: Upper Right CTU of Flexible Tile Partitioning Example

Many encoder and decoder implementation will need to process one CTU at a time.  As was mentioned in previous meetings but perhaps not rigorously studied or not as readily apparent to some without a diagram, this figure shows a direct impact to the throughput of pipelined HW decoders.  More specifically, this CTU would need to be processed at four separate times to get the different portions associated with each tile.

Another issue that will affect both encoders and decoders is how each of these portions is “stitched back together”.  There are issues with memory addressing and how memory compression is performed when “partial CTUs” are decoded and have to be “kept waiting”.  Especially with large picture/video sizes, many devices have to implement some form of memory compression to reduce the memory bandwidth impact when writing into SDRAM.  Efficiency of this compression is directly related to the region size so there is less benefit to very small sizes.  As an example, if the size used is 64x64 and the 64x64 region being decoded happens to contain multiple pieces due to flexible tiling, it is not clear how one puts the pieces back together in memory after compression.

It is also unclear if/when in-loop filtering is required but it appears this will also require more buffering to support it.  The authors recognize there may be certain use cases that do not require in-loop filtering and then this could be disabled but can imagine other use cases where one will want to enable in-loop filtering to avoid bad visual artifacts.

Finally, with blocks no longer aligned on 64x64 boundaries, the availability of the information required to decode an element could be in another tile.  It is the opinion of the authors that this is significantly more complicated and will also create many corner case edge conditions which will result in interoperability issues.

# Conclusion

The authors suggest that the flexible tile functionality be more carefully studied and examine whether the general tiles concept is already sufficient for VVC applications.  They assert that generalizing the tile concept to allow for tiles being split into units smaller than a CTU has significant implementation challenges and suggest the additional complexity should be directed towards other tools that are more broadly applicable.

# References

[1] Y. Yasugi, T. Ikai, “AHG12: Flexible Tile Partitioning” JVET-K0155, Jul. 2018, Ljubljana, SI.

[2] Y. Yasugi, T. Ikai, “AHG12: Flexible Tile Partitioning” JVET-L0359, Oct. 2018, Macau, CN.

[3] “High efficiency video coding:, ITU-T Rec. H.265 and ISO/IEC 23008-2, ITU-T and ISO/IEC JTC 1, Feb. 2018.

# Patent rights declaration(s)

Broadcom Inc. does not have current or pending patent rights relating to the technology described in this contribution.