| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0530 |
| --- | --- |

| Title: | AHG 12: On signalling of tiles |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Muhammed Coban Marta Karczewicz | Tel: Email: | +1 858 658 3973 mcoban@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribuiton proposes a hierarchical rectangular tile set and tile configuration signalling that can provide efficient signalling for existing VVC tile structure as well as flexible tile partitions.

# Introduction

VVC supports HEVC style tile structures that span across entire width and height of a picture. Flexible tile structures with tile sizes non-integer multiple of CTB sizes have been proposed []. Tile sets can be used to describe region-of-interest in a picture in combination with flexible tile structures. Each tile set can contain multiple uniform size tiles for parallelization. Combination of flexible higher-level tiles (tile set) and lower level uniform tiles would introduce signalling of non-uniform size tiles for every tile. This combination can be efficiently signalled in hierarchical structure by first signalling the tile-set layout using the existing tile  signalling scheme and then signalling tile structure within each column and row of tile sets.

# Proposal

Tile layout is signalled in PPS in hierarchical fashion. First rectangular tile set configuration is signalled. Tile set boundaries span across the width and height of a picture. Then for each tile set column and row, tile configuration is signalled as shown in Figure 1.

Figure 1:Tile set, tile and CTB layout with scan order.

The tile sets are in rectangular shape. Tiles within tile sets and tile sets within a picture are coded in raster scan order. Loop filtering enabling and disabling is signalled at tile set level. Treatment of tile sets as sub-pictures (i.e. picture boundary padding) is also signalled at this level.

## Signalling of tiles sets and tiles in picture parameter set

The changes with respect to [1][2] are highlighted.

#### Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| init_qp_minus26 | se(v) |
| transform_skip_enabled_flag | u(1) |
| single_tile_set_in_pic_flag | u(1) |
| if( !single_tile_set_in_pic_flag ) { |  |
| tile_unit_size_idc | ue(v) |
| num_tile_set_columns_minus1 | ue(v) |
| num_tile_set_rows_minus1 | ue(v) |
| uniform_tile_set_spacing_flag | u(1) |
| if( !uniform_tile_set_spacing_flag ) { |  |
| for( i = 0; i < num_tile_set_columns_minus1; i++ ) |  |
| tile_column_set_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_set_rows_minus1; i++ ) |  |
| tile_row_set_height_minus1[ i ] | ue(v) |
| } |  |
| sub_picture_tile_sets_flag | u(1) |
| if( !sub_picture_tile_sets_flag ) |  |
| loop_filter_across_tile_sets_enabled_flag | u(1) |
| } |  |
| single_tile_in_tile_set_flag | u(1) |
| if( !single_tile_tile_set_flag ) { |  |
| for( i = 0; i < num_tile_set_columns_minus1 + 1; i++ ) { |  |
| num_tile_columns_minus1 | ue(v) |
| uniform_tile_column_spacing_flag | u(1) |
| if( !uniform_tile_column_spacing_flag ) |  |
| for( j = 0; j < num_tile_columns_minus1; j++ ) |  |
| tile_width_minus1[ i * (num_tile_columns_minus1 + 1) + j] | ue(v) |
| } |  |
| for( i = 0; i < num_tile_set_rows_minus1 + 1; i++ ) { |  |
| num_tile_rows_minus1 | ue(v) |
| uniform_tile_row_spacing_flag | u(1) |
| if( !uniform_tile_row_spacing_flag ) |  |
| for( j = 0; j < num_tile_rows_minus1; j++ ) |  |
| tile_height_minus1[ i * (num_tile_rows_minus1 + 1) + j] | ue(v) |
| } |  |
| } |  |
| cu_qp_delta_enabled_flag | u(1) |
| if( cu_qp_delta_enabled_flag ) |  |
| diff_cu_qp_delta_depth | ue(v) |
| pps_cb_qp_offset | se(v) |
| pps_cr_qp_offset | se(v) |
| pps_slice_chroma_qp_offsets_present_flag | u(1) |
| deblocking_filter_control_present_flag | u(1) |
| if( deblocking_filter_control_present_flag ) { |  |
| deblocking_filter_override_enabled_flag | u(1) |
| pps_deblocking_filter_disabled_flag | u(1) |
| if( !pps_deblocking_filter_disabled_flag ) { |  |
| pps_beta_offset_div2 | se(v) |
| pps_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

single_tile_set_in_pic_flag equal to 1 specifies that there is only one tile set in each picture referring to the PPS. single_tile_set_in_pic_flag equal to 0 specifies that there is more than one tile in each picture referring to the PPS.

It is a requirement of bitstream conformance that the value of single_tile_set_in_pic_flag shall be the same for all PPSs that are activated within a CVS.

tile_unit_size_idc specifies the size of a tile unit block in luma samples.

num_tile_set_columns_minus1 plus 1 specifies the number of tile set columns partitioning the picture. Num_tile_set_columns_minus1 shall be in the range of 0 to PicWidthInCtbsY – 1, inclusive. When not present, the value of num_tile_set_columns_minus1 is inferred to be equal to 0.

num_tile_set_rows_minus1 plus 1 specifies the number of tile set rows partitioning the picture. num_tile_set_rows_minus1 shall be in the range of 0 to PicHeightInCtbsY − 1, inclusive. When not present, the value of num_tile_set_rows_minus1 is inferred to be equal to 0.

uniform_tile_set_spacing_flag equal to 1 specifies that tile set column boundaries and likewise tile row set boundaries are distributed uniformly across the picture. uniform_tile_set_spacing_flag equal to 0 specifies that tile set column boundaries and likewise tile set row boundaries are not distributed uniformly across the picture but signalled explicitly using the syntax elements tile_column_set_width_minus1[ i ] and tile_row_set_height_minus1[ i ]. When not present, the value of uniform_tile_set_spacing_flag is inferred to be equal to 1.

tile_column_set_width_minus1[ i ] plus 1 specifies the width of the i-th tile set column in units of tile unit blocksCTBs.

tile_row_set_height_minus1[ i ] plus 1 specifies the height of the i-th tile set row in units of tile unit blocksCTBs.

sub_picture_tile_sets_flag equal to 1 specifies tile set boundaries will be treated as picture boundaries.

loop_filter_across_tiles_enabled_flag equal to 1 specifies that in-loop filtering operations may be performed across tile boundaries in pictures referring to the PPS. loop_filter_across_tiles_enabled_flag equal to 0 specifies that in-loop filtering operations are not performed across tile boundaries in pictures referring to the PPS. The in-loop filtering operations include the deblocking filter, sample adaptive offset filter, and adaptive loop filter operations. When not present, the value of loop_filter_across_tiles_enabled_flag is inferred to be equal to 1.

single_tile_in_tile_set_flag equal to 1 specifies that each tile set contains a single tile.

num_tile_columns_minus1 plus 1 specifies the number of tile columns partitioning the tile set. Num_tile_columns_minus1 shall be in the range of 0 to PicWidthInCtbsY – 1, inclusive. When not present, the value of num_tile_columns_minus1 is inferred to be equal to 0.

uniform_tile_column_spacing_flag equal to 1 specifies that tile column boundaries are distributed uniformly across the tile set. uniform_tile_column_spacing_flag equal to 0 specifies that tile column boundaries are not distributed uniformly across the tile set but signalled explicitly using the syntax elements

num_tile_rows_minus1 plus 1 specifies the number of tile rows partitioning the tile set. num_tile_rows_minus1 shall be in the range of 0 to PicHeightInCtbsY − 1, inclusive. When not present, the value of num_tile_rows_minus1 is inferred to be equal to 0.

uniform_tile_row_spacing_flag equal to 1 specifies that tile row boundaries are distributed uniformly across the tile set. uniform_tile_row_spacing_flag equal to 0 specifies that tile row boundaries are not distributed uniformly across the tile set but signalled explicitly using the syntax elements

tile_column_width_minus1[ i ] plus 1 specifies the width of the i-th tile column in units of CTBs

tile_row_height_minus1[ i ] plus 1 specifies the height of the i-th tile row in units of CTBs

# Conclusion

This document proposes modifications to tile signaling in VVC. It proposes a hierarchical rectangular tile set and tile configuration signalling that can provide efficient signalling for existing VVC tile structure as well as flexible tile structures.

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 3)”, JVET-L1001-v7, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Macao, CN, October 2018.

- Y.-K Wang, Hendry, R. Sjoberg, S. Deshpande, “Spec text for the agreed starting point on slicing and tiling”, JVET-L0686, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Macao, CN, October 2018.

- Y. Yasugi, T. Ikai, “AHG12: Flexible Tile Partitioning”, JVET-M0066, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Marrakech, MA, January 2019.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).