| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0579 |
| --- | --- |

| Title: | On Frame Rate Support and Extraction in VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Andrew Segall1 Sachin Deshpande1 Miska M. Hannuksela2 | Tel: Email: | +1-360-817 8487 asegall@sharplabs.com sdeshpande@sharplabs.com miska.hannuksela@nokia.com |
| Source: | 1Sharp Laboratories of America. 2Nokia |  |  |

_____________________________

# Abstract

This contribution considers design requirements for supporting multiple frame rates within the VVC codec.  Multiple frame rate support has been included in previous video codecs, such as HEVC and AVC.  However, it is asserted that the application space for using multiple frame rates has changed since those designs.  Specifically, it is asserted that the temporal scalability of AVC and HEVC has been used for bit-rate adaptation for low and ultra-low bit-rates, unequal error protection, and trick modes. While the authors of this document believe that these are still relevant reasons for temporal scalability, the goal of this contribution is to highlight the need to also support applications requiring high quality, high frame rate renditions.  In these applications, temporal scalability is used to provide backwards compatibility to non-high frame rate equipment.  And, unlike historical designs, all extractable frame rates must be of high quality.  It is asserted that this precludes simple frame dropping schemes, as these schemes can introduce temporal judder and other artifacts.

# Introduction

Increasing frame rates are one approach to increase the quality of content in high value, consumer applications.  The benefit of higher frame rates is largely due to the shorter aperture that is used during acquisition, which reduces motion blur and leads to higher spatial resolution captures in moving scenes.  While this form of acquisition process can be found in several applications, one particular application of interest is sports broadcasting.  Of course, this content is known to be of extreme high value to consumers and, thus, the consumer entertainment community.

Support for the extraction of different frame rates has been a part of previous video codec designs, and it is anticipated by the authors that it would also be included in the first version of the VVC standard.  Historically, this support allowed for the extraction of a sub-set of frames within a video bit-stream.  While this continues to be desirable, this historical design was applicable for the extraction of the sub-set of frames in low and/or ultra-low bit-rate scenarios and has also been used for unequal error protection and trick modes.  For these usages, high quality, high frame rate rendition was not necessary.

In this document, we propose formal study of extracting multiple frame rates from a bit-stream within the context of modern, high frame rate video applications.  Unlike previous approaches, the extracted frames are expected to be of high visual quality and likely corresponding to broadcast quality and/or professional quality scenarios.  This includes both fidelity to the original sequence as well as an absence of temporal artifacts – such as temporal judder or temporal strobing.

In the remainder of this document, we summarize some recent work to address the support for higher frame rate video in the broadcast arena.  We then provide our recommendations for next steps.

# Relevant Work

Several broadcast standards groups have considered how to best support the extraction of high quality sequences at different frame rates.  For example, the ATSC 3.0 standard includes support for signaling meta-data to allow the extraction of both standard frame rate (e.g., 50/60Hz) and high frame rate (e.g., 100/120 Hz) video from a signal bit-stream.  The approach consists of combining the temporal sub-layer concept in HEVC with a post-processing step that is controlled by meta-data provided to the receiver.  Additionally, the DVB has included higher frame rates in their Phase 2 requirements and goals.  In the rest of this section, we provide a brief introduction to some of the related work.

## ATSC 3.0

A visual example of one approach considered in the ATSC 3.0 system is shown in Figure 1 and Figure 2, which are reproduces from the ATSC Standard [1].  As can be seen in Figure 1, the process begins by creating a high frame rate and a standard frame rate sequence, and then embedding these sequences into a multiple frame rate bit-stream.  In the Figure, the creation of the standard frame rate sequence is accomplished by taking a weighted average of multiple frames in the high frame rate sequence.  Then, in the embedding process, the standard frame rate sequences are assigned to temporal sub-layer 0, while the higher frame rate sequences are assigned to temporal sub-layer 1.  (Please note that any method for capturing and/or creating the standard dynamic range sequence would be allowed.)

120 fps stream

60 fps stream

Σ

Multiple frame rate temporal filtering stream

*w1

Temporal sub-layer 1

Temporal sub-layer 0

Temporal sub-layer 1

Temporal sub-layer 0

*w2

*w1

*w2

Σ

Figure 1 – Visual example of creating a multiple frame rate bit-stream to support the delivery of high quality video at both the lower and higher frame rates.  The design is created to work with existing standards that did not support high quality, frame rate scalability natively.  Taken from the ATSC 3.0 standard [1].

At the decoder, the standard frame rate is extracted using the temporal sub-layer identifiers.  Specifically, the standard frame rate can be extracted by discarding temporal sub-layer 1 and decoding temporal sub-layer 0.  The result is a high quality rendition of the original sequence, which is devoid of temporal judder and temporal strobing due to the blending (or alternative) used to create the standard frame rate video.

Alternatively, decoding the higher frame rate consists of two steps.  First, both temporal sub-layer 0 and temporal sub-layer 1 are decoded.  Then, the temporal locations corresponding to the standard frame rate video (temporal sub-layer 0) are further processed with a weighting operation to create a high frame rate frame from the current, standard frame rate frame and temporally adjacent, higher frame rate frames.  The process is summarized in Figure 2.

Multiple frame rate temporal filtering stream

Recovered

120 fps stream

F0’ = ( FTSL0 – F-1) * k + FTSL0

One frame buffer, subtract, shift and add

F0’ = ( FTSL0 – F-1) * k + FTSL0

k = temporal_filter_w1/temporal_filter_w2.

Figure 2 – Visual example of creating a high frame rate sequence from the multiple frame rate bit-stream to support the delivery of high quality video at the higher frame rate.  The design is created to work with existing standards that did not support high quality, frame rate scalability natively.  Taken from the ATSC 3.0 standard [1].

## DVB UHD-1 Phase 2

DVB UHD-1 Phase 2 services provide a second example of using higher frame rates for advanced services and markets [2].  In the DVB activity, high frame rate video is identified as a goal of the Phase 2 activity.  Similar to the ATSC 3.0 described above, it may be desirable for UHD-1 Phase 2 to be an extension of UHD-1 Phase 1, which would necessitate support for both a standard frame rate and high frame rate representation of the sequence in the same bit-stream.

# Design Suggestions

In the previous section, we provided examples of work in different standards bodies that may be relevant to the support for the extraction of multiple frame rates from a VVC bit-stream.  However, we emphasize that these solutions are naturally limited, as they are developed within the context of existing standards that did not provide explicit support of multiple, high quality frame rates in the same bit-stream.

As VVC is an evolving standard, we recommend the following design goals in the development of any support for extracting multiple frame rates from a single bit-stream:

- The VVC design should allow for the encapsulation of two or more versions of a coded video sequence at different frame rates

- The VVC design should support the efficient compression of coded video sequences that do not contain temporal judder and/or temporal strobing at all frame rate representations

- The VVC design should support a sub-bitstream extraction process that supports extracting conforming bitstreams at different frame rates from a single bitstream where the frame rate representations remaining in the extracted bitstream do not contain temporal judder an/or temporal aliasing.

- The VVC design should allow for the encapsulation of two or more versions of a coded video sequence at different frame rates and at different level and/or profile points

# Recommendation

It is recommended to create an AhG to further study the extraction of multiple frame rates from a VVC bit-stream.

# Patent rights declaration(s)

Sharp Laboratories of America may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] “ATSC Standard: Video – HEVC”, Advanced Television Systems Committee, A/341:2018, 28 Aug. 2018.

[2] “Phasing in Ultra High Definition”, Digital Video Broadcasting (DVB), Feb. 2017, https://www.dvb.org/resources/public/factsheets/dvb_-_phasing_in_uhd.pdf.