| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0626-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-M0528 (Non-CE3: A unified luma intra mode list construction process) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jie Yao 601,Yongda International Tower, 2277 Longyang Road Shanghai, 201204, China | Tel: Email: | +86-21-63350606 ext. 5809 yaojie@cn.fujitsu.com |
| Source: | Fujitsu |  |  |

_____________________________

- Abstract

This contribution reports crosscheck results of JVET-M0528(A unified luma intra mode list construction process). It is reported that the proposed method provides an average luma BD rate performance of -0.07% AI, x.xx% RA, x.xx% LDB with 100%/101% AI, xxx%/xxx% RA, xxx%/xxx% LDB encoding/decoding time over VTM-3.0.

- Algorithm description

In the current VTM-3.0 there exists three different luma intra mode list construction process: one for zero reference line index, a second for non-zero reference line index and a third for multi-hypothesis intra.

This contribution proposes the use of a single list construction process. The contribution achieves this by first signaling if a luma intra mode is angular, and for angular mode constructing lists comprising only of angular modes (which is the same as currently being used for non-zero reference line index). For non-angular luma intra mode (i.e. PLANAR, DC) an additional flag is signaled that indicates the selected mode. Note, that the unified list construction is used only for entropy coding and the encoder mode search is the same as in VTM-3.0.

For multi-hypothesis intra the only angular modes allowed are: vertical and horizontal modes. For the above listed approach, a single flag needs to be signaled to distinguish between the two angular modes (no list construction is needed).

The detailed syntax modifications is shown highlighted in yellow below:

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| if( slice_type != I ) { |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 ) |  |
| pred_mode_flag | ae(v) |
| } |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| … |  |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| … |  |
| } else { |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| if( ( y0 % CtbSizeY ) > 0 ) |  |
| intra_luma_ref_idx[ x0 ][ y0 ] | ae(v) |
| if (intra_luma_ref_idx[ x0 ][ y0 ] = = 0) |  |
| intra_luma_angular_mode_flag[ x0 ][ y0 ] | ae(v) |
| if (intra_luma_angular_mode_flag[ x0 ][ y0 ] = = 0) |  |
| intra_luma_planar_flag[ x0 ][ y0 ] | ae(v) |
| if (intra_luma_ref_idx[ x0 ][ y0 ] != 0 ¦ ¦ intra_luma_angular_mode_flag[ x0 ][ y0 ] != 0) { |  |
| intra_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_mpm_flag[ x0 ][ y0 ] ) |  |
| intra_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| else |  |
| intra_luma_mpm_remainder[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| … |  |
| if( mmvd_flag[ x0 ][ y0 ] = = 0 && merge_subblock_flag[ x0 ][ y0 ] = = 0 && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128 ) { |  |
| mh_intra_flag[ x0 ][ y0 ] | ae(v) |
| if( mh_intra_flag[ x0 ][ y0 ] ) { |  |
| intra_luma_angular_mode_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_angular_mode_flag[ x0 ][ y0 ] ) |  |
| mh_intra_luma_vert_flag[ x0 ][ y0 ] | ae(v) |
| else |  |
| intra_luma_planar_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |

intra_luma_angular_mode_flag[ x0 ][ y0 ] is context-adaptive arithmetic entropy-coded with ctxInc derivation process of using left, left-bottom, above and above-right syntax elements.

Experimental results

Experiments were performed based on VTM-3.0 software under JVET common test condition.

Table 1 shows experimental results of the proposed method compared to VTM-3.0.

Table 1. Results of the proposed method over VTM 3.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.29% | -0.33% | -0.20% | 97% | 101% |
| Class A2 | -0.04% | -0.10% | -0.07% | 97% | 101% |
| Class B | -0.08% | -0.10% | -0.06% | 101% | 101% |
| Class C | 0.03% | 0.01% | 0.03% | 100% | 102% |
| Class E | -0.01% | -0.03% | 0.01% | 102% | 101% |
| Overall | -0.07% | -0.10% | -0.05% | 100% | 101% |
| Class D | 0.04% | 0.19% | 0.06% | 104% | 102% |
| Class F | 0.21% | 0.15% | 0.15% | 122% | 102% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! |
| Class B | -0.03% | 0.06% | 0.00% | 102% | 99% |
| Class C | 0.01% | 0.11% | 0.22% | 115% | 99% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! |
| Class D | 0.01% | -0.33% | -0.27% | 137% | 100% |
| Class F | 0.09% | 0.14% | 0.11% | 148% | 97% |