| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakesh, MA, 9–18 Jan. 2019 | Document: JVET- M0637-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-M0072 (Non-CE6: On transform skip for lager block) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tadamasa Toma, Kiyofumi Abe | Tel: Email: | +81-80-3594-3378 toma.tadamasa@jp.panasonic.com abe.kiyo@jp.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

- Abstract

This contribution reports cross-check results of JVET-M0072 [1] on Non-CE6: On transform skip for lager block which proposes to allow transform skip for larger transform size than 4x4 and add signaling to indicate the maximum size of transform skip. The simulation was conducted with VTM 3.0 under the JVET common test condition (CTC) [2]. It was confirmed that the test results were matched with those of JVET-M0072.

Simulation Results

The simulation was conducted for AI and RA under CTC for Test#1, Test#2, and Test#3 as follows.

Test#1: Transform Skip for up to 8x8 Transform blocks

Test#2: Transform Skip for up to 16x16 Transform blocks

Test#3: Transform Skip for up to 32x32 Transform blocks

Test#1: Transform Skip for up to 8x8 Transform blocks

Table 1  AI condition for Test#1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.03% | 0.10% | 103% | 98% |
| Class A2 | 0.03% | 0.03% | 0.00% | 105% | 99% |
| Class B | 0.00% | -0.02% | 0.00% | 107% | 100% |
| Class C | -0.12% | -0.14% | -0.26% | 109% | 100% |
| Class E | 0.01% | -0.02% | 0.05% | 106% | 100% |
| Overall | -0.03% | -0.04% | -0.03% | 106% | 99% |
| Class D | -0.11% | -0.17% | -0.17% | 110% | 97% |
| Class F | -1.88% | -1.55% | -1.54% | 114% | 97% |

Table 2  RA condition for Test#1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.23% | 0.21% | 102% | 99% |
| Class A2 | 0.03% | -0.08% | -0.03% | 103% | 99% |
| Class B | -0.04% | 0.16% | -0.05% | 103% | 99% |
| Class C | -0.18% | -0.09% | 0.16% | 104% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.05% | 0.06% | 0.06% | 103% | 99% |
| Class D | -0.34% | -0.43% | -0.10% | 105% | 100% |
| Class F | -1.83% | -1.31% | -1.48% | 107% | 97% |

Test#2: Transform Skip for up to 16x16 Transform blocks

Table 3  AI condition for Test#2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.02% | 0.05% | 120% | 99% |
| Class A2 | 0.04% | -0.03% | -0.07% | 123% | 99% |
| Class B | 0.00% | 0.01% | 0.01% | 119% | 98% |
| Class C | -0.16% | -0.15% | -0.24% | 122% | 103% |
| Class E | 0.00% | -0.01% | 0.01% | 120% | 99% |
| Overall | -0.03% | -0.04% | -0.05% | 121% | 100% |
| Class D | -0.11% | -0.33% | -0.35% | 121% | 97% |
| Class F | -2.15% | -1.79% | -1.82% | 125% | 96% |

Table 4  RA condition for Test#2

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.11% | 0.27% | 108% | 100% |
| Class A2 | 0.06% | 0.19% | 0.01% | 110% | 99% |
| Class B | -0.10% | 0.19% | -0.01% | 109% | 99% |
| Class C | -0.27% | -0.13% | 0.05% | 113% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.09% | 0.09% | 0.07% | 110% | 99% |
| Class D | -0.49% | -0.31% | -0.06% | 114% | 98% |
| Class F | -2.21% | -1.60% | -1.92% | 115% | 98% |

Test#3: Transform Skip for up to 32x32 Transform blocks

Table 5  AI condition for Test#3

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.04% | 0.08% | 130% | 99% |
| Class A2 | 0.05% | 0.00% | -0.16% | 129% | 99% |
| Class B | -0.01% | -0.04% | -0.06% | 127% | 99% |
| Class C | -0.17% | -0.24% | -0.31% | 128% | 102% |
| Class E | 0.02% | -0.13% | -0.13% | 128% | 99% |
| Overall | -0.03% | -0.09% | -0.12% | 128% | 99% |
| Class D | -0.12% | -0.25% | -0.43% | 126% | 97% |
| Class F | -2.14% | -1.79% | -1.98% | 131% | 96% |

Table 6  RA condition for Test#3

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.04% | 0.38% | 117% | 99% |
| Class A2 | 0.08% | 0.08% | 0.04% | 117% | 99% |
| Class B | -0.13% | 0.27% | -0.04% | 118% | 99% |
| Class C | -0.33% | -0.11% | 0.02% | 122% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.11% | 0.09% | 0.08% | 119% | 99% |
| Class D | -0.58% | -0.26% | 0.06% | 124% | 99% |
| Class F | -2.36% | -1.80% | -2.11% | 124% | 97% |

Conclusion

It was confirmed that the the simulation results of BD-rate match with those proponent provided.

Reference

- T. Tsukuba, M. Ikeda, T. Suzuki, “Non-CE6: On transform skip for larger block” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11, JVET-M0072.

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations”, JVET-J1010.

Patent rights declarations

Panasonic may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).