| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0821 |
| --- | --- |

| Title: | Cross-check of JVET-M0245:AHG16-related: Chroma block coding and size restriction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | TianYang Zhou Tomohiro Ikai | Tel: Email: | +81-43-299-8526 zhou.tianyang @sharp.co.jp ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

This contribution is a cross-check report of JVET-M0245: AHG16-related: Chroma block coding and size restriction. The crosscheck was carried out under the JVET common test condition (CTC). Experimental results (partial) match with those provided by the proponent..

Introduction

In JVET-M0245[1], it proposes removal of the chroma blocks of under 16 samples in size, i.e. 2x2, 2x4, and 4x2, and replacing the 2x2 coefficient group with a 2x8 or 8x2 coefficient group for the remaining narrow chroma block sizes.

Simulation results

The software on top of VTM-3.0 was provided by the proponents [1]. Crosscheck experiments were performed under the JVET common test conditions.

Following tables show the results of the cross-check experiments under VTM-3.0	 configuration.

## Table 1-Simulation result

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0_SHARP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | 0.18% | 0.02% | 101% | 100% |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.10% | 0.24% | 101% | 100% |
| Class C | 0.09% | 0.84% | 1.25% | 102% | 98% |
| Class E | -0.01% | -0.07% | 0.03% | 100% | 108% |
| Overall |  |  |  |  |  |
| Class D | 0.15% | 1.54% | 2.34% | 102% | 99% |
| Class F (optional) | 0.20% | 1.33% | 1.69% | 103% | 102% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0_SHARP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.40% | 0.19% | 101% | 100% |
| Class C | 0.02% | 0.60% | 1.04% | 103% | 100% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.05% | 1.06% | 1.81% | 103% | 99% |
| Class F (optional) | 0.10% | 0.90% | 1.11% | 103% | 106% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0_SHARP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.03% | 0.74% | 0.83% | 103% | 100% |
| Class E | -0.01% | 0.73% | 0.93% | 100% | 105% |
| Overall |  |  |  |  |  |
| Class D | -0.01% | 0.26% | 0.61% | 104% | 100% |
| Class F (optional) |  |  |  |  |  |

Conclusion

It was confirmed that the the simulation results (partial) of bitrate and PSNR match with those provided by the proponent.

References

[1] C. Rosewarne, A. Dorrell, Building A, 5 Talavera Rd, “AHG16-related: Chroma block coding and size restriction” JVET-M0245, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019