| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0826-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-M0126 supplemental tests (CE4: Modification on history-based motion vector prediction) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yu-Cheng Lin No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 yucheng.lin@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

The document reports the results of crosschecking JVET-M0126 supplemental tests proposed by Qualcomm. JVET-M0126 proposes to have 3 changes on History-based Motion Vector Prediction (HMVP) mode and pairwise candidates not using HMVP predictors which reduce the complexity in both encoder side and decoder side. The verification task was done, and the BD-rates results match exactly with those in JVET-M0126 supplemental tests.

# Verification

Qualcomm provided the source code of their proposed method described in JVET-M0126 [1].

Four test conditions are conducted under the the common test conditions (CTC) defined in [2] and are carried out by using the VTM3.0 software [3]. In all tests, the VTM3.0 is used as the anchor.

The BD-rates of four test conditions from our experimental results match exactly of those in JVET-M0126 supplemental tests.

Table 1. Simulation results of M0126 supplemental data A compared to VTM-3.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.12% | 0.05% | 100% | 104% |
| Class A2 | -0.03% | 0.03% | 0.08% | 100% | 102% |
| Class B | -0.01% | 0.01% | -0.07% | 100% | 102% |
| Class C | 0.03% | 0.03% | 0.03% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.01% | 0.01% | 100% | 102% |
| Class D | 0.00% | 0.09% | 0.17% | 100% | 101% |
| Class F (optional) | 0.03% | 0.01% | 0.00% | 102% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | -0.15% | -0.08% | 100% | 100% |
| Class C | 0.06% | 0.24% | 0.09% | 100% | 103% |
| Class E | 0.08% | 0.28% | -0.34% | 100% | 102% |
| Overall | 0.05% | 0.09% | -0.08% | 100% | 102% |
| Class D | -0.02% | 0.09% | 0.40% | 101% | 105% |
| Class F (optional) | -0.07% | 0.00% | 1.10% | 101% | 100% |

Table 2. Simulation results of M0126 supplemental data B compared to VTM-3.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.02% | -0.01% | 101% | 100% |
| Class A2 | -0.03% | 0.02% | 0.00% | 100% | 103% |
| Class B | 0.02% | 0.07% | 0.05% | 101% | 111% |
| Class C | 0.04% | 0.01% | 0.08% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.03% | 0.04% | 101% | 104% |
| Class D | 0.00% | -0.07% | 0.00% | 100% | 100% |
| Class F (optional) | 0.07% | 0.09% | 0.16% | 101% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.07% | -0.01% | 0.11% | 100% | 98% |
| Class C | 0.01% | 0.18% | 0.19% | 101% | 99% |
| Class E | 0.04% | 0.17% | -0.26% | 101% | 100% |
| Overall | 0.04% | 0.10% | 0.05% | 100% | 99% |
| Class D | 0.04% | -0.46% | 0.22% | 101% | 100% |
| Class F (optional) | 0.16% | 0.05% | 0.43% | 101% | 98% |

Table 3. Simulation results of M0126 supplemental data C compared to VTM-3.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.15% | 0.07% | 101% | 101% |
| Class A2 | 0.00% | -0.05% | 0.08% | 100% | 102% |
| Class B | 0.03% | 0.04% | 0.00% | 101% | 109% |
| Class C | 0.04% | 0.09% | 0.09% | 100% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.00% | 0.05% | 101% | 104% |
| Class D | -0.01% | 0.12% | -0.07% | 101% | 102% |
| Class F (optional) | 0.03% | 0.00% | 0.05% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.07% | -0.19% | 0.21% | 100% | 100% |
| Class C | 0.04% | 0.21% | 0.17% | 100% | 102% |
| Class E | 0.04% | 0.05% | 0.15% | 101% | 101% |
| Overall | 0.05% | 0.00% | 0.18% | 100% | 101% |
| Class D | -0.04% | 0.46% | -0.10% | 101% | 101% |
| Class F (optional) | 0.14% | -0.06% | 0.04% | 101% | 100% |

Table 4. Simulation results of M0126 supplemental data D compared to VTM-3.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.13% | 0.05% | 101% | 101% |
| Class A2 | 0.02% | 0.07% | 0.16% | 101% | 101% |
| Class B | 0.06% | 0.13% | 0.01% | 101% | 102% |
| Class C | 0.04% | 0.06% | 0.08% | 100% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | 0.05% | 0.07% | 100% | 102% |
| Class D | 0.01% | 0.07% | 0.02% | 101% | 100% |
| Class F (optional) | 0.05% | 0.02% | 0.07% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.06% | -0.25% | -0.16% | 100% | 97% |
| Class C | 0.04% | 0.41% | -0.04% | 100% | 102% |
| Class E | 0.01% | 0.28% | -0.34% | 101% | 99% |
| Overall | 0.04% | 0.10% | -0.17% | 100% | 99% |
| Class D | 0.01% | -0.09% | 0.12% | 101% | 100% |
| Class F (optional) | 0.01% | 0.15% | -0.29% | 100% | 99% |

# References

- Y. Han, W.-J. Chien, C.-C. Chen, H. Huang, C.-H. Hung, Y. Zhang, M. Karczewicz, “CE4: Modification on History-based Motion Vector Prediction,” Document of Joint Video Experts Team, JVET-M0126

- J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010, the 12th JVET meeting, Oct. 2018, Macao, CN.

- VTM3.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-3.0

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).