| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0838-v2 |
| --- | --- |

| Title: | CE-10 related: M0390-M0096 combination |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | F. Galpin T. Poirier E. François (Technicolor) Luong Pham Van Geert Van der Auwera Adarsh K. Ramasubramonian Vadim Seregin Marta Karczewicz (Qualcomm) | Tel: Email: | +33(0)299273198 franck.galpin@technicolor.com tangi.poirier@technicolor.com edouard.francois@technicolor.com lphamvan@qti.qualcomm.com geertv@qti.qualcomm.com aramasub@qti.qualcomm.com vseregin@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Technicolor, Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution describes the combination of 2 multi-hypothesis intra-inter prediction simplification described in JVET-M0390 and JVET-M0096. The proposed method has been implemented on top of VTM-3.0. The combination is reported to result in PSNR-Y BD-rate change of 0.02%, with 94% and 101% encoding and decoding times in RA configuration, -0.04%  with 94% and 101% encoding and decoding times in LDB configuration.

# Introduction

In VTM-3.0, multi-hypothesis prediction mode combines inter bi-prediction with intra prediction. This contribution describes the combination of 2 multi-hypothesis intra-inter prediction simplification:

- JVET-M0390: the inter prediction is restricted to uni-directional,

- JVET-M0096: the intra prediction is restricted to planar mode and PDPC is used at the final stage.

# Description

In VTM-3.0, multi-hypothesis prediction mode combines inter bi-prediction with intra prediction, samples are clipped after intra and inter prediction individually and after the combination of intra and inter prediction, as shown in Figure 1.

Figure 1: Multi-hypothesis prediction pipeline in VTM-3.0

In the proposed method, inter prediction is kept with high bit-depth precision, intra prediction bitdepth is increased to be the same as inter prediction, the sample clipping is done after the combination of intra and inter prediction, as shown in Figure 2 and described in JVET-M0390. Moreover, the combination of intra and inter prediction uses the PDPC process and only the planar mode is used as described in JVET-M0096.

The uni-directional motion vector predictors derivation from triangle partitions is reused for intra-inter mode. It is to be noted that in this test only 5 inter merge candidates are tested contrary to VTM-3.0 where 6 inter merge candidates are tested for Multi-hypothesis inter-intra.

Figure 2: Proposed multi-hypothesis pipeline

Overall, the simplifications are the following:

- Reduction of pipeline for the multi-hypothesis case (no GBI, BIO etc.)

- Reduction of the maximum number of predictions used to get the final prediction from 3 to 2,

- Reduction and harmonization of the clipping and shifting operations

- Reduction of the number of modes of the intra prediction

- Reduction of the number of merge candidates tested from 6 to 5

- Harmonization of multi-hypothesis and triangle mode inter prediction construction

The enhancements are the following:

- Keep the accuracy of the inter-prediction along the pipeline (as it is in bi-prediction process),

- Improve the prediction combination using PDPC and neighbors dependent weighting for intra and inter predictions

- Remove redundant triangle flag coding for multi-hypothesis

# Simulation Results

The simulations are conducted using the VTM configuration as described in JVET-L1010 [1]. VTM-3.0 reference software is used with VTM configuration files.

Table 1: simulation results of the proposed uni-directional restriction of multi-hypothesis in random access

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.01% | 0.10% | 96% | 100% |
| Class A2 | 0.06% | 0.08% | 0.27% | 93% | 100% |
| Class B | 0.03% | 0.01% | -0.05% | 94% | 101% |
| Class C | 0.00% | 0.21% | 0.14% | 94% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.08% | 0.09% | 94% | 101% |
| Class D | -0.04% | -0.11% | -0.04% | 94% | 101% |
| Class F | -0.03% | -0.03% | 0.04% | 93% | 101% |

Table 2: simulation results of the proposed uni-directional restriction of multi-hypothesis in low delay

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | 0.11% | 0.22% | 94% | 98% |
| Class C | -0.01% | 0.58% | 0.23% | 95% | 104% |
| Class E | -0.10% | -0.17% | 0.15% | 92% | 102% |
| Overall | -0.04% | 0.20% | 0.21% | 94% | 101% |
| Class D | 0.03% | 0.37% | 0.08% | 94% | 103% |
| Class F | -0.21% | 0.09% | -0.03% | 94% | 101% |

# Conclusion

This contribution proposes two simplifications of the multi-hypothesis prediction. The first one restricts inter prediction to be uni-directional. The second one restricts the intra mode to be planar only. Additionally, the multi-hypotheses process is harmonized by keeping the bitdepth accuracy along the pipeline as it is the case for bi-prediction mode and the intra and inter combination is improved. The combination is reported to result in PSNR-Y BD-rate change of 0.02%, with 94% and 101% encoding and decoding times in RA configuration, -0.04% with 94% and 101% encoding and decoding times in LDB configuration.

# References

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).