| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0864-v1 |
| --- | --- |

| Title: | [AHG5] Enhancement of cache model by adopting block-based format |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ryoji HASHIMOTO Seiji Mochizuki | Tel: Email: | ryoji.hashimoto.te@renesas.com seiji.mochizuki.zc@renesas.com |
| Source: | Renesas Electronics Corporation |  |  |

_____________________________

- Abstract

This contribution suggests an enhancement of tool to measure the memory bandwidth in VTM. In the general hardware implementation, the reference picture is not stored with raster scan order but stored with block-based address. The results with VTM 3.0 show block-based address can reduce memory bandwidth drastically and contributes considering the hardware implementation.

Introduction

In 9th meeting, the tool to measure the memory bandwidth[1] is adopted and implemented to VTM2.0. This tool can show the memory bandwidth used in inter prediction. AHG13, tool evaluation, uses this tool to check the impact to the memory bandwidth in each coding tool. However, the format to store yuv used in VTM is different from that used in the general hardware implementation. This may not help to considering hardware implementation. Motivated by this tendency, this contribution proposes a way to translate the format VTM uses to that hardware uses.

Format to store yuv

VTM 3.0 stores each yuv component (Y, Cb, Cr) with the format as shown in Figure 1 (raster scan format). This format is just same as the file format of yuv VTM reads and writes. In this format, a pixel stays away from the upper pixel on the memory. The distance is just picture width. This requests many external memory accesses on reading or writing a block. The number of memory access may reach the height of block. In other words, this format is not suitable for reading/writing a block which is widely used in video codecs. To decrease the number of access, block-based format as shown in Figure 2 is used in the general hardware implementation. This format has the advantage in accessing rectangle area used in many coding tools such as inter prediction.

Figure 1 yuv store format VTM uses (raster scan format)

Figure 2 yuv store format hardware uses (block-based format)

Measurement result

On VTM3.0, both formats are tested in the measurement of the memory bandwidth. On the implementation, there is no change in the format VTM 3.0 stores. In other words, VTM 3.0 still uses raster scan format to store the picture. Just the cache model translate address to judge cache hit/miss based on the input parameter such as size of block. Therefore, just Cachemodel.cpp/h are updated.

The width and height of block are configurable. The parameters on the measurement is available in Table 1. Table 2 and Table 3 show the summary of the measurement result. The full results are available in xlsx file. It is clear that adopting block-based format can reduce memory bandwidth drastically, around 75%. However, address translation from x, y position to memory address requires many arithmetic operations so that this format is not suitable in the software implementation.

Table 1 Used parameter in the test

| Parameter name | Value | Remarks |
| --- | --- | --- |
| cache line size | 256 |  |
| number of cache line | 64 |  |
| number of way | 2 |  |
| block width | 16 | 2D mode only |
| block height | 16 | 2D mode only |

Table 2 The measurement result of Random access

|  | Average | Worst |
| --- | --- | --- |
| Class A1 | 25.2% | 30.4% |
| Class A2 | 22.6% | 31.0% |
| Class B | 23.2% | 30.6% |
| Class C | 19.9% | 25.2% |
| Class E |  |  |
| Overall | 23.5% | 33.0% |
| Class D | 22.3% | 26.9% |
| Class F | 25.4% | 33.0% |

Table 3 The measurement result of Low delay B

|  | Average | Worst |
| --- | --- | --- |
| Class A1 |  |  |
| Class A2 |  |  |
| Class B | 18.7% | 25.6% |
| Class C | 16.5% | 20.2% |
| Class E | 20.4% | 27.3% |
| Overall | 18.7% | 30.8% |
| Class D | 18.5% | 23.8% |
| Class F (optional) | 20.4% | 30.8% |

Conclusion

This contribution suggests the enhancement of cache model, which supports block-based format. Block-based format can reduce memory bandwidth drastically. This block-based format is widely used in the hardware implementation so that this model helps tool evaluation in terms of the hardware implementation of VVC.

[1] JVET-J0090 AHG5: Measurement result of memory bandwidth comparison with JEM and HM http://phenix.int-evry.fr/jvet/doc_end_user/current_document.php?id=3510

Patent rights declaration(s)

Renesas Electronics Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).