| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2018 | Document: JVET- M0878-v1 |
| --- | --- |

| Title: | CE8-Realted: A combination of Test CE8.1.3 and Test CE8.1.2d |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Luong Pham Van Ted Hsieh Vadim Seregin Wei-Jung Chien Marta Karczewicz 5775 Morehouse Dr San Diego, CA 92121 | Tel: Email: | +1-858-658-5664 lphamvan@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution reports the simulation results of the Core Explore test CE8.1.3 on top of the CE8.1.2d. In this combination test, only one-line buffer above the current is added to the CPR search area. In addition, this test includes the CPR search area defined in Test CE8.1.2d. With the proposed combination test, the luma BD-rate changes for [Class F, class SCC 1080p] are:

CPR on and PLT off:

AI: (-12.21%, -37.14%) and (-0.17%, -0.37%) over VTM3 and VTM3+CPR on, resp.

RA: (-9.96%, -22.17%) and (-0.09%, -0.19%) over VTM3 and VTM3+CPR on, resp.

LB: (-5.18%, -13.03%) and (-0.05%, 0.07%) over VTM3 and VTM3+CPR on, resp.

CPR on and PLT on:

AI: (-17.73%, -49.84%) and (-3.24%, -9.67%) over VTM3 and VTM3+CPR on, resp.

RA: (-13.70%, -29.18%) and (-2.52%, -5.01%) over VTM3 and VTM3+CPR on, resp.

LB: (-7.12%, -17.23%) and (-1.19%, -2.98%) over VTM3 and VTM3+CPR on, resp.

# Introduction

The current picture reference (CPR) mode was adopted into BMS software in the last Ljubljana meeting [1]. In the last meeting in Macao, the CPR reference is restricted in the reconstructed area of the current CTU. This limitation significantly reduces the performance of CPR coding compared to full-frame search.

In CE8.1.3, the one-line buffer is added to the top and left of the CPR search area of the current CTU.

In CE8.1.2d, it is proposed to reuse the reference sample memory on a CU-by-CU basis.

This test considers a coding block in the current CTU and the block of same size and location in the left CTU as a pair of collocated blocks. For each reference block in the left CTU, if its entire collocated block in the current CTU has not yet been coded, the part for storing this reference block in the reference memory buffer has not yet updated with reference samples from the current CTU. This reference block can therefore be used for CPR prediction purpose.

# Proposed Method

This combined test includes only one top-line buffer to the CPR search area. The reference sample memory on a CU-by-CU basis as proposed in CE8.1.2d is also included.

# Experimental Results

The proposed technique has been implemented on top of VTM3.0. These tests are evaluated with PLT off and PLT on. The luma gain of this test (Class F, Class SCC) are summarized in Table 1.

CPR on and PLT off:

AI: (-15.37%, -45.06%) and (-4.36%, -13.65%) over VTM3 and VTM3+CPR on, resp.

RA: (-12.38%, -27.29%) and (-3.29%, -7.17%) over VTM3 and VTM3+CPR on, resp.

LB: (-6.62%, -16.87%) and (-1.73%, -4.54%) over VTM3 and VTM3+CPR on, resp.

CPR on and PLT on:

AI: (-17.73%, -49.84%) and (-3.24%, -9.67%) over VTM3 and VTM3+CPR on, resp.

RA: (-13.70%, -29.18%) and (-2.52%, -5.01%) over VTM3 and VTM3+CPR on, resp.

LB: (-7.12%, -17.23%) and (-1.19%, -2.98%) over VTM3 and VTM3+CPR on, resp.

Table 1(a) Test CE8.1.3 with CPR on and PLT off

Table 1(b) Test CE8.1.3 with CPR on and PLT on

# Conclusions

This contribution reports the simulation results of the Core Explore test CE8.1.3 on top of CE8.1.2(d).

References

[1] Xiaozhong Xu, Xiang Li and Shan Liu, “CE8-2.2: Current picture referencing using reference index signaling”, JVET-K0076, Ljubljana, SL, July 2018

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).