| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0885_v3 |
| --- | --- |

| Title: | Non-CE: Reduced complexity bilateral filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | J. Ström Torshamnsgatan 23 164 80 Stockholm, Sweden | Tel: Email: | +46107146437 {jacob.strom} @ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution describes a new version of the bilateral filter, that is claimed to be of less hardware complexity compared to the one presented in Macau, JVET-L0172. It is claimed that one per-sample multiplication is reduced from 10x8 bits to 2x8 bits. It is further claimed that a second per-sample multiplication is reduced from 13x11 bits to 10x11 bits. One per-block multiplication is also claimed to have been removed. Two simulations for RA have been produced. The first test is stated to be implementing JVET-L0172 in VTM-3.0 with the difference that the filter is only turned on for inter blocks and where the minimum size of a filtered block is 8x8 samples. This reported result is -0.42% for RA and -0.48% for LDB. In this first test the proposed changes have not been applied. In the second test the proposed changes have been applied, but is otherwise equivalent, and it is reported to give -0.41% for RA and -0.43% for LDB.

# Introduction

When JVET-L0172 was presented at the Macau meeting, it was considered too complex. In an attempt to ameliorate this, this contribution proposes to replace one of the multiplications by a smaller multiplication and a bit shift.

## Intra case

Since the filter is turned off for the intra case, this case does not happen.

## Inter case

For the inter case, the filtering contribution from a neighboring sample is changed from

rightInfluenceCalc = max(0, k * dNLIR + m) * dIR;

to

rightInfluenceCalc = (max(0, shifted_k * dNLIR + shifted_m) * dIR) << s;

This is done by shifting -k s steps until the resulting variable -shifted_k is between 4 and 7. The variable shifted_m is obtained by shifting m s steps. Whereas previously, k could be as large as -816, the variable shifted_k is now between -7 and -4. It can therefore be written on the binary form 1ab and can be calculated as a two-bit multiplication followed by an addition: ab * abs(dNLIR) + dNLIR<<3. Thus the first multiplication has gone down from 9x8 bits to 2x8 bits. (The second factor dNLIR is 8 bits and not 11 since the output is always zero for dNLIR values larger than 227.)

Also, previously m could be as large as 2136, but now the maximum number is 972. Since this is also equal to the largest output of the max() operation, the input to the second multiplication goes down from 12 bits to 10 bits. Thus for the intra case the second multiplication goes down from 12x11 bits to 10x11 bits.

The shifted value shifted_k and the shift value s are obtained from a look-up table which is of the same size as a previous table needed to calculate k. The multiplication per block that was previously needed, is therefore also avoided.

# Test Results

## Test1: Implementation of JVET-L0172 in VTM-3.0

These are the results for random access test configuration over VTM-3.0 CTC:

|  | Over VTM-3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.22% | -0.12% | 0.00% |  |  |
| Class A2 | -0.51% | -0.19% | -0.09% |  |  |
| Class B | -0.54% | 0.09% | -0.03% |  |  |
| Class C | -0.35% | 0.20% | 0.12% |  |  |
| Class E |  |  |  |  |  |
| Overall | -0.42% | 0.02% | 0.00% |  |  |
| Class D | -0.38% | -0.07% | 0.12% |  |  |
| Class F | -0.17% | -0.01% | -0.02% |  |  |

These are the results for the low delay B test configuration over VTM-3.0 CTC:

|  | Over VTM-3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.49% | 0.29% | 0.87% |  |  |
| Class C | -0.50% | 0.69% | 0.84% |  |  |
| Class E | -0.44% | 0.86% | 0.88% |  |  |
| Overall | -0.48% | 0.57% | 0.86% |  |  |
| Class D | -0.64% | 0.70% | 1.51% |  |  |
| Class F | -0.10% | 0.13% | -0.13% |  |  |

## Test2: Implementation of JVET-L0172 in VTM-3.0 with reduced multiplications

These are the results for random access test configuration over VTM-3.0 CTC:

|  | Over VTM-3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.25% | -0.31% | 0.02% |  | 102% |
| Class A2 | -0.48% | -0.20% | -0.03% |  | 99% |
| Class B | -0.52% | 0.09% | -0.03% |  | 101% |
| Class C | -0.34% | 0.21% | 0.15% |  | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.41% | -0.02% | 0.03% |  | 100% |
| Class D | -0.38% | -0.04% | 0.10% |  | 101% |
| Class F | -0.14% | 0.05% | 0.04% |  | 100% |

These are the results for the low delay test configuration over VTM-3.0 CTC:

|  | Over VTM-3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.43% | 0.26% | 0.75% |  | 101% |
| Class C | -0.43% | 0.75% | 0.69% |  | 102% |
| Class E | -0.41% | 0.72% | 0.54% |  | 103% |
| Overall | -0.43% | 0.54% | 0.68% |  | 102% |
| Class D | -0.71% | 0.59% | 0.97% |  | 103% |
| Class F | -0.11% | 0.28% | -0.11% |  | 100% |

# Proposal

Include the bilateral filter in a CE with the following tests carried out:

- How complexity reduction as described above can be used to reduce the size of multipliers or remove them completely (shifted_k always equals 1).

- How much gain is lost when filtering cannot use samples outside the intra block

- How much gain is lost when reducing or removing the non-local filtering

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).