| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0894r1 |
| --- | --- |

| Title: | Non-CE: Test on parametrizable bilateral filter from JVET-L0406 in VTM3.0 |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Marta Karczewicz Dmytro Rusanovskyy 5775 Morehouse Drive, San Diego, CA, U.S.A. 92121 | Tel: Email: | dmytror@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution report results parametrizable Bilateral Filtering (BIF) method that was proposed in the JVET-L0407. In this contribution, results for VTM3.0 CTC are reported. Complexity estimates produced at JVET-L0407 applied.

In Test 1 original design of the JVET-0407, with BIF enabled for intra/inter and minimal block size constrains (sizes 4x4 are not filtered) was tested. Preliminary results show that results in VTM3.0 are close to those reported in JVET-L0407, -0.4/-0.7% of bd-rate reduction on average for AI and RA. For LDB, -0.5% of bd-rate reduction is reported for completed test sequences.

Results for Test2 (filtering of intra-blocks and inter-block size of smaller than 8x8 is omitted) show -0.4% of bd-rate reduction on average of RA. For LDB bd-rate reduction of -0.5% is reported for completed test sequences.

It is proposed to test parametrized BIF in the CE for the following test:

- Test on proposed low complexity parametrization (Test 1) with specified constrains.

- Test on proposed low complexity parametrization with signaling.

# Introduction

The bilateral filter is an edge-preserving filter that takes into account the geometric distances between pixels as well as the intensity differences between them. By considering intensity differences between pixels, the bilateral filter prevents blurring of edges in an image during filtering.

This contribution reports results of the Bilateral Filtering method that was originally proposed in the JVET-J0021 with parametrization of the L0406 being tested in VTM3.0 CTC.

# Technical description

In this document parametrizable BIF design proposed in L0406 Bilateral Filtering method is being tested in VTM3.0 CTC.

In Test 1, original constrains on the filter are applied, e.g. Intra and Inter blocks are filtered and blocks with QP <17 and block sizes below 4x4 are not being filtered.

In Test2, BIF is constrained from usage on intra-coded blocks and inter-coded blocks size of <8x8.

# Simulation results

Simulation results for Test 1 are shown in Table 1-3 for AI/RA/LDB respectively.

Table 1. Comparative performance of the Test 1, AI configuration

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.18% | 0.11% | 0.19% | 105% | 106% |
| Class A2 | -0.37% | 0.14% | 0.08% | 104% | 105% |
| Class B | -0.34% | 0.15% | 0.15% | 105% | 104% |
| Class C | -0.66% | 0.00% | -0.09% | 107% | 106% |
| Class E | -0.45% | 0.16% | 0.19% | 104% | 102% |
| Overall | -0.41% | 0.11% | 0.10% | 105% | 105% |
| Class D | -0.54% | 0.16% | 0.17% | 105% | 106% |
| Class F (optional) | -0.75% | 0.01% | 0.03% | 106% | 101% |

Table 2. Comparative performance of the Test 1, RA configuration

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.39% | -0.17% | -0.02% | 103% | 103% |
| Class A2 | -0.74% | -0.22% | -0.15% | 105% | 104% |
| Class B | -0.81% | 0.04% | -0.07% | 105% | 105% |
| Class C | -0.75% | 0.05% | 0.12% | 103% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.69% | -0.05% | -0.03% | 104% | 104% |
| Class D | -0.77% | -0.21% | -0.05% | 104% | 111% |
| Class F (optional) | -0.65% | 0.14% | 0.10% | 102% | 103% |

Table 3. Comparative performance of the Test 1, LDB configuration

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.64% | 0.55% | 0.70% | 107% | 112% |
| Class E | -0.42% | 1.73% | 0.35% | 103% | 103% |
| Overall |  |  |  |  |  |
| Class D | -0.86% | 0.79% | 1.51% | 105% | 116% |
| Class F (optional) | -0.47% | 0.08% | -0.15% | 104% | 106% |

Preliminary results for Test 2 are shown in Table 4-5 for RA/LDB respectively.

Table 4. Comparative performance of the Test 2, RA configuration

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.22% | -0.15% | 0.00% | 101% | 101% |
| Class A2 | -0.45% | -0.06% | 0.06% | 104% | 103% |
| Class B | -0.48% | 0.10% | -0.03% | 105% | 104% |
| Class C | -0.26% | 0.12% | 0.17% | 103% | 105% |
| Class E |  |  |  |  |  |
| Overall | -0.36% | 0.02% | 0.05% | 103% | 103% |
| Class D | -0.30% | 0.02% | 0.17% | 103% | 110% |
| Class F (optional) | -0.14% | 0.02% | 0.05% | 103% | 104% |

Table 5. Comparative performance of the Test 2, LDB configuration

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.40% | 0.58% | 0.89% | 106% | 110% |
| Class E | -0.47% | 0.38% | 0.66% | 103% | 104% |
| Overall |  |  |  |  |  |
| Class D | -0.55% | 0.42% | 0.80% | 106% | 116% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Conclusion

This contribution report results parametrizable Bilateral Filtering (BIF) method that was proposed in the JVET-L0407. In this contribution, results for VTM3.0 CTC are reported for following tests. Complexity estimates produced at previous meeting applied.

It is proposed to test parametrized BIF in the CE for the following test:

- Test on proposed low complexity parametrization (Test 1) with specified constrains

- Test on proposed low complexity parametrization with signaling.

# References

- H. Yang, S. Liu, K. Zhang, JVET-L1024, “Description of CE4: Inter prediction and motion vector coding”, Macao, October 2018.

- CE4: STMVP simplification, JVET-M0221, Marrakesh, Morocco, Jan. 2019.

- CE4: Modification on History-based Motion Vector Prediction, JVET-M0126, Marrakesh, Morocco, Jan. 2019.

- CE4-related: Modification on Merge List, JVET-M0127, Marrakesh, Morocco, Jan. 2019.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).