| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M1024-v3 |
| --- | --- |

| Title: | Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding |  |  |
| --- | --- | --- | --- |
| Status: | Output document |  |  |
| Purpose: | Core Experiment description |  |  |
| Author(s) or Contact(s): | Haitao Yang Guichun Li Kai Zhang | Email: | haitao.yang@huawei.com guichunli@tencent.com zhangkai.video@bytedance.com |
| Source: | CE4 coordinators |  |  |

_____________________________

- Abstract

The goal of Core Experiment 4 (CE4) is to investigate various inter prediction and motion vector coding techniques on top of VTM-4.0. It comprises 4 categories,

- Merge mode signaling

- STMVP

- MMVD modification

- TPM merge list modification

CE4.1: Merge mode signaling

Test specification

| Test # | Description | Source | Tester | Crosschecker |
| --- | --- | --- | --- | --- |
| 4.1.1 | Signaling MMVD syntax elements when merge_flag equal to 0. | JVET-M0069 | E. Sasaki (Sharp) | Y. Chen (Kwai) |
| 4.1.2.a | Signal a regular merge flag right after the merge flag and skip flag to indicate whether the regular merge/skip mode is used by the current CU | JVET-M0231 | Y. Chen (Kwai) | E. Sasaki (Sharp) |
| 4.1.2.b | Combination of test 4.1.2.a with 4.1.1 | JVET-M0231 | Y. Chen (Kwai) | E. Sasaki (Sharp) |
| 4.1.3 | Change the MMVD flag position, and remove unnecessary syntax element | JVET-M0359 | G. Ko (WILUS) | E. Sasaki (Sharp) |
| 4.1.4.a | Signal a merge_subblock_flag and a merge_triangle_flag consecutively | JVET-M0369 | Y. Ahn (Digital Insights) | J. Kim (SKT) |
| 4.1.4.b | Combination of test 4.1.4.a with 4.1.1 | JVET-M0369 | Y. Ahn (Digital Insights) | S. C. Lim (ETRI) |
| 4.1.4.c | Combination of test 4.1.4.a with 4.1.3 | JVET-M0369 | Y. Ahn (Digital Insights) | S. C. Lim (ETRI) |
| 4.1.5.a | Remove TMVP merge candidate for the block size with samples <= 32 | JVET-M0345 | X. Zheng (DJI & PKU) | G. Li (Tencent) |
| 4.1.5.b | Remove TMVP merge candidate for the block size with samples <= 64 | JVET-M0345 | X. Zheng (DJI & PKU) | G. Li (Tencent) |
| 4.1.5.c | Remove TMVP merge candidate for the block size of 4xN / Nx4 and 8x8 | JVET-M0345 | X. Zheng (DJI & PKU) | G. Li (Tencent) |

Detailed description of tests

CE4.1.1 in JVET-M0069 (Sharp)

In JVET-M0069, a simple syntax change of MMVD is proposed. To simplify merge data syntax, this proposal signals MMVD syntax elements as separate mode from other merge modes. A proposed syntax is shown as below. The syntax other than mmvd related part follows the draft 4.0 of VVC.

Proposed coding unit syntax (add mmvd syntax when merge_flag is equal to 0)

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| if( slice_type != I ) { |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 ) |  |
| pred_mode_flag | ae(v) |
| } |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| .. |  |
| } else { /* MODE_INTER */ |  |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 ) { |  |
| merge_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_flag[ x0 ][ y0 ] ) { |  |
| merge_data( x0, y0, cbWidth, cbHeight ) |  |
| } else { |  |
| mmvd_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_flag[ x0 ][ y0 ] = = 1 ) { |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( slice_type = = B ) |  |
| inter_pred_idc[ x0 ][ y0 ] | ae(v) |
| … |  |
| } |  |
| } |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && cu_skip_flag[ x0 ][ y0 ] = = 0 ) |  |
| cu_cbf | ae(v) |
| if( cu_cbf ) |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType ) |  |
| } |  |
| } |  |

Proposed merge data syntax (remove mmvd syntax in merge_data)

| merge_data( x0, y0, cbWidth, cbHeight ) { | Descriptor |
| --- | --- |
| mmvd_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_flag[ x0 ][ y0 ] = = 1 ) { |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 ) |  |
| merge_subblock_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumSubblockMergeCand > 1 ) |  |
| merge_subblock_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_mh_intra_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128 ) { |  |
| mh_intra_flag[ x0 ][ y0 ] | ae(v) |
| if( mh_intra_flag[ x0 ][ y0 ] ) { |  |
| if ( cbWidth <= 2 * cbHeight ¦ ¦ cbHeight <= 2 * cbWidth ) |  |
| mh_intra_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( mh_intra_luma_mpm_flag[ x0 ][ y0 ] ) |  |
| mh_intra_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| if( sps_triangle_enabled_flag && slice_type = = B && cbWidth * cbHeight >= 16 ) |  |
| merge_triangle_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_triangle_flag[ x0 ][ y0 ] ) |  |
| merge_triangle_idx[ x0 ][ y0 ] | ae(v) |
| else if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |

CE4.1.2 in JVET- M0231 (Kwai)

To reduce the average number of coded bins for merge/skip modes, it is proposed to signal a regular merge flag right after the merge flag and skip flag to indicate whether the regular merge/skip mode is used by the current CU. Specifically, the proposed flag signaling for different types of merge modes is illustrated in the Table 1.2.2.1; for skip modes, the proposed flag signaling is illustrated in the Table 1.2.2.2.

Table 1.2.2.1 The flag signaling for the different types of merge modes in the proposed scheme

| Signaled Modes | Regular Flag | MMVD Flag | Subblock Flag | CIIP Flag |
| --- | --- | --- | --- | --- |
| Regular | 1 | - | - | - |
| MMVD | 0 | 1 | - | - |
| Subblock | 0 | 0 | 1 | - |
| CIIP | 0 | 0 | 0 | 1 |
| Triangle | 0 | 0 | 0 | 0 |

Table 1.2.2.2 The flag signaling for the different types of skip modes in the proposed scheme

| Signaled Modes | Regular Flag | MMVD Flag | Subblock Flag |
| --- | --- | --- | --- |
| Regular | 1 | - | - |
| MMVD | 0 | 1 | - |
| Subblock | 0 | 0 | 1 |
| Triangle | 0 | 0 | 0 |

CE4.1.3 in JVET- M0359 (WILUS)

This proposal changes the position of mmvd_flag, and removes mmvd_merge_flag and its context model.

VVC has adopted several merge (or skip) schemes, such as subblock merge modes, merge mode with motion vector difference (MMVD), the combined inter-picture merge and intra-picture prediction, and triangular shape based motion compensation. If merge_flag is equal to 1, mmvd_flag is always parsed since it is the first syntax in merge data syntax. Therefore, when the coding unit uses the newly adopted merge mode listed above, or when it uses the conventional merge mode with the candidate index equal to or greater than 2, mmvd_flag is parsed.

It is proposed that mmvd_flag is parsed when merge_idx is equal to 0 or 1. This is because the current MMVD uses the first or second merge candidate as a base candidate. Instead of mmvd_merge_flag, merge_idx specifies which merge candidate is used as an MMVD base candidate. Thus, there is no need for separate mmvd_merge_flag and its context model.

CE4.1.4 in JVET- M0369 (Digital Insights)

In this contribution, it is proposed to adjust the order based on the type of prediction partition. By arranging the merge_subblock_flag and the merge_triangle_flag, which are syntaxes related to the type of prediction partition, sequentially, the proposed method can provide flexibility for further syntax changes. The reason that the merge_subblock_flag is positioned at the front is that the coding efficiency of affine prediction and sbTMVP is the highest. In addition, the merge_triangle_flag is placed after that to determine block-level merge and triangle-level merge.

CE4.1.5 in JVET- M0345 (DJI & PKU)

JVET-M0345 suggests removing unnecessary TMVP merge candidates at the specified blocksizes. The number of merge candidates is still same as original design when TMVP merge candidate is removed. The following three cases will be tested in CE:

- Remove TMVP merge candidate for the blocksize with samples <= 32, which means 4x4/4x8/8x4 TMVP is removed from merge list;

- Remove TMVP merge candidate for the blocksize with samples <= 64, which means 4x4/4x8/8x4/4x16/16x4/8x8 TMVP is removed from merge list;

- Remove TMVP merge candidate for the blocksize with samples of 4xN / Nx4 and 8x8, which means 4x4/4x8/8x4/4x16/16x4/32x4/4x32/8x8 TMVP is removed from merge list;

CE4.2: STMVP

Test specification

| Test # | Description | Source | Tester | Crosschecker |
| --- | --- | --- | --- | --- |
| 4.2.1.a | STMVP merge candidate produced as a weighted average of selected motion vector predictors. | JVET-M0518 | D. Rusanovskyy (Qualcomm) | F. Le Léannec (Technicolor) |
| 4.2.1.b | STMVP merge candidate with modified ordering of the merge list construction. | JVET-M0518 | D. Rusanovskyy (Qualcomm) | F. Le Léannec (Technicolor) |
| 4.2.2.a | STMVP merge candidate, consisting in an averaged motion vector between the TMVP merge candidate and some other motion vectors predictors. | JVET-M0713 | F. Le Léannec (Technicolor) | T. Zhou (Sharp) |
| 4.2.2.b | Test 4.2.2 with multiple STMVP candidates. | JVET-M0713 | F. Le Léannec (Technicolor) |  |

Complexity analysis

| Merge list size | Max number of potential candidates | Max number of candidate comparison | Max number of MV scaling | Max number of temporal candidates | Additional local buffer | Max number of memory access | Others |
| --- | --- | --- | --- | --- | --- | --- | --- |

Detailed description of tests

CE4.2.1 in JVET- M0518

CE4.2.2 in JVET- M0713

In this proposal, the spatial temporal motion vector prediction is generated based on the TMVP merge candidate, as well as one to two motion vector predictors. If less than 2 MV predictors are available to generate the STMVP candidate, then the STMVP candidate is considered as not available. The computation of the TMVP candidate is modified compared to HEVC, by clipping the bottom-right TMVP position inside the current CTU row if necessary. For limited complexity increase, STMVP will be tested without added MV scaling or candidate pruning. In addition, multiple STMVP candidates may be added to the merge candidate list.

CE4.3: MMVD modification

Test specification

| Test # | Description | Source | Tester | Crosschecker |
| --- | --- | --- | --- | --- |
| 4.3.1.a | Single distance list concept | JVET-M0206 | S. Jeong (Samsung) |  |
| 4.3.1.b | Binarization method change | JVET-M0206 | S. Jeong (Samsung) | T. Hashimoto (Sharp) |
| 4.3.1.c | Multiple distance list concept | JVET-M0206 | S. Jeong (Samsung) | T. Hashimoto (Sharp) |
| 4.3.1.d | Multiple distance list concept + binarization method change | JVET-M0206 | S. Jeong (Samsung) | S. Yoo (LGE) |
| 4.3.2.a | Binarization of MMVD distance index with grouping w/o additional contexts | JVET-M0267 | K. Zhang (Bytedance) | X. Li (Tencent) |
| 4.3.2.b | 4.3.2.a+additional context models | JVET-M0267 | K. Zhang (Bytedance) | X. Li (Tencent) |
| 4.3.2.c | 4.3.2.a + 4.1.1 | JVET-M0267 | K. Zhang (Bytedance) |  |
| 4.3.2.d | 4.3.2.b + 4.1.1 | JVET-M0267 | K. Zhang (Bytedance) |  |
| 4.3.3.a | MMVD distance table is reduced 4 and is refined based on the resolution of base motion vectors | JVET-M0307 | N. Park (LGE) | E. Sasaki (Sharp) |
| 4.3.3.b | 4.3.3.a + 4.3.4.a | JVET-M0307 | N. Park (LGE) | E. Sasaki (Sharp) |
| 4.3.3.c | 4.3.3.a + 4.1.1 | JVET-M0307 | N. Park (LGE) | E. Sasaki (Sharp) |
| 4.3.3.d | 4.3.3.b + 4.1.1 | JVET-M0307 | N. Park (LGE) | E. Sasaki (Sharp) |
| 4.3.4.a | Diagonal direction candidate for MMVD | JVET-M0060 | E. Sasaki (Sharp) | N. Park (LGE) |
| 4.3.4.b | 4.3.4.a + 4.1.1 | JVET-M0060 | E. Sasaki (Sharp) | N. Park (LGE) |

Detailed description of tests

CE4.3.1 in JVET-M0206 (Samsung)

Distance list index is coded in MMVD mode for expressing the distance from the base motion vector of a merge candidate. Currently, distance list index is coded with truncated unary coding method. This test introduces new binarization method for distance list index. This binarization method is tested on both own single distance list and own multiple distance list.

CE4.3.2 in JVET-M0267 (Bytedance)

In JVET-M0267, it is proposed to signal the MMVD distance index in a grouping way. A group index is signaled first and then an index is signaled which distance in the group is used.

CE4.3.3 in JVET-M0307 (LGE)

In this proposal, MMVD optimization scheme is proposed. Specifically, instead of 8 motion distance indices of MMVD, 4 motion distance indices are used, and the actual motion distance from the given motion distance index is further adjusted considering the magnitude of the base motion vector, block size and/or resolution of the base motion vector.

CE4.3.4 in JVET-M0060 (Sharp)

It is proposed that directional candidates are used in addition to horizontal/vertical candidates for MMVD.The directional candidates are shown as below.

| Direction IDX | 000 | 001 | 010 | 011 | 100 | 101 | 110 | 111 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x-axis | +1 | –1 | 0 | 0 | +1/2 | -1/2 | -1/2 | +1/2 |
| y-axis | 0 | 0 | +1 | –1 | +1/2 | -1/2 | +1/2 | -1/2 |

CE4.4: TPM merge list modification

Test specification

| Test # | Description | Source | Tester | Crosschecker |
| --- | --- | --- | --- | --- |
| 4.4.1.a | Use regular merge mode candidate list and two signaled indices. Candidate 1 select L0 MV if exist; otherwise select L1 MV. Candidate 2 select L1 MV if exist; otherwise select L0 MV. | JVET- M0184 | T.-D. Chuang (MediaTek) | T. Solovyev (Huawei) |
| 4.4.1.b | Use regular merge mode candidate list and two signaled indices. Candidate 1 select L0 MV if exist; otherwise scaled from L1 MV. Candidate 2 select L1 MV if exist; otherwise scaled from L0 MV. | JVET- M0184, JVET- M0317 | T.-D. Chuang (MediaTek) | X. Li (Tencent) |
| 4.4.1.c | 4.4.1.a + MV pruning and adding offset MV if necessary | JVET- M0184 | T.-D. Chuang (MediaTek) | T. Solovyev (Huawei) |
| 4.4.1.d | 4.4.1.b + MV pruning and adding offset MV if necessary | JVET- M0184, JVET- M0317 | T.-D. Chuang (MediaTek) |  |
| 4.4.2.a | Use first 3 candidates of regular merge candidate list for triangle list construction | JVET- M0194 | A. Tamse (Samsung) | H. Wang (Qualcomm) |
| 4.4.2.b | Use first 4 candidates of regular merge candidate list for triangle list construction | JVET- M0194 | A. Tamse (Samsung) | H. Wang (Qualcomm) |
| 4.4.2.c | 4.4.2.a + spatial temporal candidate only restriction | JVET- M0194 | A. Tamse (Samsung) | H. Wang (Qualcomm) |
| 4.4.2.d | 4.4.2.c + simplified pruning | JVET- M0194 | A. Tamse (Samsung) | H. Wang (Qualcomm) |
| 4.4.2.e | Use regular merge list with spatial temporal restriction | JVET-M0194 | A. Tamse (Samsung) | H. Wang (Qualcomm) |
| 4.4.2.f | 4.4.2.e + simplified pruning | JVET-M0194 | A. Tamse (Samsung) | H. Wang (Qualcomm) |
| 4.4.3.a | Share regular merge candidate list for triangle partition merge prediction. For RA, L0 and L1 motion vectors from the regular candiate list are selected in an interleaving manner. For LDB, L0 miton vectors are selected, followed by uni-prediction zero vectors. | JVET- M0233 | X. Wang (Kwai) | H. Wang (Qualcomm) |
| 4.4.3.b | Share regular merge candidate list for triangle partition merge prediction. For RA, L0 and L1 motion vectors from the regular candiate list are selected in an interleaving manner. For LDB, L0 miton vectors are selected, followed by L1 motion vectors. | JVET- M0233 | X. Wang (Kwai) | H. Wang (Qualcomm) |
| 4.4.4.a | Partial of the regular merge list construction process is reused. | JVET- M0271 | L. Zhang (Bytedance) | G. Li (Tencent) |
| 4.4.5.a | Reduce the pruning operations in TPM merge list construction: use same spatial pruning as in merge + prune temporal cand together, remove pruning for unidirectional and averaged cand in TPM merge list construction. | JVET- M0283 | A. Robert (Technicolor) | G. Li (Tencent) |
| 4.4.5.b | Reduce the pruning operations in TPM merge list construction: use same spatial pruning as in merge + prune temporal cand together, remove all pruning operations in TPM merge list construction. | JVET- M0283 | A. Robert (Technicolor) | G. Li (Tencent) |
| 4.4.6.a | Derive triangular merge list based on the candidates of regular merge list, using same scan order. If candidate is bi-predictive, L0 part is used. | JVET- M0286 | T. Solovyev (Huawei) | T.-D. Chuang (MediaTek) |
| 4.4.6.b | Derive triangular merge list based on the candidates of regular merge list, using same scan order. If candidate is bi-predictive, pruning is applied. | JVET- M0286 | T. Solovyev (Huawei) | T.-D. Chuang (MediaTek) |
| 4.4.7.a | The reference index of L1 is set equal to 0, mirroring process is applied after offset addition | JVET- M0317 | J. Li (Panasonic) |  |
| 4.4.7.b | The reference index of L1 is set equal to 0, mirroring process is applied before offset | JVET- M0317 | J. Li (Panasonic) |  |
| 4.4.7.c | The reference index of L1 is set equal to be the same as that of L0, mirroring process is applied after offset addition | JVET- M0317 | J. Li (Panasonic) | R.-L. Liao (Alibaba) |
| 4.4.7.d | The reference index of L1 is set equal to be the same as that of L0, mirroring process is applied before offset addition | JVET- M0317 | J. Li (Panasonic) | R.-L. Liao (Alibaba) |
| 4.4.8.a | Reuse regular merge list derivation for triangle merging candidate list construction | JVET- M0349 | X. Zheng (DJI & PKU) | J. Li (Panasonic) |
| 4.4.8.b | Use candidates before HMVP in the regular merge list as the candidates for triangle merging candidate list construction | JVET- M0349 | X. Zheng (DJI & PKU) | J. Li (Panasonic) |
| 4.4.9.a | Derive merge list based on the regular list, bi-MV is split into uni-MVs and pruning is applied. | JVET- M0399 | H. Wang (Qualcomm) | X. Li (Tencent) |
| 4.4.9.b | 4.4.9.a with no pruning in the final TPM merge list. | JVET- M0399 | H. Wang (Qualcomm) | X. Wang (Kwai) |
| 4.4.9.c | Merge list is derived based on the spatial and temporal MV candidates from the regular merge list, bi-MV is split into uni-MVs and pruning is applied | JVET- M0399 | H. Wang (Qualcomm) | X. Wang (Kwai) |
| 4.4.9.d | 4.4.9.c with no pruning in the final TPM merge list. | JVET- M0399 | H. Wang (Qualcomm) | X. Wang (Kwai) |

Complexity analysis

| Test# | Merge list size | Max number of potential candidates (stage1+stage2) | Max number of candidate comparison (stage1+stage2) | Max number of MV scaling (stage1+stage2) | Max number of temporal candidates (stage1+stage2) | Max number of MV average (stage1+stage2) | Max number of encoder MCs (stage1+stage2) | Others |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Regular merge | 6 | 13 | 9 | 2 | 2 | 2 | 12 |  |
| TPM | 5 | 28(7+21) | 84(10+74) | 11(4+7) | 2(2+0) | 7(0+7) | 5 |  |
| 4.4.x |  |  |  |  |  |  |  |  |

Detailed description of tests

CE4.4.1 in JVET- M0184 (MediaTek) and JVET-M0317 (Panasonic)

To reduce the complexity of triangular merge candidate derivation, it is proposed to discard the current triangle merging candidate list and reuse the existing regular merge candidate list of non-triangle prediction to generate a uni-prediction MV for each of the two triangular partitions. In the proposed triangle prediction, two regular merge candidates from the existing merging candidate list are first selected. Next, for the first triangular merge candidate, it takes the L0 MV of the first selected regular merge candidate if the L0 MV exist; otherwise, the L1 MV of the first selected regular merge candidate is used. For the second triangular merge candidate, it takes the L1 MV of the first selected regular merge candidate if the L1 MV exist; otherwise, the L0 MV of the second selected regular merge candidate is used. Finally, an offset MV of (1/4 pel, 0) is added to the merging candidate of the second partition if the two triangular merge candidates are the same. Another test is to combine the concept of JVET-M0317 that two different candidates are from different reference list of MVs. Two regular merge candidates from the existing merging candidate list are first selected. For the first triangular merge candidate, it takes the L0 MV of the first selected regular merge candidate if the L0 MV exist; otherwise, the L0 MV is scaled from the L1 MV of the first selected regular merge candidate. For the second triangular merge candidate, it takes the L1 MV of the first selected regular merge candidate if the L1 MV exist; otherwise, the L1 MV is scaled from the L0 MV of the second selected regular merge candidate. Finally, an offset MV of (1/4 pel, 0) is added to the merging candidate of the second partition if the two triangular merge candidates are the same. The result of using partial regular merge candidate list will be tested as well. The partial regular merge candidate list is the constructed merge candidate before adding the history candidate.

CE4.4.2 in JVET- M0194 (Samsung)

TPM generates uni-directional MV list by following two steps

- Step 1. Construct an MV candidate list by checking spatially neighbour MV and adding up to two temporal MVs

- Step 2. Use the above constructed list to construct a new list of five uni-directional MVs

Unification of candidate list derivation

To unify the step 1 of the triangle mode candidate derivation process with the regular merge list candidate derivation, step 1 is simply replaced with the regular merge list derivation process.

Furthermore since the step 2 requires only a few candidates from step 1 in order to completely fill up the uni-directional MV list of five candidates, only the first N of candidates from the regular merge candidate list are used. This restriction further simplifies the list derivation process

Restriction to only spatial and temporal candidates

When using the regular merge list candidate construction process, HMVP buffer is accessed for adding HMVP candidates and averaging operations are performed for adding pairwise average candidates. These operations although not frequently performed (since the method requires only the first N candidates, N=3, 4), add new computations in the worst case compared to the original step 1 of the triangle merge list construction process. To address this, the regular merge candidate list construction process only till temporal candidates is followed, i.e. only spatial and temporal candidates are added. HMVP, average and zero candidates are not added. The list construction process stops after the

Pruning simplification

Step 2 of the triangle mode candidate list constructs a list of five uni-directional MVs. When performing the pruning operation, each newly candidate added is added after comparing it with each of the existing MVs in the list. This results in many comparisons especially for the candidates added near the end of the list, which needs to be repeated if the candidate fails the pruning check. A simpler pruning method is proposed to reduce the total number of comparisons. Each candidate is only compared against the previously added candidate, and not all the previously added candidates. Hence each candidate undergoes only one MV comparison (except for the first candidate, which doesn’t require any comparison).

CE4.4.3 in JVET-M0233 (Kwai)

Use the regular merge candidate list for triangle partition merge prediction. For RA (where backward prediction is used), L0 and L1 motion vectors from the regular candidate list are selected in a simple interleaving manner as the triangle uni-prediction merge candidates. For LDB (where backward prediction is not used), L0 motion vectors from the regular candidate list are selected as the triangle uni-prediction merge candidates, followed by zero motion vectors or L1 motion vectors from the regular merge list.

CE4.4.4 in JVET-M0271 (Bytedance)

To simplify the TPM merge list construction process, three methods are presented in this contribution. In solution #1, the regular merge list construction process is reused. Only regular merge candidates may be pruned and there is no TPM candidate pruning operations. In solution #2, the same spatial and one temporal blocks may be accessed to derive regular merge candidates without pruning. Pruning is only invoked to derived TPM candidates. Different from solution #2, solution #3 need to always access to 2 temporal blocks. Detailed descriptions of the three methods are described in the following sub-sections.

CE4.4.5 in JVET-M0283 (Technicolor)

This contribution then proposes to:

- Harmonize first spatial candidates pruning as in Merge mode since these candidates are the same as in Merge,

- Perform a simple pruning of the temporal candidates together,

- Remove the pruning of uni-directional candidates since the pre-list is already (partially) pruned,

- Remove the pruning of averaged candidates.

In a second, more drastic, version where all pruning of the motion predictor list construction are removed, it is, in addition, proposed to:

- Remove the pruning of L0 parts of bi-directional candidates,

- Remove the pruning of L1 parts of bi-directional candidates.

CE4.4.6 in JVET-M0286 (Huawei)

The additional merge candidate derivation process for triangular prediction mode is removed. Instead, triangular merge list is created based on regular merge list, using same scan order. Bi-prediction candidates are converted to uni-prediction candidates.

2 alternative solutions for conversion bi-predictive candidates to uni-predictive are tested:

- If candidate is bi-predictive, L0 part is used

- If candidate is bi-predictive, pruning is applied.

CE4.4.7 in JVET-M0317 (Panasonic)

To unify the candidate list derivation of triangular mode with that of regular merge mode, the triangular candidate list is derived from the first few candidates in regular merge candidate list. If the candidate is a bi-prediction one, it is directly copied to the triangular candidate list. If the candidate is a uni-prediction one from L0, the reference index and motion vector of L1 is derived from those of L0. An offset is added to L1 and a mirroring process is applied if reference pictures of L0 and L1 come from different directions. 4 variations are tested:

- the reference index of L1 is set equal to 0, mirroring process is applied after offset addition

- the reference index of L1 is set equal to 0, mirroring process is applied before offset addition

- the reference index of L1 is set equal to be the same as that of L0, mirroring process is applied after offset addition

- the reference index of L1 is set equal to be the same as that of L0, mirroring process is applied before offset addition

Similarly, the same process is applied if the uni-prediction candidate is from L1.

Besides, to remove the mapping process used in mv storage for triangular mode, one of the triangular prediction units must be predicted from L0 and another one must be predicted from L1.

CE4.4.8 in JVET-M0349 (DJI & PKU)

To simplify triangle mode merge list construction process, JVET-M0349 proposes to reuse current inter merge list as the list for triangle mode. To further improve coding efficiency for triangle mode, M0349 also proposes to use candidates before HMVP in the regular merge list, which means only four spatial candidates and tmvp candidates in the list are used at triangle motion information derivation.

CE4.4.9 in JVET-M0399 (Qualcomm)

In the contribution, regular merge list construction is used to derive TPM merge list. After deriving merge candidates based on the regular merge list, the bi-MV is split into uni-L0-MV and uni-L1-MV, which are added to the final TPM merge list. Full pruning or no pruning is performed when final uni-directional TPM list is derived.

Test 1. Derive merge list based on the regular list, bi-MV is split into uni-MVs and pruning is applied.

Test 2. Test 1 with no pruning in the final TPM merge list.

Test 3. Merge list is derived based on the spatial and temporal MV candidates from the regular merge list, bi-MV is split into uni-MVs and pruning is applied.

Test 4. Test 3 with no pruning in the final TPM merge list.

Test condition and evaluation criteria

Each testing item is required to be compared against VTM, evaluated using BD rates, encoding time, decoding time according to Common Test Conditions (CTC) in JVET-M1010. Simulation with RA and LDB configurations are mandatory.

Sub-CE specific test condition and evaluation criteria may be defined for a sub-CE if considered informational for tool evaluation. Please refer to the section of test condition and evaluation criteria in each sub-CE for information.

SW integration guideline

All tests are required to be implemented on top of VTM-4.0.

Timeline

| T1 | Feb. 8, 2019 | Finalize CE description |
| --- | --- | --- |
| T2 | Feb. 25, 2019 | Deliver software packages and begin cross-checking |
| T3 | Mar. 4, 2019 | Software frozen, testers provide full results, and final crosschecking begins |
| T4 | Mar. 12, 2019 | Submit contribution documents with respective texts and complexity-analysis data |

Participant list

| Company/ Organization | Participant Name | Email Address |
| --- | --- | --- |
| Technicolor | Antoine Robert Philippe Bordes Tangi Poirier Fabrice Le Léannec Franck Galpin Ya Chen | antoine.robert@technicolor.com philippe.bordes@technicolor.com tangi.poirier@technicolor.com fabrice.leleannec@technicolor.com franck.galpin@technicolor.com Ya.CHEN@technicolor.com |
| HHI | Martin Winken Robert Skupin Jens Brandenburg Anastasia Henkel | martin.winken@hhi.fraunhofer.de robert.skupin@hhi.fraunhofer.de jens.brandenburg@hhi.fraunhofer.de anastasia.henkel@hhi-extern.fraunhofer.de |
| Huawei | Huanbang Chen Haitao Yang Jianle Chen Sychev Maxim Solovyev Timofey Alexey Filippov Vasily Rufitskiy Semih Esenlik Alexander Karabutov | chenhuanbang@huawei.com haitao.yang@huawei.com jianle.chen@huawei.com Sychev.Maxim@huawei.com solovyev.timofey@huawei.com alexey.filippov@huawei.com vasily.rufitskiy@huawei.com semih.esenlik@huawei.com karabutov.alexander@huawei.com |
| LGE | Seung Hwan Kim Jaehyun Lim Naeri Park Hyeongmun Jang Jie Zhao Seethal Paluri | seunghwan3.kim@lge.com Jaehyun.lim@lge.com naeri.park@lge.com hm.jang@lge.com jie.zhao@lge.com seethal.paluri@lge.com |
| Tencent | Xiaozhong Xu Jing Ye Guichun Li Shan Liu Meng Xu Xiang Li | xiaozhongxu@tencent.com jingye@tencent.com guichunli@tenceNnt.com shanl@tencent.com mengxxu@tencent.com xlxiangli@tencent.com |
| USTC | Yue Li Li Li | lytt@mail.ustc.edu.cn lili90th@gmail.com |
| HiSilicon | Na Zhang Xu Chen Jianhua Zhang | zhangna25@huawei.com anderson.chen@hisilicon.com zhengjianhua@hisilicon.com |
| BBC | Saverio Blasi | saverio.blasi@bbc.co.uk |
| ETRI | Jungwon Kang Sung-Chang Lim Hahyun Lee | jungwon@etri.re.kr sclim@etri.re.kr hanilee@etri.re.kr |
| Panasonic | Kiyofumi Abe Jingya Li Ru-Ling Liao | abe.kiyo@jp.panasonic.com jingya.li@sg.panasonic.com ruling.liao@sg.panasonic.com |
| Fujitsu | Jianqing Zhu Kimihiko Kazui | zhujianqing@cn.fujitsu.com kazui.kimihiko@jp.fujitsu.com |
| Sony | Kenji Kondo | kenji.kondo@sony.com |
| Qualcomm | Wei-Jung Chien Vadim Seregin Chun-Chi Chen Yan Zhang Yu Han Han Huang Chao-Hsiung Hung Luong Pham Van Dmytro Rusanovskyy | wchien@qti.qualcomm.com vseregin@qti.qualcomm.com chunchic@qti.qualcomm.com yzh@qti.qualcomm.com yuhan@qti.qualcomm.com hanhuang@qti.qualcomm.com chaohsiu@qti.qualcomm.com lphamvan@qti.qualcomm.com dmytror@qti.qualcomm.com |
| Alibaba | Yu-Chen Sun | yuchen.s@alibaba-inc.com |
| Hikvision | Fangdong Chen | chenfangdong@hikvision.com |
| Sharp | Weijia Zhu Byeongdoo Choi Tomonori Hashimoto Takeshi Chujoh Tomohiro Ikai Kiran Misra Eiichi Sasaki Tianyang Zhou | zhuw@sharplabs.com choib@sharplabs.com tomonori.hashimoto@sharp.co.jp chujoh.takeshi@sharp.co.jp ikai.tomohiro@sharp.co.jp misrak@sharplabs.com eiichi.sasaki@sharp.co.jp zhou.tianyang@sharp.co.jp |
| Ericsson | Ruoyang Yu Julien Le Tanou | ruoyang.yu@ericsson.com julien.le.tanou@ericsson.com |
| Samsung Electronics | Min Woo Park Seungsoo Jeong Anish Tamse Kiho Choi Jie Chen Minsoo Park Yinji Piao | m.w.park@samsung.com ss00.jeong@samsung.com tamse.anish@samsung.com kiho14.choi@samsung.com jie1222.chen@samsung.com mss.park@samsung.com yj1003.piao@samsung.com |
| Nokia | Jani Lainema Alireza Aminlou Ramin Ghaznavi-Youvalari | jani.lainema@nokia.com alireza.aminlou@nokia.com ramin.ghaznavi-youvalari@nokia.com |
| InterDigital | Yuwen He Daniel Luo Hua Yang Wei Chen Saurav Bandyopadhyay | Yuwen.He@interdigital.com Daniel.Luo@InterDigital.com hua.yang@interdigtial.com wei.chen@interdigital.com Saurav.Bandyopadhyay@InterDigital.com |
| Arris | Krit Panusopone Yue Yu David Baylon Seungwook Hong Limin Wang | Krit.panusopone@arris.com Yue.yu@arris.com David.baylone@arris.com Seungwook.hong@arris.com Limin.wang@arris.com |
| MediaTek | Yu-Wen Huang Chih-Wei Hsu Ching-Yeh Chen Tzu-Der Chuang Shih-Ta Hsiang Yuling Hsiao Jenny Lai Yu-Chi Su Zhi-Yi Lin Chun-Chia Chen Yucheng Lin | yuwen.huang@mediatek.com cw.hsu@mediatek.com chingyeh.chen@mediatek.com Peter.Chuang@mediatek.com shih-ta.hsiang@mediatek.com Yuling.Hsiao@mediatek.com Jenny.Lai@mediatek.com Yu-Chi.Su@mediatek.com Zhi-Yi.Lin@mediatek.com chunchia.chen@mediatek.com Yucheng.Lin@mediatek.com |
| NHK | Shunsuke Iwamura | iwamura.s-gc@nhk.or.jp |
| Netflix | Andrey Norkin | anorkin@netflix.com |
| Orange | Gordon Clare | gordon.clare@orange.com |
| Broadcom | Minhua Zhou Pensong Chen Wade Wan | minhua.zhou@broadcom.com peisong.chen@broadcom.com wade.wan@broadcom.com |
| Canon | Jonathan Taquet | Jonathan.TAQUET@crf.canon.fr |
| PKU | Zhao Wang Suhong Wang Xuewei Meng | zhaowang@pku.edu.cn suhong.wang@pku.edu.cn xwmeng@pku.edu.cn |
| Chips&Media | Wook Je Jeong | nick.jeong@chipsnmedia.com |
| Aachen University | Johannes Sauer | sauer@ient.rwth-aachen.de |
| HIT | Yang Wang Xiaopeng Fan | wangyang.cs@hit.edu.cn fxp@hit.edu.cn |
| WILUS | Greg Ko | greg.ko@wilusgroup.com |
| ITRI | Ching-Chieh Lin | JackLin@itri.com |
| Bytedance | Kai Zhang Li Zhang | zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Kwai | Yi-Wen Chen Xianglin Wang | yiwenchen@kwai.com xianglinwang@kwai.com |
| HNU | Haechul Choi Heeji Han Daehyeok Gwon jaeryun Choe | choihc@hanbat.ac.kr sarhakor@naver.com skyeye0530@naver.com tjswo9395@naver.com |
| Dolby | Peng Yin | pyin@dolby.com |
| Intel | Grace Yu | grace.yu@intel.com |
| DJI | Xiaozhen Zheng | xiaozhen.zheng@dji.com |
| Alibaba | Yan Ye Jie Chen | yan.ye@alibaba-inc.com jiechen.cj@alibaba-inc.com |
| Digital Insights | Yongjo Ahn | yjahn@digitalinsights.co.kr |