| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M1033-v2 |
| --- | --- |

| Title: | Description of Core Experiment 13 (CE13): Neural Network based Filter for Video Coding |  |  |
| --- | --- | --- | --- |
| Status: | Output document |  |  |
| Purpose: | Core experiment description |  |  |
| Author(s) or Contact(s): | Yiming Li Shan Liu Kei Kawamura | Email: | userlym@whu.edu.cn shanl@tencent.com ki-kawamura@kddi.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

The purpose of this Core Experiment (CE) is to investigate the coding tools related to neural network (NN) based loop filter on top of the Test Model. In this CE, the impact of the position of the NN based filter in the filter chain, the benefit of the CTU level NN filter adaptive on/off switching, and the generalization capability when QP is different from the training QP will be investigated. In addition to the encoder and decoder run-times, the network information, e.g., network parameter numbers, number of layers, and convolution kernel size will also be evaluated. The list of tools and experiments to be performed are detailed in this document.

Introduction

The goal of Core Experiment 13 (CE13) is to investigate the neural network based loop filter techniques on top of VTM4.0. It comprises 2 categories,

- CE 13.1: Topics on top of sequence-adaptive (two-pass) methods

- CE 13.2: Topics on top of sequence-independent (one-pass) methods

Each category will investigate the following problems:

- The impact of NN filter position in the filter chain.

- The benefit of the CTU/block level NN filter adaptive on/off.

- The generalization capability of the NN filter when the test QP is not the same as the training QP.

Participant list

| Affiliation | Participant Name | Email Address | Proponent | Cross-checker |
| --- | --- | --- | --- | --- |
| Hikvision | Jiabao Yao Chaoyi Lin Li Wang | yaojiabao@hikvision.com linchaoyi@hikvision.com wangli7@hikvision.com | X |  |
| Intel | Hujun Yin Zhiping Deng Rongzhen Yang | hujun.yin@intel.com zhipin.deng@intel.com rongzhen.yang@intel.com | X |  |
| KDDI | Kei Kawamura Yoshitaka Kidani Kyohei Unno | ki-kawamura@kddi.com yo-kidani@kddi.com ky-unno@kddi-research.jp | X | X |
| MediaTek | Yu-Wen Huang Yu-Ling Hsiao Ching-Yeh Chen Tzu-Der Chuang Olena Chubach | yuwen.huang@mediatek.com Yuling.Hsiao@mediatek.com chingyeh.chen@mediatek.com Peter.Chuang@mediatek.com Olena.Chubach@mediatek.com | X |  |
| Tencent | Shan Liu Byeongdoo Choi | shanl@tencent.com bdchoi@tencent.com | X |  |
| USTC | Yue Li Ning Yan Yuanying Dai Dong Liu | lytt@mail.ustc.edu.cn nyan@mail.ustc.edu.cn daiyy@mail.ustc.edu.cn dongeliu@ustc.edu.cn | X |  |
| Wuhan Univ. | Yingbin Wang Yiming Li Zhenzhong Chen | wyb@whu.edu.cn userlym@whu.edu.cn zzchen@whu.edu.cn | X |  |
| LGE | Hyeongmun Jang Junghak Nam | hm.jang@lge.com junghak.nam@lge.com |  |  |
| Kuai | Xianglin Wang Yiwen Chen Xiaoyu Xiu Tsung-Chuan Ma | xianglinwang@kuaishou.com yiwenchen@kwai.com xiaoyuxiu@kwai.com tsung-chuanma@kwai.com |  |  |
| Technicolor | Fabien Racapé Franck Galpin | Fabien.Racape@technicolor.com Franck.Galpin@technicolor.com |  |  |
| NWPU | Shuai Wan | swan@nwpu.edu.cn |  |  |
| Xidian Univ. | Junyan Huo Yanzhuo Ma | jyhuo@mail.xidian.edu.cn yzma@mail.xidian.edu.cn |  |  |
| Huawei | Sergey Ikonin Seungwook Hong | Sergey.Ikonin@huawei.com seuhong@outlook.com |  |  |
| Sharplabs | Andrew Segall Kiran Misra | asegall@sharplabs.com misrak@sharplabs.com |  |  |
| Fujitus | Jianqing Zhu Kimihiko Kazui | zhujianqing@cn.fujitsu.com kazui.kimihiko@jp.fujitsu.com |  |  |
| Qualcomm | Nan Hu Yan Zhang | nanh@qti.qualcomm.com yzh@qti.qualcomm.com |  |  |
| Nokia | Francesco Cricri Caglar Aytekin | francesco.cricri@nokia.com caglar.aytekin@nokia.com |  |  |
| Bytedance | Li Zhang | lizhang.idm@bytedance.com |  |  |
| Ubilinx | Eric Chai Weimin Zeng | eric.chai@ublnx.com weimin.zeng@ublnx.com |  |  |
| Panasonic | Chong Soon Lim Han Boon Teo | chongsoon.lim@sg.panasonic.com hanboon.teo@sg.panasonic.com |  |  |
| ETRI | Seunghyun Cho | shcho@etri.re.kr |  |  |
| InterDigital | Philippe Hanhart Yuwen He Rahul Vanam Shilin Wu | Philippe.Hanhart@InterDIgital.com Yuwen.He@Interdigital.com Rahul.Vanam@Interdigital.com Shilin.Wu@Interdigital.com |  |  |
| Samsung | Chirag Pujara Raj Narayana Gadde | chirag.p@samsung.com raj.gadde@samsung.com |  |  |
| Dolby | Peng Yin | pyin@dolby.com |  |  |
| Netflix | Andrey Norkin | anorkin@netflix.com |  |  |

Description of tools

CE13-1 Topics on top of sequence-adaptive methods

Technical descriptions

JVET-M0159 AHG9: Convolutional neural network loop filter  [Y.-L. Hsiao, C.-Y. Chen, T.-D. Chuang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

The proposed CNNLF is one network shared by luma and chroma, as shown in the following Figure 1. In this design, each coding tree unit (CTU) is independently processed by CNNLF. In Packing, every 2x2-sample block of luma are treated as one sample of four channels, so there are six channels in total (Y0, Y1, Y2, Y3, Cb, Cr). After Packing, the size of the reconstructed samples becomes 64x64x6. After 3 layers convolution, the size of the results is still 64x64x6. In Unpacking, the first 4 channels ensemble the luma coding tree block (CTB), and last 2 channels are for the two chroma CTBs. Finally, the CNNLF output samples are calculated by summing ALF output samples and the Unpacking output samples.

Figure 1. The proposed network design of CNNLF in this contribution.

The parameters of CNNLF are signaled in the I-slice header of each random access segment (RAS, roughly 1 sec in the conducted tests), and the parameters are trained by the pictures in this RAS with temporal ID equal to 0 or 1; and multi-level on/off control is applied at slice level and coding tree block (CTB) level for each color component.

To reduce the computation overhead of generating the required data for CNNLF training process, only pictures with temporal ID equal to 0 or 1 are used to derive CNNLF parameters in the training process. That is, only these pictures are required to be encoded twice. The first round is to generate the required data for CNNLF training process and derive the CNNLF parameters. The second round is to generate the final bitstream by enabling CNNLF with the derived parameters.

JVET-M0566 Adaptive convolutional neural network loop filter  [H. Yin, R. Yang, X. Fang, S. Ma, Y. Yu (Intel)]

The tested ACNNLF has the following properties:

- Two layer CNN, as shown in the figure 2;

- Up to 3 ACNNLFs for luma and 3 ACNNLFs for chroma adaptively trained with video data;

- The encoder selects the best ACNNLF for each of luma and chroma in each CTB from the trained set of ACNNLFs, to perform encoding. The weights of the trained set of ACNNLFs are signaled in the tile group header and the index of selected ACNNLF is signaled in the coded stream in CTU level;

The decoder receives the ACNNLF weights (up to 3 sets for luma and 3 set for chroma) and the CTU level selected ACNNLF and uses the selected ACNNLF to perform the filtering process for each luma and chroma block during frame reconstruction.

Figure 2. ACNNLF for luma

The parameters of ACNNLF are signaled in the tile group header. In RA configuration, only the I-slice contains ACNNLF parameters of this RAS. The on/off control is applied at the tile group header level for luma and chroma components, and the acnnlf_luma_slice_enable_flag and acnnlf_chroma_slice_enable_flag syntax elements are used to indicate ACNNLF enable/disable. CTB level information indicates which ACNNLF is selected or if there is no ACNNLF applied in the CTB.

Summary of Tests

Since there are 3 parts need to investigate (loop filter position, CTU level on/off, and generalization capability). When testing one part, the test set corresponding to other parts should use the default set (as define in Section 4.1).

| Test # | Test Part | Description | Source | Tester | Crosschecker |
| --- | --- | --- | --- | --- | --- |
| CE13-1.1a | Loop filter chain | Loop filter chain: DBF + SAO + ALF + NN filter. NN filter follows the description in M0159 | JVET-M0159 | Y.-L. Hsiao (Mediatek) |  |
| CE13-1.1b |  | Loop filter chain: NN filter only. NN filter follows the description in M0159 |  |  |  |
| CE13-1.2a | Loop filter chain | Loop filter chain: DBF + SAO + ALF + NN filter. NN filter follows the description in M0566 | JVET-M0566 | H. Yin (Intel) |  |
| CE13-1.2b |  | Loop filter chain: NN filter only. NN filter follows the description in M0566 |  |  |  |

CE13-2 Topics on top of sequence-independent methods

Technical descriptions

JVET-M0351 AHG9: Convolutional Neural Network Filter (CNNF) for Intra Frame [C. Lin, J. Yao, L. Wang (Hikvision)]

The network description in shown in Figure 3. The CNNF is trained offline and includes two inputs: reconstruction and quantization parameter (QP), which makes it QP-independent. Currently the CNNF is only applied on I-frame, replacing DF and SAO.

Figure 3 CNNF network description

In order to investigate the impact of NN filter position in the filter chain, the performance of various combination with other existing loop filter will be tested.

JVET-M0508 AHG9: Test Results of Dense Residual Convolutional Neural Network based In-Loop Filter [Y. Wang, Z. Chen, Y. Li (Wuhan Univ.), L. Zhao, S. Liu, X. Li (Tencent)]

The network structure of DRNLF is shown in Fig. 4, where N and M denotes the number of dense residual unit (DRU) and convolution kernel, respectively. Specifically, N is set to 4 and M is set to 32 for the tradeoff between computational efficiency and performance. Normalized QP map is concatenated with the reconstructed frame as the input to DRN. So the input has 4 channels in total.

Figure 4. Network structure of DRNLF.

The main body of the proposed network consists of series of DRU. The structure of DRU is shown in Fig. 5. The DRU directly propagates its input to the subsequent unit through the shortcut. To further reduce the computational cost, 3x3 depth-wise separable convolutional (DSC) layer is applied in DRU.

Figure 5. Dense Residual Unit (DRU)

Finally, the output of the network has 3 channels, which corresponds to Y, Cb, Cr, respectively. The filter is applied for both intra and inter pictures. Additional flag is signaled for each CTUs to indicate the on/off of the DRNLF.

JVET-M0510 AHG9: CNN-based in-loop filter proposed by USTC [Y. Dai, D. Liu, Y. Li, F. Wu (USTC)]

Fig 6. The structure of the first proposed network

Residue learning technique is integrated into the network as Fig. 6 shows. To reduce memory usage, parameters of several layers are shared, which means we can achieve the performance of deeper network with fewer parameters. As shown in Fig.6, layers of the same color represent the same parameters. The proposed CNN-based filter is located between DF and SAO. In order to investigate the impact of NN filter position in the filter chain, the performance of various combination with other existing loop filter will also be tested.

JVET-M0872 AHG9: A Result of Convolutional Neural Network Filter [K. Kawamura, S. Naito (KDDI)]

The filter structure is shown in Figure 7. This filter has four layers with 3x3 taps. The input of “sum” block is residual signal from the left signal and reconstructed signal just after deblocking filter. The output of “sum” block is filtered pixels. Actual output is weighted sum of after-filtered and before-filtered pixels based on the distance of edge.

Figure 7. The structure of proposed CNNF

Filter coefficients are pre-defined in both the encoder and the decoder so that additional side information is not required. A representation of filter coefficients is fixed point value (integer value). The filter is applied for both intra and inter pictures.

Summary of Tests

Since there are 3 parts need to investigate (loop filter position, CTU level on/off, and generalization capability). When test one part, the test set corresponding to other parts should use the default set (as define in Section 4.1).

| Test # | Test Part | Description | Source | Tester | Crosschecker |
| --- | --- | --- | --- | --- | --- |
| CE13-2.1a | Loop filter chain | DBF + NN filter + SAO + ALF, NN filter follows the description in M0351 | JVET-M0351 | C. Lin (Hikvision) |  |
| CE13-2.1b |  | NN filter only, NN filter follows the description in M0351 |  |  |  |
| CE13-2.1c |  | NN filter + ALF, NN filter follows the description in M0351 |  |  |  |
| CE13-2.2a | Loop filter chain | DBF + NN filter + SAO + ALF, NN filter follows the description in M0508 | JVET-M0508 | Y. Wang (Wuhan Univ.) |  |
| CE13-2.2b |  | NN filter only, NN filter follows the description in M0508 |  |  |  |
| CE13-2.2c |  | NN filter + ALF, NN filter follows the description in M0508 |  |  |  |
| CE13-2.3a | Network generalization capability (QP) | CTC QP for training, CTC QP for testing (the same as CE13-2.2.a) |  |  |  |
| CE13-2.3b |  | CTC QP for training, CTC QP + 2 for testing |  |  |  |
| CE13-2.3c |  | CTC QP for training, CTC QP - 2 for testing |  |  |  |
| CE13-2.4a | Loop filter chain | DBF + NN filter + SAO + ALF, NN filter follows the description in M0510 | JVET-M0510 | Y. Dai (USTC) |  |
| CE13-2.4b |  | NN filter only, NN filter follows the description in M0510 |  |  |  |
| CE13-2.4c |  | NN filter + ALF, NN filter follows the description in M0510 |  |  |  |
| CE13-2.5a | Network generalization capability (QP) | CTC QP for training, CTC QP for testing(the same as CE13-2.4.a) |  |  |  |
| CE13-2.5b |  | CTC QP for training, CTC QP + 2 for testing |  |  |  |
| CE13-2.5c |  | CTC QP for training, CTC QP - 2 for testing |  |  |  |
| CE13-2.6a | Loop filter chain | DBF + NN filter + SAO + ALF, NN filter follows the description in M0872 | JVET-M0872 | K. Kawamura (KDDI) |  |
| CE13-2.6b |  | NN filter only, NN filter follows the description in M0872 |  |  |  |
| CE13-2.6c |  | NN filter + ALF, NN filter follows the description in M0872 |  |  |  |
| CE13-2.7a | CTU switch on/off | NN filter adaptive on/off in CTU level |  |  |  |
| CE13-2.7b |  | always use NN filter for each CTU |  |  |  |

Test condition and test set definition

Test set definition

The summary of different test set

| Test part | # | Description For SubCE1 | Description For SubCE2 |
| --- | --- | --- | --- |
|  | Anchor | DBF + SAO + ALF, no NN filter, use CTC QP to test | DBF + SAO + ALF, no NN filter, use CTC QP to test |
| Filter position | 1a(Default) | DBF + SAO + ALF+ NN filter | DBF + NN filter + SAO + ALF |
|  | 2a | NN filter. | NN filter. |
|  | 3a | N/A | NN filter + ALF. |
| CTU switch on/off | 1b(Default) | NN filter CTU switch | NN filter CTU switch |
|  | 2b | N/A | always use NN filter |
| Generalization (QP) | 1c(Default) | use CTC QP | use CTC QP |
|  | 2c | N/A | use CTC QP + 2 to test |
|  | 3c | N/A | use CTC QP - 2 to test |

Test condition

The following test conditions shall be used for this CE:

Software

The Test Model VTM-4.0 shall be used for these experiments, as specified in JVET-M1010.

Test configuration

This CE shall use the following two test conditions:

- Common Test Conditions (CTC).

- CTC, but short test, which only need to test the first intra period.

| Purpose | # | CTC test (Full test) |  |  | First intra period test (Short version) |
| --- | --- | --- | --- | --- | --- |
|  |  | All Intra | Random Access | Low delay | Random Access |
|  | anchor | M (For SubCE2) | M | O | O,S |
|  | 1a+1b+1c | M (For SubCE2) | M | O | O,S |
| Filter position | 2a+1b+1c | N/A | N/A | N/A | O,S |
|  | 3a+1b+1c | N/A | N/A | N/A | O,S |
| CTU switch | 1a+2b+1c | N/A | N/A | N/A | O,S |
| Generalization (QP) | 1a+1b+2c | N/A | N/A | N/A | O,S |
|  | 1a+1b+3c | N/A | N/A | N/A | O,S |

Note: M denotes mandatory test. O denotes the optional test. S denotes the short test. The full test should follow the CTC. The short test should also follow the CTC, except the number of frames, which only need to test the first intra period.

The meaning of each label in this table (e.g., 1a, 2a) is described in Section 4.1. For example, 1a+1b+2c means using CTC QP for training, but using CTC QP + 2 for testing, the filter position and CTU on/off should use the default set. From this table, the CE tests following 1a+1b+2c (e.g., CE13-2.3b, CE13-2.5b) only need to provide the short version results.

Network analysis

For the understanding of network, e.g., complexity, the following information shall be provided for each test planned for this CE: (just list for example. More information is specified in JVET-M1006)

- Total Conv. Layers: Total numbers of convolutional layers in the network structure.

- Total FC Layers: Total number of fully-connected layers in the network structure.

- Framework: The neural network framework in the inference stage. (e.g. Caffe, Tensorflow, Pytorch)

- Param. Num: Total numbers of the parameters in the neural network.

- Mem.P(MB): Memory usage of the total parameters. Mem=P∙B1024*1024, where P is the parameters number, and B indicates the byte for storing one parameter.

- Mem.T(MB): Temporary memory. It denotes the memory used to store the outputs (feature map) in each intermediate layer (forward pass).

- Parameter precision: float or integer? And the bit-depth.

Timeline

| T1 | Feb. 11, 2019 | - CE description is finalized and uploaded - Release of VTM software |
| --- | --- | --- |
| T2 | (2 weeks after VTM-4.0 SW release) | - AHG9 provides anchor bitsteam. - Proponents provide initial SW of CE tools - Cross-checkers may start initial study of tools / software |
| T3 | Mar.1 | - SW of each CE test is finalized - Cross-checkers start final cross checks |
| T4 | Document upload deadline, Mar. 12, 2019 | - Complete results of each CE test shall be provided - Cross-checker report crosschecking results to proponents and CE coordinators - CE13 summary report is provided |