| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0029-v2 |
| --- | --- |

| Title: | CE9: Summary report on decoder motion vector derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Semih Esenlik Xiaoyu Xiu | Tel: Email: | semih.esenlik@huawei.com, xiaoyuxiu@kwai.com |
| Source: | CE Coordinators |  |  |

_____________________________

# Abstract

The tools in the scope of this CE include bi-directional optical flow (BDOF) and decoder motion vector refinement (DMVR).

The core experiment summary report is organized into 2 sub-tests as follows:

- CE9-1: DMVR Specific Simplification (3 tests)

- CE9-2: BDOF Specific Simplification (9 tests)

This report summarises the status of each experiment. Crosscheck results are integrated in the report.

# Introduction

The description of Core Experiment 9 (CE9) on decoder motion vector derivation [1] has been finalized on Feb. 8, 2019. After this date the CE description document is updated to include updated timeline and crosschecker informations and typo corrections. A total 12 tests are defined in CE9 description. The goal of this Core Experiment (CE) is to further investigate the techniques of decoder motion vector refinement and bi-directional optical flow techniques.

# Experimental Conditions and Evaluation

## Software and Anchors

The VTM-4.0 software has been used in the CE9 as anchor software.

## Test Sequences, Bit Rates and Coding Conditions

CTC as described in [2].

## Reporting of the Results

All test results and crosscheck results were uploaded to git servers and included in the CE9 summary report. The BD-rate, the encoding and decoding with the same SIMD setting for both anchor and test are evaluated.

## Additional evaluation of BDOF tests

In subsection 4 (CE9-2: BDOF specific simplification) the following suggestion from the meeting note should be taken into account: “The maximum size where early termination is used should be blocks of size 256 luma samples.”

# CE9-1: DMVR Specific Simplifications

## Test Specifications

| # | Description | Document |
| --- | --- | --- |
| CE9-1.1a | Disable 8x8/4xN CUs for DMVR. | JVET-N0407 |
| CE9-1.1b | CE9-1.1.a + Remove boundary padding process | JVET-N0407 |
| CE9-1.2 | Impact of bit-depth restriction applied to interpolated samples used for refinement in DMVR. | JVET-N0291 |

## Summary of results

| Test | Description | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- |
| CE9-1.1a | Disable 8x8/4xN CUs for DMVR. | 0.04% | 0.07% | 0.03% | 100% | 99% |
| CE9-1.1b | CE9-1.1.a + Remove boundary padding process | 0.04% | 0.04% | 0.05% | 99% | 99% |
| CE9-1.2 | Impact of bit-depth restriction applied to interpolated samples used for refinement in DMVR | -0.01% | -0.04% | -0.04% | 101% | 101% |

## Crosscheck Results

|  |  | Tester |  | Crosscheck |  | Crosschecker comments |
| --- | --- | --- | --- | --- | --- | --- |
| Test | Crosschecker | EncT | DecT | EncT | DecT |  |
| CE9-1.1a | S. Esenlik | 100% | 99% | 100% | 100% | The software was investigated, which is aligned with the CE description. |
| CE9-1.1b | S. Esenlik | 99% | 99% | 100% | 100% | The software was investigated, which is aligned with the CE description. |
| CE9-1.2 | C.-C. Chen T. Chujoh | 101% | 101% | 100% | 100% | As CE description, the bit precision of interpolation filter of DMVR is changed. SIMD code is not changed.(by T. Chujoh) |

## Related Contributions

| Doc. # | Related test # | Title |
| --- | --- | --- |
| JVET-N0407 | CE9-1.1 | CE9: Removal of 4xN/8x8 CUs and Boundary Padding Process from DMVR (Test 9.1.1) |
| JVET-N0291 | CE9-1.2 | CE9: Results of DMVR related Tests CE9-1.2 |

## Worst case memory BW investigation for CE9-1.1a and CE9-1.1b

The memory bandwidth of DMVR is measured by using the same methodology as introduced in previous CE9 summary report (JVET-M0029) to calculate the number of required reference samples for MC, as follows:

F(W, H, Taps, SR) = ((W + 2 * SR) + Taps - 1 ) * ((H + 2 * SR) + Taps - 1 ) * 2,

where W and H represents the width and height of a given CU, respectively, Taps is the interpolation filter length of MV, SR denotes the search range (i.e. 2) of DMVR and the trailing 2 is for bi-prediction. To understand the full impact on bandwidth when introducing a new coding tool to DMVR, this contribution takes the memory bandwidth of Y/U/V channels into account. Besides, in order to compare the memory bandwidth among different CU sizes, this contribution normalizes the memory bandwidth numbers by the number of pixels of a given CU, as follows:

Fyuv(W, H) = ( F(W, H, 8, 2) + F(W>>1, H>>1, 4, 1) + F(W>>1, H>>1, 4, 1) ) / (W * H).

- Memory bandwidth analysis.

|  | (VTM-4.0) 4x16 DMVR | (VTM-4.0) 8x8 DMVR | (CE9-1.1a) 8x16/16x8 DMVR | (CE9-1.1b) 8x16/16x8 DMVR |
| --- | --- | --- | --- | --- |
| Bandwidth (Fyuv) | Fyuv(4,16) = 11.3 | Fyuv(8,8) = 10.1 | Fyuv(8,16) = 7.8 Fyuv(16,8) = 7.8 | Fyuv(8+4,16+4) = 11.7 Fyuv(16+4,8+4) = 11.7 |

## Description of the tests

### CE9-1.1: JVET-N0407

This test disables the use of 8x8/4xN CUs for DMVR. In addition, the boundary padding process for  generating the filter input samples within the extended area is removed. As instead, the filter input samples within the extended area are loaded directly from the given reference picture. Specifically, the required number of samples to be loaded for DMVR is changed from (w+7)*(h+7) to (w+7+4)*(h+7+4).

Fig. Filter input samples (i.e. (w+7)*(h+7)) plus padded samples (i.e. plotted as white circles) at the extended area. (Example of 4x4 CU)

### CE9-1.2: JVET-L0291

This test studies the impact of restricting the bit-depth of the interpolated samples used for refinement cost calculations to 8-bits. Specifically, the method proposes to restrict the final output of bilinear interpolation to 8-bits before performing the cost evaluations so that the internal memory requirements are reduced. Also, 8-bit cost evaluation becomes more SIMD-friendly where most software platforms can get a 2x speed-up on the cost evaluations when compared to using the existing bit-depth in the adopted DMVR. In JVET-L0174, it was shown that there was no BDRATE impact and that the decoding time is reduced by 1%. This was scheduled for study in CE9.2.2 at the L-meeting. At the JVET closing plenary at the M-meeting, further study of this was requested and allowed.

# CE9-2: BDOF Specific Simplifications

## Test Specifications

| # | Description | Document |
| --- | --- | --- |
| CE9-2.1 | Disable SAD based early termination | JVET-N0177 |
| CE9-2.2a | The SAD region is divided for each 16x16 sub-blocks for 1st stage of early termination. | JVET-N0187 |
| CE9-2.2b | The SAD region is divided for each 16x16 sub-blocks for 1st stage of early termination. And an early termination for 2nd stage (4x4 block level) is disabled. | JVET-N0187 |
| CE9-2.2c | The early termination for 1st stage (CU level) is disabled. | JVET-N0187 |
| CE9-2.3a | When a CU contains more than 256 luma samples, it is split into multiple sub-blocks with 256 luma samples and BDOF is performed for each sub-block. | JVET-N0270 |
| CE9-2.3b | SAD is calculated only for 4 corners (instead of all 16 luma samples) of each 4x4 sub-block. | JVET-N0270 |
| CE9-2.4 | Implicitly split BDOF application region along 16x16 boundaries | JVET-N0178 |
| CE9-2.5 | Simplification of BDOF’s optical flow parameter derivation | JVET-N0198 |
| CE9-2.6 | CE9.2.1 + CE9.2.4 | JVET-N0178 |

## Summary of results

| Test | Description | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- |
| CE9-2.1 | Disable SAD based early termination | -0.02% | -0.01% | 0.00% | 101% | 104% |
| CE9-2.2a | The SAD region is divided for each 16x16 sub-blocks for 1st stage of early termination. | 0.03% | -0.01% | -0.03% | 100% | 102% |
| CE9-2.2b | The SAD region is divided for each 16x16 sub-blocks for 1st stage of early termination. And an early termination for 2nd stage (4x4 block level) is disabled. | 0.03% | 0.00% | -0.02% | 101% | 102% |
| CE9-2.2c | The early termination for 1st stage (CU level) is disabled. | 0.00% | -0.01% | 0.01% | 101% | 103% |
| CE9-2.3a | When a CU contains more than 256 (luma samples, it is split into multiple sub-blocks with 256 luma samples and BDOF is performed for each sub-block. | 0.01% | -0.01% | -0.02% | 100% | 100% |
| CE9-2.3b | SAD is calculated only for 4 corners (instead of all 16 luma samples) of each 4x4 sub-block. | 0.01% | -0.03% | -0.03% | 99% | 100% |
| CE9-2.4 | Implicitly split BDOF application region along 16x16 boundaries | 0.01% | 0.00% | -0.01% | 101% | 100% |
| CE9-2.5 | Simplification of BDOF’s optical flow parameter derivation | 0.10% | 0.04% | 0.06% | 99% | 100% |
| CE9-2.6 | CE9.2.1 + CE9.2.4 | 0.00% | 0.00% | -0.01% | 102% | 104% |

## Crosscheck Results

|  |  | Tester |  | Crosscheck |  | Crosschecker comments |
| --- | --- | --- | --- | --- | --- | --- |
| Test | Crosschecker | EncT | DecT | EncT | DecT |  |
| CE9-2.1 | X. Xiu | 101% | 104% | 101% | 105% | The software implementation is aligned with CE description by disabling both CU-level and sub-CU-level SAD-based BDOF early termination. |
| CE9-2.2a | T. Chujoh | 100% | 102% | 100% | 103% | As CE description, the unit of early termination and BDOF is changed to 16x16 block. |
| CE9-2.2b | T. Chujoh | 101% | 102% | 101% | 103% | As CE description, the unit of early termination and BDOF is changed to 16x16 block and 4x4 early termination is disabled. |
| CE9-2.2c | T. Chujoh | 101% | 103% | 100% | 103% | As CE description, the CU based early termination is not used. |
| CE9-2.3a | T. Chujoh | 100% | 100% | 101% | 101% | As CE description, the unit size of MC is changed when BDOF applies. |
| CE9-2.3b | T. Chujoh | 99% | 100% | 100% | 100% | As CE description, SAD cost function is changed. The encoding and decoding times were not changed. |
| CE9-2.4 | K. Kondo/X. Xiu | 101% | 100% | 101% 102% | 101% 101% | When an application region of BDOF is larger than 16x16, the region is split into sub-block. |
| CE9-2.5 | S. Sethuraman | 99% | 100% | 100% | 100% | CE is on top of CE9-2.2c to obey section 2.4. C and SIMD versions are not matching, Results are based on SIMD as informed by proponent. |
| CE9-2.6 | K. Kondo/X. Xiu | 102% | 104% | 101% 102% | 105% 105% | On CE9-2.4, an early termination of BDOF is disabled. |

## Related Contributions

| Doc. # | Related test # | Title |
| --- | --- | --- |
| JVET-N0177 | CE9-2.1 | CE9: Disabling SAD based early termination of BDOF (CE9-2.1) |
| JVET-N0187 | CE9-2.2 | CE9-2.2: On early termination of BDOF |
| JVET-N0270 | CE9-2.3 | CE9-2.3: Modifications on BDOF |
| JVET-N0178 | CE9-2.4 | CE9: Implicit splitting of BDOF application region (CE9-2.4 and CE9-2.6) |
| JVET-N0198 | CE9-2.5 | CE9: Simplification of BDOF’s optical flow parameter derivation (Test 9.2.5) |
| JVET-N0178 | CE9-2.6 | CE9: Implicit splitting of BDOF application region (CE9-2.4 and CE9-2.6) |

## Description of the tests

### CE9-2.1: JVET-N0177

Disabling of SAD based early termination of BDOF is tested.

### CE9-2.2: JVET-N0187

In VTM-3.0, BDOF has 2-stages early termination for CU level and 4x4 block level. As mentioned in JVET-M0073, the CU level early termination requires large buffer size to store inter prediction blocks. To consider the trade-off for hardware complexity and decoding time, it’s tested that the SAD region for early termination is divided for each 16x16 size sub-block to reduce buffer size.

1) The SAD region is divided for each 16x16 sub-blocks for 1st stage of early termination.

2) The SAD region is divided for each 16x16 sub-blocks for 1st stage of early termination. And an early termination for 2nd stage (4x4 block level) is disabled.

3) The early termination for 1st stage (CU level) is disabled.

### CE9-2.3: JVET-N0270

Modify BDOF in two aspects: first, when a CU contains more than 256 luma samples, it is split into sub-blocks and each sub-block contains 256 luma samples. BDOF is performed for each sub-block. Second, SAD of each 4x4 sub-block is calculated using the 4 corner luma samples instead of all 16 luma samples.

### CE9-2.4: JVET-N0178

Splitting BDOF application region into 16x16 sub-regions, if the region is bigger than 16x16 luma samples in size. Same sub-pu splitting algorithm as DMVR is used.

### CE9-2.5: JVET-N0198

This contribution proposes to replace VTM4’s method of calculating BDOF’s optical flow parameter with following:

s1=[i,j]∊Ω absIx1+Ix0.

s2=[i,j]∊Ω absIy1+Iy0.

s3=[i,j]∊Ω signIx1+Ix0*(I1-I0).

s4=[i,j]∊Ω signIy1+Iy0*(I1-I0).

vx =-s3s1.

vy =-s4s2.

If S1 > th & s2 < th

Horizontal flow = Vx, Vertical flow = 0

If S2 > th & s2 < th

Horizontal flow = 0, Veritcal flow = Vy,

Else

Horizontal flow = Vx, Veritcal flow = Vy,

The proposed method removes all multiplication operations.

# Recommendations

- Review all CE tests’ contributions and cross-checks.

- Make decisions on each part on technology under consideration by reviewing CE results.

# Acknowledgement

- Thanks to all proponents for providing CE s/w in time with proper announcement and explanation about modification in the CE9 reflector.

- Thanks to all volunteers for providing the comprehensive cross-checks.

# References

- “Description of Core Experiment 9 (CE9): Decoder Motion Vector Derivation”, JVET-M1029, S. Esenlik, X.Xiu

- “JVET common test conditions and software reference configurations”, JVET-M1010, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring.