INTERNATIONAL ORGANISATION FOR STANDARDISATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC1/SC29/WG11

CODING OF MOVING PICTURES AND AUDIO

ISO/IEC JTC1/SC29/WG11 MPEG2019/M46668

March 2019, Geneva, CH

| Source | Nokia |
| --- | --- |
| Status | Input contribution |
| Title | Comments on the draft requirements on immersive media access and delivery |
| Author | Miska M. Hannuksela, Emre B. Aksu, Ari Hourunranta |

# Abstract

This contribution proposes the following changes to the draft requirements on immersive media access and delivery (MPEG N18134):

- The following new requirement is proposed:

1.x0. It shall be possible to control whether decoded objects of the same timestamp are output separately or spatially arranged onto the same output picture.

- Requirement 1.1 is proposed to be clarified as follows (underlined text indicates the proposed clarifications):

1.1. The decoding interface inputs a single multiplexed bitstream comprising bitstreams(s) of the video objects. The decoding interface shall not require more than simple operations (like concatenation) to multiplex the bitstream(s) corresponding to these objects into the single multiplexed bitstream, by the user of the decoding resources

NB: This requirement enables decoding of all objects by feeding a single bitstream through a single decoding resource.

NB: The multiplexed bitstream is not required to contain all video objects that were encoded. The user of the decoding resources selects which video objects are included into the multiplexed bitstream.

- It is proposed to replace requirement 1.2 ("It shall be possible to maintain absolute, frame-level sync between these decoded objects") with the following requirements:

1.x1. The decoding interface shall provide a possibility to output decoded objects in decoding order (possibly out of presentation order).

1.x2. When video object bitstreams have a non-contradicting decoding order, the decoding interface shall provide a possibility to output decoded objects in presentation order with frame-level sync across video objects.

A non-contradicting decoding order of two bitstreams is such that when only those coded objects that have the same presentation timestamp are kept in both bitstreams, the coded objects appear in the same order in both bitstreams with respect to their presentation timestamps. This definition allows one bitstream may have low picture rate than the other.

# Background: immersive media access and delivery

## Overview

The draft requirements on immersive media access and delivery are described in MPEG N18134 [1] and a presentation of the topic is available in MPEG M46578 [2]. A key message is a request to avoid the "shoehorning approach" of merging motion-constrained tile sets (MCTSs) of HEVC into the same coded picture, depicted in Figure 1. In many applications, such shoehorning is unnecessary since the decoded pictures are not displayed as such but undergo a rendering process. Furthermore, MCTSs require specific encoding constraints, and merging of MCTSs into the same coded picture requires rewriting of some header and parameter set data.

Figure 1. Shoehorning of HEVC MCTSs into the same coded picture (source: MPEG M46578).

Instead of shoehorning, it is suggested in the MPEG N18134 and M46578 that decoders should offer the capability of decoding several sub-picture sequences independently with the same decoder instance as illustrated in Figure 2.

Figure 2. Decoding of sub-picture sequences independently (source: MPEG M46578).

## Draft requirements

The draft requirements on video decoding interface are copied from MPEG N18134 below:

In the context of these requirements, an “Object” is defined as a visual element that is independently decodable and represented by its own fully conformant bitstream. This is a draft definition that implies that there is no “subbitstream” or something similar.

The decoding interface shall allow multiple independent video objects to be decoded

The decoding interface shall not require rewriting of the bitstream(s) corresponding to these objects, by the user of the decoding resources

NB: bitstream rewriting refers to more than simple operations (like concatenation) that would be required to decode all objects by feeding a single bitstream through a single decoding resource

It shall be possible to maintain absolute, frame-level sync between these decoded objects

The maximum number of objects shall be a function of the total number of macroblocks/CTU/samples/pixels per second that the decoding resources can process

Note: This should allow for a number of objects that is at least an order of magnitude more than the amount of decoders that can typically be instantiated in a device today (e.g. 4).

Note: it is understood that the number of pixels that a decoder can handle will be constrained by a Level specification, and that this Level specification also constrains the number of objects.

It shall be possible to simultaneously decode objects with different parameters:

    - different object sizes and, for rectangular objects, aspect ratios

    - different framerates

e.g., have a 30 fps fall-back layer in tiled VR streaming, ad a 90 fps foreground (viewport) layer

    - Different resolutions (sampling density)

e.g. have a mix of 4K, 6K and 8K representations for different parts of the same 8K ERP or cubemap.

The interface shall support decoding objects that have different image parameters, such as color space and bit depth.

The interface shall support maintaining the association of metadata with the decoded object

It shall be possible to keep objects in absolute (frame-level) sync with their associated metadata

The conformance requirements for the decoding interface shall include a well-defined maximum decoding latency for all objects on a per-frame basis

It shall be possible to do all of the above with protected content

It should be possible to mix protected and non-protected content

Note: whether support for different keys is required for these objects is for discussion, and it may not impact the decoding interface.

The decoding interface shall allow efficient handling by graphics processors

Note: this might be done by, e.g., allowing decoded objects to be packed into texture buffer(s) as appropriate

The following requirement is perhaps not on the decoding interface, but is nevertheless important for the system:

It shall be possible to manipulate decoded objects without having access to the actual pixel information

Note: this is to be able to manipulate objects in a secure pipeline. Keeping with the tiling example: this would allow placing protected tiles in the right position on a sphere.

# Proposal

## Compositing an output picture

### Issues

It is asserted that there are environments and use cases where composing a single output picture per a presentation timestamp is desirable. These environments and use cases include the following:

- After decoding many types of content are processed further as single pictures. The region-of-interest extraction use case described in JVET-M0536 [3] is selected as an example where composing output pictures is believed to be more reasonable than outputting video objects separately. In the example depicted in Figure 3 four video objects are encoded. When all four video objects are received, it is reasonable that the decoder outputs a single picture per each output timestamp. When the decoder passes all decoded picture content of the same presentation timestamp to the renderer in one buffer, the renderer operation can omit buffering for synchronization and thus be simpler.

Figure 3 - example of ROIs (source: JVET-M0536)

- In some applications it is reasonable to encode video objects separately to achieve better compression and/or to turn off loop filtering across boundaries between video objects. However, video objects are always intended to be "consumed" together and hence are stored in a single bitstream and a single track in the encapsulating file. The metadata in the file format or media description could be such that it describes the video objects collectively.

For example, it may be reasonable to encode the cube faces or the cube face rows of cubemaps separately to allow motion vectors over face or face row boundaries. In viewport-independent storage or streaming, all cube faces of the cubemap are stored in the same bitstream and in the same file format track and described with track-specific file format metadata. In this approach, the metadata can simply indicate that the bitstream/track contains a cube map, and individual cube faces or cube face rows need not be separately described by the metadata (e.g. through region-wise packing and/or content coverage).

### Proposed requirement

1.x It shall be possible to control whether decoded objects of the same timestamp are output separately or spatially arranged onto the same output picture.

## Single-stream input

### Issues

It is asserted that in many video processing/decoding APIs, the input data to the decoder is logically a single bitstream. In other words, APIs do not include means for identifying separate video object streams. While MPEG N18134 could be interpreted to follow the assumption of inputting a single bitstream into the decoder, the phrasing could be more explicit.

### Proposed clarified requirement

Clarify requirement 1.1 as follows (underlined text indicates the proposed clarifications):

1.1.	The decoding interface inputs a single multiplexed bitstream comprising bitstreams(s) of the video objects. The decoding interface shall not require more than simple operations (like concatenation) to multiplex the bitstream(s) corresponding to these objects into the single multiplexed bitstream, by the user of the decoding resources

NB: This requirement enables decoding of all objects by feeding a single bitstream through a single decoding resource.

NB: The multiplexed bitstream is not required to contain all video objects that were encoded. The user of the decoding resources selects which video objects are included into the multiplexed bitstream.

## Decoder output in decoding order

### Issues

MPEG N18134 and M46578 imply that the bitstreams of the video objects are not required to have the same inter prediction hierarchy. In other words, the decoding order of pictures relative to the output order of pictures may differ in the bitstreams. MPEG N18134 anyhow includes a requirement to output the decoded objects synchronously. It is asserted that the following challenges arise from these requirements:

- The initial delay and size of the decoded object buffer depends on which other objects are fed to the same decoder instance. Since the selection of video objects changes dynamically in many of the immersive media applications (e.g. VR video streaming), it is challenging to determine a proper initial delay and size for the decoded object buffer.

A similar implication on decoded picture buffer was studied in Nokia's SHVC call-for-proposals response, which included the possibility to use different inter prediction hierarchies in different layers of the scalable video bitstream. The CfP response with the uneven inter prediction hierarchy configuration was described in JCTVC-K0047 [4], and a presentation was included in JCTVC-K0040 [5]. The topic was studied further in JCTVC-L0171 [6].

A consequence of allowing different inter prediction hierarchies for different video objects is that in a multiplexed bitstream the coded video objects of different bitstreams would not necessarily appear in ascending decoding timestamp. It would not be possible to define the concept of access unit as a contiguous piece of coded data (in the decoding order within the multiplexed bitstream) having the same presentation timestamp.

- Immersive media applications typically require further processing of the decoder output pictures prior to rendering. For example, in V-PCC a 3D point cloud is reconstructed from the decoded pictures of different types (texture, geometry, occupancy) and auxiliary information. Outputting decoded pictures from the video decoder in their decoding order would enable architectures where some processing for rendering happens immediately (in decoding order) to reduce overall latency.

### Proposed requirements

It is proposed to replace requirement 1.2 ("It shall be possible to maintain absolute, frame-level sync between these decoded objects") with the following requirements:

1.x1. The decoding interface shall provide a possibility to output decoded objects in decoding order (possibly out of presentation order).

1.x2. When video object bitstreams have a non-contradicting decoding order, the decoding interface shall provide a possibility to output decoded objects in presentation order with frame-level sync across video objects.

A non-contradicting decoding order of two bitstreams is such that when only those coded objects that have the same presentation timestamp are kept in both bitstreams, the coded objects appear in the same order in both bitstreams with respect to their presentation timestamps. This definition allows one bitstream may have low picture rate than the other.

# References

- E. Thomas, T. Stockhammer, R. Koenen (editors), "Draft requirements on immersive media delivery and access," MPEG N18134, 
http://phenix.it-sudparis.eu/mpeg/doc_end_user/documents/125_Marrakech/wg11/w18134.zip.

- E. Thomas, R. Koenen, T. Stockhammer, "On the decoding interface for immersive media," MPEG M46578, 
http://phenix.it-sudparis.eu/mpeg/doc_end_user/documents/125_Marrakech/wg11/m46578-v3-m46578-OnthedecodinginterfaceforImmersiveMedia.zip.

- E. Thomas, A. Gabriel, "AHG12: On picture-level tiles and sequence-level tiles for VVC," JVET-M0536, 
http://phenix.it-sudparis.eu/jvet/doc_end_user/documents/13_Marrakech/wg11/JVET-M0536-v1.zip.

- O. Bici, D. Bugdayci, A. Aminlou, A. Hallapuro, M. Hannuksela, J. Lainema, K. Ugur, "Description of scalable video coding technology proposal by Nokia (encoder configuration 2)," JCTVC-K0047,
http://phenix.it-sudparis.eu/jct/doc_end_user/documents/11_Shanghai/wg11/JCTVC-K0047-v2.zip.

- O. Bici, D. Bugdayci, A. Aminlou, A. Hallapuro, M. Hannuksela, J. Lainema, K. Ugur, "Description of scalable video coding technology proposal by Nokia (encoder configuration 1)," JCTVC-K0040, 
http://phenix.it-sudparis.eu/jct/doc_end_user/documents/11_Shanghai/wg11/JCTVC-K0040-v3.zip.

- M. M. Hannuksela, A. Hallapuro, K. Ugur, "SHVC HLS: support for unequal BL and EL GOP lengths," JCTVC-L0171, http://phenix.it-sudparis.eu/jct/doc_end_user/documents/12_Geneva/wg11/JCTVC-L0171-v2.zip.