| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0068-v1 |
| --- | --- |

| Title: | CE2: On restriction of memory bandwidth consumption of affine mode (CE2-4.8) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minhua Zhou 16340 West Bernardo DR San Diego, CA-92127, USA | Tel: Email: | +1 858 521 5838 minhua.zhou@broadcom.com |
| Source: | Broadcom Inc. |  |  |

_____________________________

# Abstract

To constrain the memory bandwidth consumption of the affine mode at PU level, it is proposed to impose a PU level constraint on the decoder side. If it is detected that the affine model parameters lead to the reference block bounding box sizes exceeding pre-defined thresholds, a fallback is triggered in which a single motion vector is used for a PU coded in affine mode. The single vector is generated at the center point of the PU by using the affine motion model parameters. Otherwise, the PU is processed according to the specification of the affine mode in the VTM4.0.

For the setting of threshold 17*17 for 2x2 sub-block vectors of bi-directional affine mode and  17*13 for 2x1 and 1x2 sub-block vectors of unidirectional affine mode, the average BD-differences compared to the VTM4.0 are as follows:

- For Test CE2-4.8a in which the fallback mode is not triggered, the BD-rate difference is 0.02% in RA and 0.00% in LD_B, respectively.

- For Test CE2-4.8b in which the fallback mode is triggered , the BD-rate difference is -0.01% in RA and 0.01% in LD_B, respectively.

# Introduction

The affine motion compensation prediction introduces a more complex motion model for better compression efficiency. In the affine motion model, the motion vector v=(vx,vy) at a sample position (x,y) inside a PU is defined as:

vx=ax+cy+evy=bx+dy+f               Equation 1

Where a,b,c,d,e   and f are the affine motion model parameters. In the VTM4.0, 4x4 sub-block motion vectors are generated and used in the motion compensation of a PU coded in affine mode

The model parameters a,b,c,d determine how far the sub-block motion vectors can go apart from each other within a PU. In the worst case, the sub-block vectors can go far apart, resulting in zero overlapping among reference blocks used for the motion compensation, and hence non-affordable peak memory bandwidth consumption of the affine mode.

Efforts are being made to constrain the worst case memory bandwidth consumption of the affine mode by the sub-block vector clipping or by the 8x8/4x8/8x4 sub-block size adaptation [3]. Those methods have one thing in common, namely, they just constrain the sub-block motion vector spread (i.e. how far the sub-block vectors can go apart from each other) within the 8x8 non-overlapped regions of a PU, but not the affine motion model parameters. Within the PU the sub-block motion vectors from one 8x8 region to the next 8x8 region can still go widely apart, which can lead to a huge peak memory bandwidth consumption in the worst case. Figure 1 depicts such an example, in which sub-block motion vector spread is constrained within 8x8 regions of a PU and hence a tight reference block bounding box can be drawn for each 2x2 sub-block vector group, but the reference block bounding boxes can still be completely non-overlapped in the worst case.

Figure 1: Reference block bounding boxes can still go far apart even if the sub-block motion vector spared is constrained within each 8x8 region of a PU

Unlike the regular motion vectors which can be fully independent from block to block, the sub-block motion vectors of the affine mode are interconnected by a linear equation, which provides an opportunity to constrain the sub-block motion vector spread at the PU level to avoid the above mentioned problem.

The design should consider the following aspects:

- Constrain the memory bandwidth of the affine mode by enabling enough reference block overlapping from block to block.

- Reduce the number of reference patch fetches from the cache memory for motion compensation.

- Reduce the computational complexity caused by the 4x4 block based affine motion compensation

In JVET-M0049 [4], a bitstream constraint is proposed to address aspects 1) and 2). However, a bitstream constraint in general is not safe from the decoder design point of view, and could also be too restrictive from the encoder point of view for sequences with large affine motion. It is more desirable to impose the constraint on the decoder side.

# Algorithm description

To impose the same constraint proposed in [4] on the decoder side, the following algorithm is tested. The decoder checks whether the following conditions are satisfied:

- If a PU uses bi-directional affine mode (i.e. bi-pred), the reference block bounding box size bxW4*bxH4 is less than or equal to Thred4.

- Otherwise, if the PU is coded in unidirectional affine mode, the reference block bounding box size bxWh*bxHh is less than or equal to Thredh and the reference block bounding box size bxWv*bxHv is less than or equal to Thredv.

If yes, the decoder generates the sub-block vectors of the PU in the same way as defined in the VTM4.0. Otherwise, all the sub-block vectors of the PU are set to the same vector which is computed at the center position of the PU by using the affine motion model.

As defined in JVET-M0049, the width and height of the reference block bounding box of 2x2, 2x1 and 1x2 sub-block vectors, i.e. bxW4,bxH4, bxWh,bxHh and (bxWv,bxHv), are computed by:

Figure 2: Illustration of reference block bounding box of 2x2 sub-block motion vectors in a PU coded in bi-directional affine mode

bxW4=max0, 41+a, 4c,41+a+4c-min0, 41+a, 4c,41+a+4c+11bxH4=max0, 4b, 41+d,4b+41+d-min 0, 4b, 41+d,4b+41+d+11

Figure 3: Illustration of reference block bounding box of 2x1 sub-block motion vectors in a PU coded in unidirectional affine mode (horizontal direction)

bxWh=max0, 41+a-min0, 41+a+11bxHh=max0, 4b -min0, 4b +11

Figure 4: Illustration of reference block bounding box of 1x2 sub-block motion vectors in a PU coded in unidirectional affine mode (vertical direction)

bxWv=max0, 4c-min0, 4c+11                                               bxHv=max0, 41+d -min0, 41+d +11

Where the affine motion model is defined as:

vx=ax+cy+evy=bx+dy+f

The thresholds for a PU coded in unidirectional and bidirectional affine mode are defined as:

Thred4=(15+δx)*(15+δy)Thredh=(15+δx)*(11+δy)Thredv=(11+δx)*(15+δy)

Where δx and δy are parameters used for controlling the thresholds.

# Experimental results

Tests were carried out by using the VTM4.0 software, and the common test conditions (CTC) defined in [1] was followed. In the tests, threshold of luma reference block bounding box is set to 17*17 for 2x2 sub-block vectors of bi-directional affine mode, and to 17*13 for both 2x1 and 1x2 sub-block vectors of unidirectional affine mode (i.e. setting δx=δy=2), respectively.

The following two tests were carried out:

- Test CE2-4.8a : the encoder controls the affine motion estimation so that the fallback mode is never triggered in the bitstreams.

- Test CE2-4.8b : the encoder allows the use of fallback mode if the reference block bounding box sizes exceed the pre-defined thresholds.

As shown in Table 1, for Test CE2-4.8a in which the fallback mode is not triggered, the BD-rate difference is 0.02% in RA and 0.00% in LD_B, respectively. For Test CE2-4.8b in which the fallback mode is triggered (see Table 2), the BD-rate difference is -0.01% in RA and 0.01% in LD_B, respectively. Triggering the fallback mode on the encoder side appears to be providing slightly better coding efficiency.

Note that the run time may not be accurate due to the heterogeneous nature of the LSF farm.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.01% | 0.06% | 90% | 93% |
| Class A2 | 0.05% | 0.02% | 0.05% | 101% | 105% |
| Class B | 0.02% | -0.02% | 0.06% | 89% | 89% |
| Class C | 0.01% | 0.00% | -0.03% | 105% | 106% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.00% | 0.03% | 95% | 97% |
| Class D | 0.01% | 0.02% | -0.06% | 95% | 102% |
| Class F (optional) | 0.05% | 0.07% | 0.03% | 94% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.02% | -0.16% | 98% | 98% |
| Class C | -0.04% | 0.26% | 0.13% | 100% | 95% |
| Class E | 0.02% | 0.39% | -0.67% | 73% | 85% |
| Overall | 0.00% | 0.18% | -0.19% | 92% | 94% |
| Class D | -0.05% | -0.33% | -0.48% | 93% | 91% |
| Class F (optional) | -0.16% | -0.17% | -0.72% | 91% | 85% |

Table 1: Summary results of Test CE2-4.8a  ( δx=δy=2)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.03% | 0.03% | 104% | 104% |
| Class A2 | -0.01% | 0.00% | 0.01% | 101% | 97% |
| Class B | 0.00% | 0.01% | -0.02% | 98% | 97% |
| Class C | -0.02% | -0.07% | 0.02% | 93% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.01% | 0.01% | 98% | 99% |
| Class D | -0.01% | -0.11% | -0.03% | 95% | 103% |
| Class F (optional) | -0.01% | 0.03% | -0.04% | 91% | 97% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.02% | -0.20% | 111% | 107% |
| Class C | -0.02% | 0.34% | -0.23% | 105% | 100% |
| Class E | 0.04% | 0.18% | -0.40% | 86% | 90% |
| Overall | 0.01% | 0.17% | -0.26% | 102% | 100% |
| Class D | 0.00% | -0.08% | -0.67% | 106% | 107% |
| Class F (optional) | -0.09% | -0.31% | -0.58% | 105% | 95% |

Table 2: Summary results of Test CE2-4.8b  ( δx=δy=2)

# Complexity analysis

In terms of computational complexity, the 8x8 bi-directional affine mode is the worst case. For list0 and list 1, a total of 16 additions, 24 max/min operations, 2 comparisons and 2 multiplications are needed for a PU coded in affine mode.   It is assumed that shifts come for free when loading the data to the registers. The detailed analysis is provided in Table 3.

| Operations | #adds | #max/min | #comparisons | #multiplications |
| --- | --- | --- | --- | --- |
| 4(1+a) | 1 |  |  |  |
| 4(1+a)+4c | 1 |  |  |  |
| 4(1+d) | 1 |  |  |  |
| 4b + 4(1+d) | 1 |  |  |  |
| Maximum |  | 6 |  |  |
| Minimum |  | 6 |  |  |
| Maximum – Minimum +11 | 4 |  |  |  |
| bxW4*byW4 |  |  |  | 1 |
| bxW4*bxH4≤Thred4 |  |  | 1 |  |
| Overall (for a list) | 8 | 12 | 1 | 1 |

Table 3: Computational complexity analysis

Table 4: Memory bandwidth per sample analysis using CE2 template spreadsheet

The CE2 template spreadsheet is used for computing the memory bandwidth number per sample. The filter taps in the table are adjusted to integrate the impact of the sub-block motion vector spread. The bold row in Table 4 is the number for the 8x8 bidirectional prediction.

| Bandwidth measurement for uni/bi-prediction | Operations of SB MV derivation per 8x8 block (e.g. clipping) | Complexity of MC per 8x8 bi-prediction: Multiplication, addition | Others (additional operations per CU to determine SB size, MC fetching unit) Multiplication, addition, comparison, look-up size |
| --- | --- | --- | --- |
| See section 5 and Table 4 | Not applicable | Not applicable in the worst case (i.e. the fallback mode is not triggered). The fallback mode consumes ~30% less computation when compared to the 4x4 based MC used by the affine mode, as the MC in the fallback mode is 8x8 based | See Table 3 16 additions, 24 max/min operations, 2 comparisons and 2 multiplications per 8x8 block in the worst case |

Table 5: Complexity Analysis

# Memory bandwidth measurements

We conducted a memory bandwidth study by running both the CE2-4.8 software and the VTM4.0 with a commercial motion compensation cache model integrated. The Random Access (RA) summary results of Test CE2-4.8a and Test CE2-4.8b are provided in the Table 6 to Table 9. Additional tests were conducted for ( δx=δy=3) for both Test CE2-4.8a and Test CE2-4.8b to study the trend when the restrictions are loosened.

As shown in Tables below, the restrictions do not have an impact on memory bandwidth consumption for the CTC sequences when compared to VTM4.0 (i.e. w/o restrictions) according to  the “MBW_diff” measurements.

|  | Random Access Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |  |  |  |
|  | Y | U | V | TCM_diff | ABW_diff | MBW_diff | EncT | DecT |
| Class A1 | 0.02% | -0.01% | 0.06% | -0.06% | -0.11% | -0.32% | 90% | 108% |
| Class A2 | 0.05% | 0.02% | 0.05% | -0.02% | -0.16% | 0.32% | 101% | 115% |
| Class B | 0.02% | -0.02% | 0.06% | -0.02% | -0.04% | 0.00% | 89% | 116% |
| Class C | 0.01% | 0.00% | -0.03% | 0.02% | 0.02% | 0.46% | 105% | 113% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | 0.02% | 0.00% | 0.03% | -0.03% | -0.05% | 0.46% | 95% | 113% |
| Class D | 0.01% | 0.02% | -0.06% | -0.03% | 0.00% | -0.21% | 95% | 114% |
| Class F (optional) | 0.05% | 0.07% | 0.03% | -0.04% | 0.00% | 0.31% | 94% | 115% |

Table 6: Memory bandwidth measurement for Test CE2-4.8a  ( δx=δy=2)

|  | Random Access Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |  |  |  |
|  | Y | U | V | TCM_diff | ABW_diff | MBW_diff | EncT | DecT |
| Class A1 | 0.01% | 0.03% | 0.03% | -0.04% | -0.04% | -0.32% | 104% | 99% |
| Class A2 | -0.01% | 0.00% | 0.01% | 0.03% | -0.03% | 0.32% | 101% | 124% |
| Class B | 0.00% | 0.01% | -0.02% | -0.02% | 0.02% | -0.56% | 98% | 109% |
| Class C | -0.02% | -0.07% | 0.02% | 0.04% | 0.11% | 0.70% | 93% | 117% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.01% | -0.01% | 0.01% | 0.01% | 0.03% | 0.70% | 98% | 112% |
| Class D | -0.01% | -0.11% | -0.03% | -0.03% | 0.02% | -0.21% | 95% | 121% |
| Class F (optional) | -0.01% | 0.03% | -0.04% | 0.08% | 0.08% | 0.62% | 91% | 112% |

Table 7: Memory bandwidth measurement for Test CE2-4.8b  ( δx=δy=2)

|  | Random Access Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |  |  |  |
|  | Y | U | V | TCM_diff | ABW_diff | MBW_diff | EncT | DecT |
| Class A1 | 0.01% | 0.02% | -0.01% | -0.05% | 0.00% | -0.32% | 100% | 101% |
| Class A2 | 0.02% | -0.07% | 0.06% | 0.00% | -0.03% | 0.63% | 105% | 111% |
| Class B | 0.02% | 0.01% | -0.01% | 0.00% | 0.08% | 0.37% | 92% | 105% |
| Class C | 0.02% | 0.03% | -0.03% | 0.04% | 0.05% | 0.70% | 93% | 110% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | 0.02% | 0.00% | 0.00% | 0.00% | 0.02% | 0.70% | 97% | 107% |
| Class D | 0.01% | 0.03% | 0.00% | -0.02% | 0.00% | -0.21% | 95% | 119% |
| Class F (optional) | 0.01% | 0.03% | -0.03% | 0.02% | 0.05% | -0.93% | 93% | 116% |

Table 8: Memory bandwidth measurement for Test CE2-4.8a  ( δx=δy=3)

|  | Random Access Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |  |  |  |
|  | Y | U | V | TCM_diff | ABW_diff | MBW_diff | EncT | DecT |
| Class A1 | 0.00% | -0.06% | -0.07% | 0.00% | 0.00% | -0.32% | 92% | 111% |
| Class A2 | -0.01% | -0.08% | -0.02% | 0.04% | 0.00% | 0.32% | 89% | 114% |
| Class B | -0.01% | -0.08% | -0.03% | -0.01% | 0.00% | 0.74% | 96% | 111% |
| Class C | -0.01% | -0.08% | -0.03% | 0.04% | 0.05% | 0.70% | 93% | 117% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.01% | -0.08% | -0.04% | 0.02% | 0.02% | 4.04% | 93% | 113% |
| Class D | 0.00% | 0.00% | -0.02% | 0.00% | 0.00% | -1.03% | 89% | 130% |
| Class F (optional) | -0.01% | 0.04% | -0.01% | 0.07% | 0.08% | 4.04% | 101% | 123% |

Table 9: Memory bandwidth measurement for Test CE2-4.8b  ( δx=δy=3)

Where

- TCM_diff : Total cache misses (over all the frames coded), percentage increase relative to VTM4.0.

- ABW_diff: Average memory bandwidth (over all the frames coded), percentage increase relative to VTM4.0.

- MBW_diff : Worst case (Max) memory bandwidth (among all the frames coded), percentage increase relative to VTM4.0.

# References

[1] J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-M1010, the 13th JVET meeting, Jan. 2019, Marrakesh, MA

[2] B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 3)”, JVET-M1001-v6, the 13th JVET meeting, Jan. 2019, Marrakesh, MA

[3] Y. He, C.-Y. Chen and C-C Chen, “Description of Core Experiment 6 (CE2): Sub-block based motion prediction”, JVET-M1022, the 13th JVET meeting, Jan. 2019, Marrakesh, MA

[4] M. Zhou, “A restriction on memory bandwidth consumption of affine mode”, JVET-M0049, the 13th JVET meeting, Jan. 2019, Marrakesh, MA

# Patent rights declaration(s)

Broadcom Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Spec Text Changes relative to JVET-M1001-v6

In section 8.5.5.9 Derivation process for motion vector arrays from affine control point motion vectors,

Text for CE2-4.8a and CE2-4.8b as decoder conformance check

Replace

For xSbIdx = 0..numSbX − 1 and ySbIdx = 0..numSbY − 1, the following applies:

- The luma motion vector mvLX[ xSbIdx ][ ySbIdx ] is derived as follows :

xPosCb = 2 + ( xSbIdx << 2 )		(8-776)

yPosCb = 2 + ( ySbIdx << 2 )		(8-777)

mvLX[ xSbIdx ][ ySbIdx ][ 0 ] = ( mvScaleHor + dHorX * xPosCb + dHorY * yPosCb )	(8-778)

mvLX[ xSbIdx ][ ySbIdx ][ 1 ] = ( mvScaleVer + dVerX * xPosCb + dVerY * yPosCb )	(8-779)

with (new items marked in yellow)

The variables (bxWX4,bxHX4) , (bxWXh,bxHXh) and (bxWXv,bxHXv) are derived as follows:

maxW4=max0, 42048+dHorX,4dHorY,42048+dHorX+4dHorYminW4=min0, 42048+dHorX,4dHorY,42048+dHorX+4dHorYmaxH4=max0, 4dVerX, 42048+dVerY,4dVerX+42048+dVerYminH4=min0, 4dVerX, 42048+dVerY,4dVerX+42048+dVerY  (8-xxx)

bxWX4=(maxW4-minW4≫11)+11bxHX4=(maxH4-minH4≫11)+11                                                            (8-xxx)

bxWXh=(max0, 42048+dHorX-min0, 42048+dHorX≫11)+11bxHXh=(max0, 4dVerX-min0, 4dVerX≫11)+11                                          (8-xxx)

bxWXv=(max0, 4dVerY-min0, 4dVerY≫11)+11                                         bxHXv=(max0, 42048+dHorY-min0, 42048+dHorY≫11)+11    (8-xxx)

The variable isFallbackModeTriggered is set equal to 1, and is derived as follows:

- If predFlagL0[ xCb ][ yCb] is equal to 1 and predFlagL1[ xCb ][ yCb] is equal to 1, and bxWX4*bxHX4 is less than or equal to 289,  isFallbackModeTriggered is set equal to 0.

- Otherwise, if predFlagL0[ xCb ][ yCb] is equal to 1 or predFlagL1[ xCb ][ yCb] is equal to 1, and both  bxWXh*bxHXh  and bxWXv*bxHXv  are less than or equal to 221, isFallbackModeTriggered is set equal to 0.

For xSbIdx = 0..numSbX − 1 and ySbIdx = 0..numSbY − 1, the following applies:

- The luma motion vector mvLX[ xSbIdx ][ ySbIdx ] is derived as follows :

  - If  isFallbackModeTriggered is equal to 1, the variables  xPosCb and yPosCb are derived as follows:

xPosCb =  (cbWidth>>1)		(8-776)

yPosCb =  (cbHeight>>1)		(8-777)

  - Otherwise, the variables  xPosCb and yPosCb are derived as follows:

xPosCb = 2 + ( xSbIdx << 2 )		(8-776)

yPosCb = 2 + ( ySbIdx << 2 )		(8-777)

mvLX[ xSbIdx ][ ySbIdx ][ 0 ] = ( mvScaleHor + dHorX * xPosCb + dHorY * yPosCb )	(8-778)

mvLX[ xSbIdx ][ ySbIdx ][ 1 ] = ( mvScaleVer + dVerX * xPosCb + dVerY * yPosCb )	(8-779)

Text for CE2-4.8a as bitstream restriction

Add (new items marked in yellow)

It is a requirement of bitstream conformance that

- If predFlagL0[ xCb ][ yCb] is equal to 1 and predFlagL1[ xCb ][ yCb] is equal to 1, bxWX4*bxHX4 shall be  less than or equal to 289

- Otherwise, if predFlagL0[ xCb ][ yCb] is equal to 1 or predFlagL1[ xCb ][ yCb] is equal to 1, both  bxWXh*bxHXh  and bxWXv*bxHXv  shall be less than or equal to 221

Where the values of (bxWX4,bxHX4) , (bxWXh,bxHXh) and (bxWXv,bxHXv) are computed by:

maxW4=max0, 42048+dHorX,4dHorY,42048+dHorX+4dHorYminW4=min0, 42048+dHorX,4dHorY,42048+dHorX+4dHorYmaxH4=max0, 4dVerX, 42048+dVerY,4dVerX+42048+dVerYminH4=min0, 4dVerX, 42048+dVerY,4dVerX+42048+dVerY  (8-xxx)

bxWX4=(maxW4-minW4≫11)+11bxHX4=(maxH4-minH4≫11)+11                                                            (8-xxx)

bxWXh=(max0, 42048+dHorX-min0, 42048+dHorX≫11)+11bxHXh=(max0, 4dVerX-min0, 4dVerX≫11)+11                                          (8-xxx)

bxWXv=(max0, 4dVerY-min0, 4dVerY≫11)+11                                         bxHXv=(max0, 42048+dHorY-min0, 42048+dHorY≫11)+11    8-xxx