| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0080-v2 |
| --- | --- |

| Title: | AHG14: Revised software for ultra low-latency encoding |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kimihiko Kazui | Tel: Email: | +81 44 754 2049 kazui.kimihiko@fujitsu.com |
| Source: | FUJITSU LABORATORIES LTD. |  |  |

_____________________________

- Abstract

This contribution proposes a revised software for ultra low-latency encoding in accordance with the mandates of AHG14 “Progressive intra refresh”. The first software was proposed in JVET-M0197 [2].

The proposed software is based on VTM-4.0 and incorporates the progressive intra refresh scheme and multiple decoding units to simulate the reduction of the CPB size. The proposed software does not include any normative changes to the current VVC design [1].

This contribution also proposes testing conditions, including parallel processing for reducing the turnaround time of simulation.

In Version 2, test results are updated.

Introduction

The mandates of AHG14 “Progressive intra refresh” for the 14th meeting are as follows;

- Define relevant test conditions for the study of progressive intra refresh for random access without intra frames

- Update the implementation of encoder-only intra refresh in the VTM model in the AHG14 fork of the software repository.

- Evaluate different ways to produce intra refresh within VVC and characterize their coding efficiency impact, subjective quality, and delay characteristics, including encoder-only approaches and normative approaches

- Consider the use of constrained intra prediction and tile-based approaches

- Study recovery point handling, including practical implementation issues and perfect-versus-approximate decoded picture recovery.

- Consider the potential need for starting a coded video sequence without an intra picture.

This contribution mainly focuses on testing conditions and software.

Firstly, the target end-to-end delay and required encoding scheme is discussed. Then the feature of the revised software is explained with test results. Finally, parallel processing is proposed to reduce the turnaround time of simulation.

Target end-to-end delay and encoding scheme

Target end-to-end delay

The target end-to-end delay of the proposed software is around 50ms. Such level of latency is required for remote control applications such as wireless display and vehicle control which involve feedback or operation with the aid of visual information.

It is noted that required end-to-end delay for conventional audio-visual human communication application is around 150ms. Such level of latency is also covered by this contribution.

Delay model

The end-to-end delay of the first decoding unit in a bitstream is modelled as in Figure 1. To simplify the discussion, it is assumed that each access unit has the same number of decoding units M and each decoding unit in the same access unit has the same number of bits and coding units. Moreover, it is assumed that the first access unit (i.e. IDR access unit) has the largest number of bits. If there exists IRAP pictures in the middle of a bitstream, the size of such pictures is assumed to be the same as the IDR picture. With those assumption, the end-to-end delay of the first decoding unit becomes the overall latency of the bitstream.

Figure 1: End-to-end delay of first decoding unit

- The delay of capturing process TC is fixed to one picture interval dT (33ms for 30Hz sequence).

- The delay of encoding process TE is proportional to the number of coding tree units in a decoding unit.TC is equal to dT if each decoding unit contains all coding units of an access unit.

- The delay of transmission TX is equal to ( BDU / bitrate ) where BDU is the number of bits in the first decoding unit.

- The delay of buffering TB is dependent to the size of the CPB. TB is equal to 0 if BDU is equal to the size of the CPB.

- The delay of decoding process TD is proportional to the number of coding tree units in a decoding unit and is assumed equal to TE.

- The delay of output process TO is dependent to the number of decoding units in each access unit. It is equal to dT minus TD. TO is equal to 0 if each access unit contains only one access unit. In other words, ( TD + TO ) is equal to dT.

Modelling of TX, TB, TD and TO with CPB occupancy

Figure 2 shows the CPB occupancy in three cases; 1) AU-based (i.e. M=1), 2) DU-based, 3) DU-based with uniform bit allocation to all access units. In Figure 2 it is assumed that M is equal to 4 and the size of the first IDR access unit (and following IRAP access units) in bits is equal to (N*bitrate/frame_rate), where N is equal to 4. It is noted that the size of each non-IRAP access unit is dependent on the IRAP interval and less than (bitrate/frame_rate).

Figure 2: CPB occupancy and delay

As illustrated in Figure 2, TB is proportional to the size of the first access unit in size. TB can be expressed as follows;

TB = max( 0, dT*( N – ( N + M – 1 ) / M ) )

N = ( BAU * frame_rate / bitrate ) = ( M * BDU * frame_rate / bitrate )

When N is equal to 4 and M is equal to 4, TB is equal to (9/4)*dT.

Encoding configuration required for target end-to-end delay

In order to reduce the end-to-end delay as small as possible, the factor N, which is equal to ( BAU * frame_rate / bitrate ), should be closer to 1 (i.e. equal bit allocation to all access units) so to reduce TB to 0, in addition to increase the number of decoding units in an access unit M.

Regarding the factor N, encoding of random-access picture (including the first IDR picture) in all-intra mode tends to require larger number for N (e.g. 4) to maintain objective quality. In this regard, progressive intra refresh scheme, in which all pictures are encoded in non-all-intra mode, is suitable for reducing N closer to 1.

Regarding the number of decoding units M, each row of coding tree unit can be a decoding unit with similar number of bits if intra coded coding units are located in the vertical striped shape region as explained later.

The end-to-end delay D ( = TC + TE + TX + TB + TD + TO ) with progressive intra refresh scheme is dT * ( 1 + 1/M + 1 /M + 1 ). If the frame rate is 30Hz and the number of rows of coding tree unit is 9 (i.e. 1080p sequence with 128x128 CTU), D is 73ms. If IDR-based refresh is used with N=4, D is 142ms.

Allocation of intra-coded coding units in access unit

The conventional progressive intra refresh scheme refreshes each picture progressively by allocating intra-coded coding units dispersedly in each access units in the same refresh cycle, instead of allocating all intra-coded coding units in one access unit (i.e. IRAP access unit), in order to average the number of bits for each access unit. Since the size of the CPB buffer (i.e. TX and TC) is proportional to the size of the largest access unit in bits, uniform allocation of intra-coded coding unit leads to the minimum CPB size.

The progressive intra refresh scheme can further reduce the end-to-end delay using decoding unit. If the size of each decoding unit in bits and in coding units is averaged in each access unit, then the size of the CPB buffer for decoding unit is minimized to ( 1 / M ), where M is the number of decoding units in an access unit.

Restrictions on the allocation of intra-coded coding unit are as follows;

- Each coding unit of the same spatial position shall be intra-coded at least once in one refresh cycle to enable perfect reconstruction (i.e. “exact match”) at a recovery point picture.

- Each intra-coded coding unit in the same access unit shall be contained in each decoding unit uniformly to average the size of each decoding unit in bits and in number of coding units since an intra-coded coding unit has larger number of bits than an inter-coded coding unit.

Typical allocation of intra-coded coding unit is summarized in Figure 3 It is assumed that the number and the shape of decoding units are the same for all access units.

Figure 3: Typical allocation of decoding unit and intra-coded coding unit

Figure 4: Availability of neighboring sample in intra coding

Figure 4 shows available samples for intra prediction in each intra-coded coding unit allocation. Figure 4 only shows the case when the current intra-coded coding unit locates at a decoding unit boundary. From the coding efficiency point of view, the approach of “isolated region” (i.e. case 3-1 and 3-2) is suitable.

One possible drawback of the approach of “horizontal striped region” and “isolated region” is that the processing order of coding unit becomes different from the typical scanning order of raw sample (i.e. raster-scanning). In this sense, the approach of “vertical striped region” is suitable especially when an encoder can coding-tree-unit-row-based pipeline processing for input and encoding, for example.

From the viewpoint of implementing MV restriction, the approaches 1-1, 2-1 and 1-3 (i.e. the shape of the refresh boundary is a line) are suitable since the decision process of clean region and dirty region is simple.

In this contribution, the approach 1-1 (i.e. allocating intra-coded coding units in one vertically striped region) is used since it is the most straightforward scheme and is used in the previous version. Upon the agreement, implementation of another approach would be taken in the design of the next version.

Software implementation

As explained in section 2, the approach of allocating intra-coded coding unit in one vertically striped region is used in the software implementation. The detail is as follows;

Software base

VTM-4.0 is used as the code base. The proposed ultra-low latency coding is enabled by setting macro JVET_N0080_PIR equal to 1.

Picture partitioning, allocation of intra CU

Figure 5 illustrates how a picture is partitioned into decoding units and where intra coding unit is allocated.

Decoding unit

Each CTU row contains one tile (not tile group) to simulate one-DU-per-CTU-row encoding condition. This is just provisional since the feature of tile group is not integrated into VTM yet. Once it is ready to use tile group in VTM, implementation of decoding unit using tile group will be done in the next version.

Since there is no need to restrict in-loop filtering at horizontal tile boundary, LFCrossTileBoundaryFlag is set to 1.

The proposed software derives the parameters of tile (i.e. NumTileRowsMinus1) from the height of a picture, instead of reading the parameters from the configuration file.

Figure 5: Picture partitioning and allocation of intra-coded coding unit

Position of refresh boundary

A vertical refresh boundary line is virtually defined only at encoder side. No tile boundary is allocated to the position of the refresh boundary since the tile boundary is restricted to be on the 128x128 grid in the current design. The horizontal position of the vertical refresh boundary line is shifting w1 samples to the right in each picture, where w1 is a multiple of the minimum width of a coding unit.

Allocation of intra CU

All coding units with horizontal position in the range of ( XPOS_RB – w1 )..( XPOS_RB – 1 ) are forced to be coded in intra, where XPOS_RB is the horizontal position of the refresh boundary.

Additionally, all coding units with horizontal position in the range of ( XPOS_RB )..( XPOS_RB + w2 – 1 ) (hereinafter called as redundant intra-coded coding unit) are also forced to be coded in intra. The reason of this redundant intra coding is to avoid contamination from the dirty area thought the in-loop filtering process. As illustrated in Figure 6, samples within at most 11 sample positions left from the right edge of the intra coding unit, which is adjacent to the right inter coding unit, are contaminated through the three in-loop filtering process. In this contribution, w2, the width of intra-coded coding unit at the right side of the refresh boundary, is set to 16 if w1 is larger than 24, 8 otherwise.

It is noted that w2 can be 0 if there is a normative restriction that the refresh boundary is treated as a tile boundary in in-loop filtering.

Figure 6: Contamination of sample through in-loop filtering

Restriction on intra prediction mode

Since there is no normative control in intra coding at the refresh boundary, it is controlled so that any intra-coded coding unit does not use any sample in the dirty area.

In addition, any redundant intra-coded coding unit is also controlled not to use any sample in the right side of the coding unit. Those restrictions are illustrated in Figure 7.

Figure 7: Restriction on intra prediction

Restriction on motion vector

The same approach as in JVET-M0197 [2] is used in this contribution. Firstly, the motion vector of an inter-coded coding unit in the clean area is restricted to point to samples inside the clean area of its reference picture. Secondly, the use of motion vector of pictures which are decoded before the last recovery point picture is disabled in the TMVP mode.

One changes relative to the previous version is that DMVR adopted at the previous meeting is disabled in this implementation since there is no way for a decode to know whether derived motion vector is allowed to use.

Cycle of random-access picture

The width of intra-coded coding unit (i.e. w1 in Figure 5) is set explicitly in the first version [2].

In the revised version, two types of derivation schemes are implemented and tested.

Scheme 1

The width w1 is calculated from the “ideal” cycle of random-access picture C1 and the frame rate.

The width w1 is calculated as follows;

w1 = unit_blk * ceil ( ( picture_width + ( C1 * frame_rate / 2) ) / (C1 * frame_rate / unit_blk ),

The value of unit_blk is set equal to 16 in the test. It is noted the width of the stripe of intra-coded coding unit may be less than w1 only at a recovery point picture (i.e. the position of the refresh boundary is on the right boundary of a picture).

Actual cycle of random-access picture C2 is calculated as follows;

C2 = ceil ( picture_width / w1 )

The corresponding parameter of C1 (in unit of 0.1 second) in the configuration file is IntraRefreshPeriod.

This scheme is enable with the macro CYCLE_CONFIG equal to 1.

Scheme 2

It is controlled to set C2 equals to C1.

The first t1 pictures in one refresh cycle (i.e. C1) have the width of the stripe w1 equal to wF and the remaining t2 pictures (i.e. t2 = C1 – t1) have w1 equal to ( wF – unit_blk ).

There is the following constraint;

wF = ceil( picture_width / C1 / unit_blk ) * unit_blk

t1 + t2 = C1

t1 * wF + t2 * ( wF – unit_blk ) = picture_width

The value of unit_blk is set equal to 8 in the test.

The values of C1 and t1 are set by IntraRefreshPeriod and RACycle in the configuration file, respectively.

This scheme is enable with the macro CYCLE_CONFIG equal to 2.

First access unit in bitstream

The first picture (i.e. POC=0) is encoded as an IDR picture. The same tile structure as for non-IDR pictures is applied.

It is noted that there is no control in the software to set the size of the IDR picture in bits similar to non-IDR pictures. As a result, the size of the decoding units in the first IDR picture in bits is beyond the intended value.

It is suggested not to use the encoding result of first IDR picture and non-IDR pictures in the first refresh cycle for BD-Rate calculation.

Suggested test conditions

Test sequences

It is proposed to use the same set of test sequences as for the low delay B condition.

Refresh cycle

It is proposed to set the value of refresh cycle equal to 1 second, similarly as the random access condition.

Actual refresh cycle of each test sequence is summarized in Table 1.

Table 1: Actual refresh cycle

Scheme 1

Scheme 2

Number of decoding units

It is proposed to set the number of decoding units equal to the number of rows of coding tree unit.

Simulation results

The performance of proposed software is compared with the low delay B condition similar as in the previous contribution. The purpose of this test is just for information.

For the ultra-low latency condition, conditions in section 4 are applied.

For the low delay B condition, the interval of IRAP picture is set equally to that for the random access condition.

The simulation result is summarized in Table 2.

Table 2: BD-Rate of proposed ultra-low latency condition against low delay B condition
(C1: IRAP cycle for low-delay B, refresh cycle for scheme 2)
(C2: refresh cycle for scheme 1)

| Class | Sequence | C1 | Scheme 1 |  |  |  | Scheme 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | C2 | Y | Cb | Cr | Y | Cb | Cr |
| B | MarketPlace | 64 | 60 | 79.18% | 79.18% | 79.18% |  |  |  |
|  | RitualDance | 64 | 60 | 22.10% | 30.62% | 33.13% | 22.27% | 30.70% | 33.61% |
|  | Cactus | 48 | 60 | 28.92% | 62.51% | 81.06% | 31.33% | 64.69% | 83.58% |
|  | BasketballDrive | 48 | 60 | 11.68% | 37.58% | 32.46% | 12.42% | 38.67% | 33.29% |
|  | BQTerrace | 64 | 60 | 31.88% | 180.20% | 264.66% | 31.44% | 177.90% | 259.12% |
| C | BasketballDrill | 48 | 52 | 43.21% | 48.53% | 44.90% | 49.79% | 54.62% | 51.27% |
|  | BQMall | 64 | 52 | 32.21% | 60.20% | 57.57% | 31.48% | 59.77% | 56.49% |
|  | PartyScene | 48 | 52 | 47.58% | 83.28% | 90.07% | 60.10% | 95.12% | 102.17% |
|  | RaceHorses | 32 | 26 | 14.04% | 41.72% | 37.35% | 14.50% | 41.70% | 37.28% |
| D | BasketballPass | 64 | 26 | 23.68% | 48.15% | 33.91% | 25.40% | 49.57% | 35.49% |
|  | BQSquare | 48 | 26 | 48.39% | 122.43% | 164.13% | 78.94% | 190.86% | 222.01% |
|  | BlowingBubbles | 48 | 26 | 61.66% | 93.76% | 99.53% | 65.48% | 99.64% | 104.03% |
|  | RaceHorses | 32 | 26 | 23.86% | 42.97% | 43.47% | 24.81% | 50.04% | 51.50% |
| E | FourPeople | 64 | 80 | 52.18% | 94.94% | 97.68% | 68.87% | 110.01% | 115.11% |
|  | Johnny | 64 | 80 | 59.83% | 91.70% | 97.79% | 85.13% | 115.38% | 125.32% |
|  | KristenAndSara | 64 | 80 | 51.75% | 81.80% | 88.03% | 67.49% | 97.33% | 100.58% |
| F | BasketballDrillText | 48 | 60 | 48.18% | 62.36% | 62.05% | 56.60% | 69.91% | 69.45% |
|  | ArenaOfValor | 64 | 52 | 42.81% | 51.64% | 49.91% | 42.82% | 51.60% | 50.02% |
|  | SlideEditing | 32 | 27 | 75.61% | 82.83% | 84.52% | 63.89% | 70.67% | 72.98% |
|  | SlideShow | 16 | 20 | 30.99% | 48.91% | 51.57% | 38.73% | 57.49% | 62.05% |

Parallel processing

Purpose

The performance of parallel processing is investigated, aiming the reduction of the turnaround time of simulation.

Scheme

Each thread encode ( 1 + ( 1 + L ) * C1 ) frames and the result of last ( L * C1 ) frames is used for calculation of BD-Rate. This encoding scheme is illustrated in Figure 8.

Figure 8: Multi-thread parallel processing ( L = 2 )

Turn-around time is reduced from M to ( 1 + L ), where M is the number of cycles in the test sequence.

Performance

The performance of proposed parallel processing is compared with single thread processing.

The simulation result is summarized in Table 3

Table 3: BD-Rate of parallel processing against single thread processing
( Scheme 2, L = 2 )

| Class | Sequence | C1 | Scheme 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Y | Cb | Cr |
| B | MarketPlace | 64 |  |  |  |
|  | RitualDance | 64 |  |  |  |
|  | Cactus | 48 |  |  |  |
|  | BasketballDrive | 48 |  |  |  |
|  | BQTerrace | 64 |  |  |  |
| C | BasketballDrill | 48 | -0.02% | 0.25% | 0.33% |
|  | BQMall | 64 | -0.02% | 0.01% | 0.11% |
|  | PartyScene | 48 | 0.00% | 0.09% | 0.16% |
|  | RaceHorses | 32 | -0.19% | -0.18% | -0.17% |
| D | BasketballPass | 64 | -0.20% | -0.83% | -0.23% |
|  | BQSquare | 48 | 0.07% | -0.28% | -0.59% |
|  | BlowingBubbles | 48 | 0.05% | -0.25% | -0.03% |
|  | RaceHorses | 32 | 0.15% | -0.13% | 0.36% |
| E | FourPeople | 64 |  |  |  |
|  | Johnny | 64 |  |  |  |
|  | KristenAndSara | 64 |  |  |  |
| F | BasketballDrillText | 48 |  |  |  |
|  | ArenaOfValor | 64 |  |  |  |
|  | SlideEditing | 32 |  |  |  |
|  | SlideShow | 16 |  |  |  |

Conclusion

This contribution proposes a revised software, test conditions and parallel processing for ultra low-latency encoding. It is requested to decide the common test condition for ultra low-latency encoding, taking this contribution into consideration.

References

[1] “Versatile Video Coding (Draft 4)” JVET-M1001, January 2019

[2] K.Kazui, “AHG14: Software for ultra low-latency encoding”, JVET-M0197, January 2019

Patent rights declaration(s)

Fujitsu Limited may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).