| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0096-v1 |
| --- | --- |

| Title: | CE8-related: A fixed updating order for IBC reference memory |  |  |
| --- | --- | --- | --- |
| Status: | Input Documents to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chen-Yen Lai, Tzu-Der Chuang, Ching-Yeh Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {jenny.lai, peter.chuang, chingyeh.chen, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, a fixed updating order for intra block copy (IBC) reference memory is proposed to replace the two updating orders depending on the coding block (CB) order. Currently in VTM4.0, the IBC reference memory used to store reconstructed samples in the left 128×128-luma/64×64×2-chroma coding tree unit (CTU) is updated according to the CB order within the current CTU. The updating order can be replacing “top-left => top-right => bottom-left => bottom-right” or “top-left => bottom-left => top-right => bottom-right” 64×64-luma/32×32×2-chroma units of the left CTU. In this proposal, it is proposed to always apply only one updating order as replacing “bottom-left => top-left => top-right => bottom-right” 64×64-luma/32×32×2-chroma units of the left CTU. In addition, it is proposed to add an encoder-only normative constraint in the VVC specification to ensure the maximum CB size for IBC is “64×64 for luma and 32×32 for chroma” or smaller. It is reported that IBC with the proposed method achieves XXX%, XXX%, and XXX% luma BD-rates in AI condition for common test condition (CTC), Class F, and TGM sequences, respectively, compared to VTM4.0 IBC. It is asserted that the process of updating IBC reference memory is simplified while achieving coding gains.

# Introduction

In VTM4.0 [1], an IBC CB can be predicted by reconstructed samples in the left CTU while the size of the IBC reference memory is kept as low as 128×128-luma/64×64×2-chroma. The reconstructed samples of the left CTU are stored in the IBC reference memory when the left CTU is reconstructed. Later, in the IBC reference memory, the reconstructed samples of the left CTU are replaced with reconstructed samples of the current CTU according to the CB order within the current CTU. The reconstructed samples available in the IBC reference memory can be used to predict the current CB. Four cases are described as follows, assuming the CTU size is 128×128-luma/64×64×2-chroma.

- In case that the current CB belongs to the top-left 64×64-luma/32×32×2-chroma unit of the current CTU, in addition to the reconstructed samples of the current CTU, the current CB can refer to the reconstructed samples in the top-right, bottom-right, and bottom-left 64×64-luma/32×32×2-chroma units of the left CTU.

- In case that the current CB belongs to the top-right 64×64-luma/32×32×2-chroma unit of the current CTU, in addition to the reconstructed samples of the current CTU, if the bottom-left 64×64-luma/32×32×2-chroma unit of the current CTU has not yet been reconstructed, the current CB can refer to the reconstructed samples in the bottom-left and bottom-right 64×64-luma/32×32×2-chroma units of the left CTU; otherwise, the current CB can refer to the reconstructed samples in the bottom-right 64×64-luma/32×32×2-chroma unit of the left CTU.

- In case that the current CB belongs to the bottom-left 64×64-luma/32×32×2-chroma unit of the current CTU, in addition to the reconstructed samples of the current CTU, if the top-right 64×64-luma/32×32×2-chroma unit of the current CTU has not yet been reconstructed, the current CB can refer to the reconstructed samples in the top-right and bottom-right 64×64-luma/32×32×2-chroma units of the left CTU; otherwise, the current CB can refer to the reconstructed samples in the bottom-right 64×64-luma/32×32×2-chroma unit of the left CTU.

- In case that the current CB belongs to the bottom-right 64×64-luma/32×32×2-chroma unit of the current CTU, the current CB can only refer to the reconstructed samples of the current CTU.

As described in the above cases, the updating orders to replace 64×64-luma/32×32×2-chroma units of the left CTU are different.

# Proposed method

Only one updating order is applied. Before the current CTU is decoded, all the reconstructed samples of the left CTU are stored in the IBC reference memory. Next, the four 64×64-luma/32×32×2-chroma units of the left CTU, following the order of “bottom-left => top-left => top-right => bottom-right” 64×64-luma/32×32×2-chroma units of the left CTU, are replaced with the 64×64-luma/32×32×2-chroma units of the current CTU regardless of the CB order within the current CTU.

Figure 1 shows an example of processing 64×64-luma/32×32×2-chroma units of the current CTU in the order of top-left => top-right => bottom-left => bottom-right. The 64×64-luma/32×32×2-chroma units of the left CTU, following the order of bottom-left => top-left => top-right => bottom-right, are replaced with the 64×64-luma/32×32×2-chroma units of the current CTU.

X

Curr

X

X

Curr

X

X

X

X

Curr

X

X

X

Curr

Figure 1.  Processing order within the current CTU: top-left => top-right => bottom-left => bottom-right; order to discard units within the left CTU: => bottom-left => top-left => top-right => bottom-right

Figure 2 shows another example of processing 64×64-luma/32×32×2-chroma units of the current CTU in the order of top-left => bottom-left => top-right => bottom-right. The 64×64-luma/32×32×2-chroma units of the left CTU, following the order of bottom-left => top-left => top-right => bottom-right, are replaced with the 64×64-luma/32×32×2-chroma units of the current CTU.

X

Curr

X

X

Curr

X

X

X

X

Curr

X

X

X

Curr

Figure 2.  Processing order within the current CTU: top-left => bottom-left => top-right => bottom-right; order to discard units within the left CTU: => bottom-left => top-left => top-right => bottom-right

# Experimental results

The proposed IBC has been implemented on VTM4.0. The simulations were performed following the JVET common test conditions (CTC) [2]. Table 1 shows the simulation results of the proposed IBC compared to the VTM4.0 IBC.

Table 1.  Simulation results of the proposed IBC compared to the VTM4.0 IBC (to be updated)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 (IBC enabled) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.02% | -0.01% | 103% | 103% |
| Class A2 | 0.12% | 1.20% | 1.00% | 15% | 51% |
| Class B | 0.00% | -0.01% | 0.00% | 102% | 99% |
| Class C | 0.00% | 0.00% | -0.01% | 102% | 97% |
| Class E | -0.01% | -0.07% | -0.05% | 102% | 101% |
| Overall | 0.02% | 0.18% | 0.15% | 74% | 89% |
| Class D | 0.00% | 0.01% | 0.00% | 102% | 99% |
| Class F (optional) | -0.64% | -0.58% | -0.65% | 106% | 101% |
| TGM | -1.63% | -1.66% | -1.68% | 105% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-4.0 (IBC enabled) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | 0% | 1% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | 0% | 1% |
| Class B | #VALUE! | #VALUE! | #VALUE! | 0% | 2% |
| Class C | #VALUE! | #VALUE! | #VALUE! | 0% | 12% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | 0% | 3% |
| Class D | #VALUE! | #VALUE! | #VALUE! | 0% | 44% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | 0% | 10% |
| TGM | #VALUE! | #VALUE! | #VALUE! | 0% | 4% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 (IBC enabled) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | 0% | 5% |
| Class C | 0.02% | 0.17% | 0.10% | 100% | 101% |
| Class E | -0.03% | 0.42% | 0.44% | 100% | 102% |
| Overall | #VALUE! | #VALUE! | #VALUE! | 2% | 28% |
| Class D | -0.01% | -0.05% | 0.02% | 100% | 100% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | 11% | 50% |
| TGM | #VALUE! | #VALUE! | #VALUE! | 11% | 52% |

# Conclusions

This contribution proposed a method of IBC with a fixed updating order for IBC reference memory, as a simplification to the VTM4.0 IBC. The proposed IBC reportedly achieves XXX%, XXX%, and XXX% luma BD-rates for AI, RA, and LB, respectively, compared to VTM4.0 IBC.

# References

- VTM4.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-4.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-M1010.

# Patent rights declaration(s)

MediaTek Inc.  may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).