| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0099 |
| --- | --- |

| Title: | AHG12/AHG17: On Sub-picture parameter set |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong He, Ahmad Hamza 9276 Scranton Road Suite 300 San Diego, CA 92121, USA | Tel: Email: | 1-858-210-4807 Yong.He@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

The contribution proposes a new parameter set, sub-picture parameter set (SPPS), to meet the agreed design goal of “extracting VCL NAL units of a subset of MCTSs from one VVC bitstream and reposition them to another VVC bitstream without substantial difficulty (e.g. without VLC-level modifications)”.

A sub-picture may refer to its own sub-picture parameter set (SPPS). A sub-picture parameter set comprises sub-picture level syntax elements, and a PPS syntax is simplified to comprise picture level syntax elements for the composite output picture.

# Introduction

360-degree video viewport-dependent streaming requires the capability to extract independent decodable sub-pictures or MCTSs from multiple input video bitstreams and reposition them to an output bitstream. Each sub-picture may be coded at different resolutions, coding configuration (e.g. IDR interval) and quality. Current parameter set design and the activation process may be optimized to streamline the sub-picture extraction and reposition process.

JVET-M0261 option 3 proposed each sub-picture to refer to its own PPS and sub-pictures are treated like pictures in the decoding process. It simplifies the extraction and reposition process as the activated PPS can be extracted and repositioned along with the associated sub-picture VCL NAL unit straightforwardly. While the PPS activation for both composite picture and its sub-pictures is not elaborated. This contribution keeps PPS to be activated for the composite picture and proposes a new sub-picture parameter set to be activated for each sub-picture. Each sub-picture may refer to its own SPPS and a SPPS may refer to a PPS.

Figure 1 shows an example of SPPS activated for each sub-picture before and after extraction and reposition. Let’s assume a picture can be encoded into a high-resolution picture with 4 sub-pictures (#0, #1, #2 and #3) and a low-resolution with 4 sub-pictures (#a, #b, #c and #d). Depending on the viewport, high-resolution sub-pictures and low-resolution sub-pictures may be extracted and combined into an output picture. Each sub-picture within a picture may refer to its own SPPS (e.g. SPPS #0, #1, #2, #3, #A or #B), and the SPPS referred by the sub-pictures within a composite picture shall refer to the same PPS (e.g. PPS #0 or #1) which is activated for the composite picture. Each extracted sub-picture and the associated SPPS can be passed through to the output bitstream. A new PPS (e.g. PPS #2 or PPS #3) is activated for the output picture. The PPS reference id at SPPS can be updated to refer to new PPS, and none of the syntax element in tile group header needs to be modified.

Figure 1 PPS and SPPS reference example for sub-pictures

Figure 2 shows the general SPS, PPS and SPPS activation example, a sub-picture or tile group may refer to a SPPS, and a SPPS may refer to a PPS and a PPS may refer to a SPS.

Figure 2 parameter set activation example

# Sub-picture Parameter Set

Sub-picture parameter set inherits most parameters from PPS except tile group configuration since “it is required that each MCTS/sub-picture contains only one tile group” as agreed in 13th JVET meeting. Table 1 shows the syntax struct of proposed SPPS. Those syntax elements not from PPS are highlighted in turquoise . The tile partitioning syntax elements are shows in Table 2.

Table 1 Proposed sub-picture parameter set syntax (SPPS)

| subpic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| spps_subpic_parameter_set_id | ue(v) |
| spps_pic_parameter_set_id | ue(v) |
| if( rect_tile_group_flag && ! single_tile_group_in_pic_flag) { |  |
| subpic_width_in_luma_samples | ue(v) |
| subpic_height_in_luma_samples | ue(v) |
| tile_partitioning_struct() |  |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| cabac_init_present_flag | u(1) |
| for( i = 0; i < 2; i++ ) |  |
| num_ref_idx_default_active_minus1[ i ] | ue(v) |
| rpl1_idx_present_flag | u(1) |
| init_qp_minus26 | se(v) |
| transform_skip_enabled_flag | u(1) |
| if( transform_skip_enabled_flag ) |  |
| log2_transform_skip_max_size_minus2 | ue(v) |
| cu_qp_delta_enabled_flag | u(1) |
| if( cu_qp_delta_enabled_flag ) |  |
| cu_qp_delta_subdiv | ue(v) |
| spps_cb_qp_offset | se(v) |
| spps_cr_qp_offset | se(v) |
| spps_tile_group_chroma_qp_offsets_present_flag | u(1) |
| weighted_pred_flag | u(1) |
| weighted_bipred_flag | u(1) |
| deblocking_filter_control_present_flag | u(1) |
| if( deblocking_filter_control_present_flag ) { |  |
| deblocking_filter_override_enabled_flag | u(1) |
| spps_deblocking_filter_disabled_flag | u(1) |
| if( !pps_deblocking_filter_disabled_flag ) { |  |
| spps_beta_offset_div2 | se(v) |
| spps_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

Table 2 Tile partitioning struct syntax

| tile_partitioning_struct( ) { |  |
| --- | --- |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_tile_spacing_flag | u(1) |
| if( !uniform_tile_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| } |  |
| } |  |

spps_subpic_parameter_set_id identifies the SPPS for reference by other syntax elements. The value of spps_subpic_parameter_set_id shall be in the range of 0 to 63, inclusive.

spps_pic_parameter_set_id specifies the value of pps_pic_parameter_set_id for the active PPS. The value of spps_picq_parameter_set_id shall be in the range of 0 to 63, inclusive.

subpic_width_in_luma_samples specifies the width of each decoded sub-picture in units of luma samples. subpic_width_in_luma_samples shall not be equal to 0 and shall be an integer multiple of MinCbSizeY. When not present, the value of subpic_width_in_luma_samples is inferred to be equal to the value of pic_width_in_luma_samples in associated PPS.

subpic_height_in_luma_samples specifies the height of each decoded sub-picture in units of luma samples. subpic_height_in_luma_samples shall not be equal to 0 and shall be an integer multiple of MinCbSizeY. When not present, the value of subpic_height_in_luma_samples is inferred to be equal to the value of pic_height_in_luma_samples in associated PPS.

# Picture Parameter Set

The PPS is simplified to comprise only the parameters relevant to the output picture such as resolution and tile group partitioning. Table 3 is the syntax struct of PPS.

Table 3 Proposed PPS syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag) { |  |
| single_tile_per_tile_group_flag | u(1) |
| if( !single_tile_per_tile_group_flag && !single_tile_per_tile_group_flag) |  |
| rect_tile_group_flag | u(1) |
| if( !rect_tile_group_flag && !single_tile_per_tile_group_flag) |  |
| tile_partitioning_struct() |  |
| if( rect_tile_group_flag && !single_tile_per_tile_group_flag ) { |  |
| num_tile_groups_in_pic_minus1 | ue(v) |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) { |  |
| if( i > 0 ) |  |
| top_left_ctu_idx[ i ] | u(v) |
| bottom_right_ctu_idx[ i ] | u(v) |
| } |  |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| if( loop_filter_across_tiles_enabled_flag ) |  |
| loop_filter_across_tile_groups_enabled_flag | u(1) |
| } |  |
| if ( rect_tile_group_flag ) { |  |
| signalled_tile_group_id_flag | u(1) |
| if( signalled_tile_group_id_flag ) { |  |
| signalled_tile_group_id_length_minus1 | ue(v) |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) |  |
| tile_group_id[ i ] | u(v) |
| } |  |
| } |  |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

pic_width_in_luma_samples specifies the width of each decoded picture in units of luma samples. pic_width_in_luma_samples shall not be equal to 0 and shall be an integer multiple of MinCbSizeY.

pic_height_in_luma_samples specifies the height of each decoded picture in units of luma samples. pic_height_in_luma_samples shall not be equal to 0 and shall be an integer multiple of MinCbSizeY.

single_tile_group_in_pic_flag equal to 1 specifies that there is only one tile group in each picture referring to the PPS. single_tile_in_pic_flag equal to 0 specifies that there is more than one tile group in each picture referring to the PPS.

pic_width_in_luma_samples and pic_height_in_luma_samples are added into PPS to indicate the size of output picture. This allows dynamic output picture resolution changes due to the combination of sub-pictures in different resolutions. Figure 3 shows several output picture resolutions with multiple sub-pictures for viewport dependent video delivery to match the viewport(s). When a viewport is crossing two sub-pictures, both sub-pictures covering the viewport may be delivered in high-resolution. All high-resolution or low-resolution sub-pictures may also be delivered when there is sufficient bandwidth or lack of bandwidth.

Figure 3 Example of output composite pictures with multiple input sub-pictures

# Sequence Parameter Set

The signaling of picture resolution in the SPS is the same as JVET-M0135 proposed (Table 4).

Table 4 SPS syntax (proposed in JVET-M0135)

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| num_pic_size_in_luma_samples_minus1 | ue(v) |
| for( i = 0; i <= num_pic_size_in_luma_samples_minus1; i++ ) { |  |
| pic_width_in_luma_samples[ i ] | ue(v) |
| pic_height_in_luma_samples[ i ] | ue(v) |
| } |  |
| ... |  |
| } |  |

# Tile group header

A tile group refers both PPS and SPPS in its tile group header.

Table 5 Tile group header syntax

| tile_group_header( ) { | Descriptor |
| --- | --- |
| tile_group_subpic_parameter_set_id | ue(v) |
| if( rect_tile_group_flag ¦ ¦ NumTilesInPic > 1 ) |  |
| tile_group_address | u(v) |
| …. |  |

tile_group_subpic_parameter_set_id specifies the value of spps_pic_parameter_set_id for the SPPS in use. The value of tile_group_subpic_parameter_set_id shall be in the range of 0 to 63, inclusive.

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).