| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0103-v2 |
| --- | --- |

| Title: | CE3/7-related: Coefficient group size harmonisation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Rosewarne, J. Gan Building A, 5 Talavera Rd, Macquarie Park, 2113, NSW, Australia | Tel: Email: | +61 2 9805 2844 chris.rosewarne@cisra.canon.com.au jonathan.gan@cisra.canon.com.au |
| Source: | Canon Information Systems Research Australia Pty Ltd. |  |  |

_____________________________

# Abstract

Prior to VTM-4.0 two coefficient group sizes were present in the design, 2x2 and 4x4 for chroma and always 4x4 for luma. VTM-4.0 includes intra sub partitions, which introduces additional coefficient group sizes in luma, 1x16, 2x8, 8x2, and 16x1. The selection of coefficient group sizes becomes dependent upon the TB size and the channel type (luma or chroma). This contribution proposes to set the coefficient group size based upon TB size only, i.e., remove the dependency on channel type. As a consequence, the above-mentioned 2x8 and 8x2 sizes become available in chroma. Moreover, sizes of 2x4 and 4x2 also become available for TBs of these sizes. The resulting BD-rate impact for AI is 0.00%, -0.04%, -0.05%, for RA is -0.09%, 0.05%, -0.01%, and for LDB is 0.00%, 0.14%, -0.12% in Y, Cb, and Cr channels, respectively.

# Introduction

VTM-4.0 uses 2x2 coefficient group sizes for chroma TBs and various coefficient group sizes for luma, due to the introduction of intra sub partitions. These are summarised by the following excerpt from the file Rom.cpp, showing the relationship between log2 TB width and height to log2 coefficient group width and height:

uint32_t g_log2SbbSize[2][MAX_CU_DEPTH+1][MAX_CU_DEPTH+1][2] =

{

//===== luma =====

{

{ {0,0}, {0,1}, {0,2}, {0,3}, {0,4}, {0,4}, {0,4}, {0,4} },

{ {1,0}, {1,1}, {1,2}, {1,3}, {1,3}, {1,3}, {1,3}, {1,3} },

{ {2,0}, {2,1}, {2,2}, {2,2}, {2,2}, {2,2}, {2,2}, {2,2} },

{ {3,0}, {3,1}, {2,2}, {2,2}, {2,2}, {2,2}, {2,2}, {2,2} },

{ {4,0}, {3,1}, {2,2}, {2,2}, {2,2}, {2,2}, {2,2}, {2,2} },

{ {4,0}, {3,1}, {2,2}, {2,2}, {2,2}, {2,2}, {2,2}, {2,2} },

{ {4,0}, {3,1}, {2,2}, {2,2}, {2,2}, {2,2}, {2,2}, {2,2} },

{ {4,0}, {3,1}, {2,2}, {2,2}, {2,2}, {2,2}, {2,2}, {2,2} }

},

//===== chroma =====

{

{ {0,0}, {0,0}, {0,0}, {0,0}, {0,0}, {0,0}, {0,0}, {0,0} },

{ {0,0}, {1,1}, {1,1}, {1,1}, {1,1}, {1,1}, {1,1}, {1,1} },

{ {0,0}, {1,1}, {2,2}, {2,2}, {2,2}, {2,2}, {2,2}, {2,2} },

{ {0,0}, {1,1}, {2,2}, {2,2}, {2,2}, {2,2}, {2,2}, {2,2} },

{ {0,0}, {1,1}, {2,2}, {2,2}, {2,2}, {2,2}, {2,2}, {2,2} },

{ {0,0}, {1,1}, {2,2}, {2,2}, {2,2}, {2,2}, {2,2}, {2,2} },

{ {0,0}, {1,1}, {2,2}, {2,2}, {2,2}, {2,2}, {2,2}, {2,2} },

{ {0,0}, {1,1}, {2,2}, {2,2}, {2,2}, {2,2}, {2,2}, {2,2} }

},

};

# Description of the proposed method

This contribution proposes to remove the dependency on channel type in selecting coefficient group size for a given transform block, thus “harmonising” coefficient group size selection between channel types.

The following code snippet shows the proposed mapping from log2 TB widthand height to log2 coefficient group width and height:

uint32_t g_log2SbbSize[MAX_CU_DEPTH + 1][MAX_CU_DEPTH + 1][2] =

//===== luma/chroma =====

{

{ { 0,0 },{ 0,1 },{ 0,2 },{ 0,3 },{ 0,4 },{ 0,4 },{ 0,4 },{ 0,4 } },

{ { 1,0 },{ 1,1 },{ 1,2 },{ 1,3 },{ 1,3 },{ 1,3 },{ 1,3 },{ 1,3 } },

{ { 2,0 },{ 2,1 },{ 2,2 },{ 2,2 },{ 2,2 },{ 2,2 },{ 2,2 },{ 2,2 } },

{ { 3,0 },{ 3,1 },{ 2,2 },{ 2,2 },{ 2,2 },{ 2,2 },{ 2,2 },{ 2,2 } },

{ { 4,0 },{ 3,1 },{ 2,2 },{ 2,2 },{ 2,2 },{ 2,2 },{ 2,2 },{ 2,2 } },

{ { 4,0 },{ 3,1 },{ 2,2 },{ 2,2 },{ 2,2 },{ 2,2 },{ 2,2 },{ 2,2 } },

{ { 4,0 },{ 3,1 },{ 2,2 },{ 2,2 },{ 2,2 },{ 2,2 },{ 2,2 },{ 2,2 } },

{ { 4,0 },{ 3,1 },{ 2,2 },{ 2,2 },{ 2,2 },{ 2,2 },{ 2,2 },{ 2,2 } }

};

# Experimental results

Simulation results under the JVET common test conditions [1] are presented in Table 1.

Table 1. Coefficient group size harmonization results.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.03% | 0.00% | 97% | 97% |
| Class A2 | -0.02% | 0.05% | -0.04% | 95% | 91% |
| Class B | 0.00% | -0.02% | -0.03% | 95% | 93% |
| Class C | 0.01% | -0.16% | -0.03% | 97% | 94% |
| Class E | 0.00% | -0.01% | -0.14% | 97% | 95% |
| Overall | 0.00% | -0.04% | -0.05% | 96% | 94% |
| Class D | 0.01% | -0.06% | -0.23% | 97% | 91% |
| Class F | 0.01% | -0.07% | -0.13% | 97% | 94% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.07% | -0.03% | 89% | 93% |
| Class A2 | -0.03% | -0.12% | -0.03% | 92% | 97% |
| Class B | 0.00% | 0.00% | -0.17% | 94% | 100% |
| Class C | -0.31% | 0.34% | 0.24% | 90% | 67% |
| Class E |  |  |  |  |  |
| Overall | -0.09% | 0.05% | -0.01% | 92% | 88% |
| Class D | -0.02% | -0.16% | -0.20% | 86% | 69% |
| Class F | -0.01% | -0.20% | -0.18% | 91% | 63% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.20% | -0.34% | 90% | 94% |
| Class C | -0.05% | 0.32% | 0.00% | 93% | 100% |
| Class E | 0.06% | 0.46% | 0.08% | 101% | 110% |
| Overall | 0.00% | 0.14% | -0.12% | 94% | 100% |
| Class D | -0.01% | -0.06% | -0.05% | 97% | 103% |
| Class F | -0.10% | -0.74% | -0.32% | 97% | 105% |

# Conclusion

This input document proposes harmonising the coefficient group sizes used between luma and chroma channels for VTM-4.0.

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-M1010, Marrakech, MA, January, 2019.

# Patent rights declaration(s)

Canon Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).