| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0122-v2 |
| --- | --- |

| Title: | CE6/CE8-related: Alternative Implementation of residual rearrangement for transform skipped blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Tsukuba Masaru Ikeda Yoichi Yagasaki Teruhiko Suzuki 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | +81-50-3750-2779 Takeshi.Tsukuba@sony.com Masaru.Ikeda@sony.com Yoichi.Yagasaki@sony.com Teruhiko.S@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

This contribution proposes an alternative implementation of transform skip and residual rearrangement CE8-4.1b using MTS framework: 1D transform skip by Identity Transform and 1D residual reorder. It is asserted that the proposal can unify MTS and residual rearrangement with on average x.xx% and y.yy% of bdrate changes over VTM-4.0 CTC in classA1/A2/B/C/E for AI and RA, respectively; especially -0. 29 % and -0.17% of bdrate changes in Class D for AI and RA, respectively; -0.82% and -0.67% of bdrate changes in ClassF for AI and RA, respectively; -1.02% and -0.76% of bdrate changes in SCC sequences, respectively

Runtime is not reliable due to heterogenous grids for simulation.

It is recommended to study unification between MTS and residual rearrangement in CE or AHG.

In v2, simulation results are updated.

Introduction

Residual rearrangement for 2D transform skipped blocks was proposed in JVET-M0278[1]/JVET-N0428 [2]. CE8-4.1b in [2] applies residual rotation to intra transform-skipped block as shown in Figure 1-1.

Figure 1-1: Residual rotation [2]

Matrix form of 2D transform skip and residual rotation is defined using Identity matrix I and cross-identity matrix J as below:

Rotation: X= JM∙IM∙x∙INt∙JNt,    -(A1)

where x is a MxN input vector, X is MxN output vector, IN is a NxN identity matrix and JN is a NxN cross-identity matrix.

Transform using Identity matrix, called Identity Transform (IDT), has been proposed in JVET-C0022[3], JVET-M0501[4] and JVET-N0360[5]. In addition, JVET-M0501/JVET-N0360 proposes unification between transform skip and MTS by using Identity Transform.

Proposal

Matrix forms described in equation (A1) is rewritten using I∙A=A∙I:

Rotation:　X=JM∙IM∙x∙INt∙JNt=IM∙(JM∙(x∙JNt)∙INt),    -(A1')

Therefore, forward process of 2D transform skip and residual rearrangement can be done in transform process.

Figure 2-1 shows the block diagrams of dequantization, inverse transform and residual rearrangement for [1][2] and proposed approach, respectively. Note residual rearrangement is called “reorder” in Figure 2-1. As can be seen, transform skip and residual rearrangement are unified with MTS.

Following modification are necessary for proposal:

- Identity Transform, which is implemented as a scaling of the input coefficients

- 1D residual reorder

Proposed text changes are described in section 6.

Figure 2-1: Block diagram on JVET-M0278/JVET-N0428 [1][2] and proposed approach.

Simulation Results

Proposed method is integrated on top of CE6-1.1 software [6] and following modification applies to CE6-1.1:

- Disabled replacing DCT8 with IDT, i.e. only support 2D transform skip using IDT

- Enabled residual rotation for intra transform-skipped blocks in CE8-4.1b using IDT and 1D reorder in transform process.

Following tests are conducted under CTC [7] with additional SCC sequences described in CE6 [8].

Test#1: Proposal over VTM-4.0

Table 3-1shows the summary of coding performances for Test#1.

Detail results are available in the attached spreadsheets.

Table 3-1 Results of Test#1

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0_IBC0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.22% | -0.20% | -0.18% | 88% | 94% |
| Class E | -0.12% | -0.08% | -0.19% | 81% | 97% |
| Overall |  |  |  |  |  |
| Class D | -0.29% | -0.26% | -0.37% | 85% | 89% |
| Class F | -0.82% | -0.49% | -0.69% | 95% | 98% |
| Class SCC | -1.02% | -0.59% | -0.60% | 84% | 93% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-4.0_IBC0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.12% | -0.20% | -0.26% | 96% | 97% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.17% | -0.32% | -0.21% | 98% | 99% |
| Class F | -0.67% | -0.58% | -0.64% | 96% | 98% |
| Class SCC | -0.76% | -0.46% | -0.55% | 92% | 95% |

Conclusion

This contribution proposes an alternative implementation of transform skip and residual rearrangement CE8-4.1b using MTS framework: 1D transform skip by Identity Transform and 1D residual reorder.

It is recommended to study unification between MTS and residual rearrangement in CE or AHG.

References

[1] S. Yoo, J. Choi, J. Heo, J. Choi, L. Li, J. Lim, S. Kim , “Non-CE7 : Residual rearrangement for transform skipped blocks,” JVET-M0278, Jan. 2019

[2] S. Yoo, J. Choi, J. Heo, J. Choi, L. Li, J. Lim, S. Kim , “CE8-4.1 : Rearrangement of the residual block for transform skip,” JVET-N0428, Mar. 2019

[3] P. Philippe, V. Lorcy, “Proposed improvements to the Adaptive multiple Core transform,” JVET-C0022, May 2016

[4] X. Zhao, X. Li, S. Liu, “CE6 related: Unification of Transform Skip mode and MTS, ” JVET-M0501, Jan. 2019

[5] X. Zhao, X. Li, S. Liu, “CE6-1: Unification between MTS and Transform Skip mode, ” JVET-N0360, Mar. 2019

[6] CE6-1.1 Software, https://vcgit.hhi.fraunhofer.de/jvet-m-ce6/VVCSoftware_VTM/tree/CE6-1.1

[7] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, Jan. 2019

[8] X. Zhao, H. E. Egilmez, “Description of Core Experiment 6 (CE6): Transforms and transform signalling, ” JVET-M1026, Jan. 2019

Appendix: Proposed Changes in WD

- 8.7.2 Scaling and transformation process

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable cIdx specifying the colour component of the current block,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height.

Output of this process is the (nTbW)x(nTbH) array of residual samples resSamples[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1.

The variables bitDepth, bdShift and tsShift are derived as follows:

bitDepth = ( cIdx  = =  0 ) ? BitDepthY : BitDepthC		(8-1013)

bdShift = Max( 20 − bitDepth, 0 )		(8-1014)

tsShift = 5 + ( ( Log2( nTbW ) + Log2( nTbH ) ) / 2 )		(8-1015)

The (nTbW)x(nTbH) array of residual samples resSamples is derived as follows:

  - The scaling process for transform coefficients as specified in clause 8.7.3 is invoked with the transform block location ( xTbY, yTbY ), the transform block width nTbW and the transform block height nTbH, the colour component variable cIdx and the bit depth of the current colour component bitDepth as inputs, and the output is an (nTbW)x(nTbH) array of scaled transform coefficients d.

  - The (nTbW)x(nTbH) array of residual samples r is derived as follows:

- If transform_skip_flag[ xTbY ][ yTbY ] is equal to 1 and cIdx is equal to 0, the residual sample array values r[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

r[ x ][ y ] = d[ x ][ y ]  <<  tsShift	(8-1016)

- Otherwise (transform_skip_flag[ xTbY ][ yTbY ] is equal to 0 or and cIdx is not equal to 0), the transformation process for scaled transform coefficients as specified in clause 8.7.4.1 is invoked with the transform block location ( xTbY, yTbY ), the transform block width nTbW and the transform block height nTbH, the colour component variable cIdx and the (nTbW)x(nTbH) array of scaled transform coefficients d as inputs, and the output is an (nTbW)x(nTbH) array of residual samples r.

  - The residual samples resSamples[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

resSamples[ x ][ y ] = ( r[ x ][ y ] + ( 1  <<  ( bdShift − 1 ) ) )  >>  bdShift	(8-1017)

- 8.7.3 Scaling process for transform coefficients

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height,

- a variable cIdx specifying the colour component of the current block,

- a variable bitDepth specifying the bit depth of the current colour component.

Output of this process is the (nTbW)x(nTbH) array d of scaled transform coefficients with elements d[ x ][ y ].

The quantization parameter qP is derived as follows:

- If cIdx is equal to 0, the following applies:

qP = Qp′Y		(8-1018)

- Otherwise, if cIdx is equal to 1, the following applies:

qP = Qp′Cb		(8-1019)

- Otherwise (cIdx is equal to 2), the following applies:

qP = Qp′Cr		(8-1020)

The variable rectNonTsFlag is derived as follows:

rectNonTsFlag = ( ( ( Log2( nTbW ) + Log2( nTbH ) ) & 1 )  = =  1 && 	(8-1021)
		transform_skip_flag[ xTbY ][ yTbY ] = = 0 )

The variables  bdShift, rectNorm and bdOffset are derived as follows:

bdShift = bitDepth + ( ( rectNonTsFlag  ?  8  :  0 ) + 		(8-1022)
	( Log2( nTbW ) + Log2( nTbH ) ) / 2 ) − 5 + dep_quant_enabled_flag

rectNorm = rectNonTsFlag  ?  181  :  1	(8-1023)

bdOffset = ( 1 << bdShift ) >> 1		(8-1024)

The list levelScale[ ] is specified as levelScale[ k ] = { 40, 45, 51, 57, 64, 72 } with k = 0..5.

For the derivation of the scaled transform coefficients d[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1, the following applies:

- The intermediate scaling factor m[ x ][ y ] is set equal to 16.

- The scaling factor ls[ x ][ y ] is derived as follows:

–	If dep_quant_enabled_flag is equal to 1, the following applies:

ls[ x ][ y ] = ( m[ x ][ y ] * levelScale[ (qP + 1) % 6 ] ) << ( (qP + 1) / 6 )	(8-1025)

–	Otherwise (dep_quant_enabled_flag is equal to 0), the following applies:

ls[ x ][ y ] = ( m[ x ][ y ] * levelScale[ qP % 6 ] ) << ( qP / 6 )	(8-1026)

- The value dnc[ x ][ y ] is derived as follows:

dnc[ x ][ y ] = 			(8-1027)
	( TransCoeffLevel[ xTbY ][ yTbY ][ cIdx ][ x ][ y ] * ls[ x ][ y ] * rectNorm +bdOffset )  >>  bdShift

- The scaled transform coefficient d[ x ][ y ] is derived as follows:

d[ x ][ y ] = Clip3( CoeffMin, CoeffMax, dnc[ x ][ y ] )	(8-1028)

- 8.7.4 Transformation process for scaled transform coefficients

- 8.7.4.1 General

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable nTbW specifying the width of the current transform block,

- a variable nTbH specifying the height of the current transform block,

- a variable cIdx specifying the colour component of the current block,

- an (nTbW)x(nTbH) array d[ x ][ y ] of scaled transform coefficients with x = 0..nTbW − 1, y = 0..nTbH − 1.

Output of this process is the (nTbW)x(nTbH) array r[ x ][ y ] of residual samples with x = 0..nTbW − 1, y = 0..nTbH − 1.

The variable implicitMtsEnabled is derived as follows:

- If sps_mts_enabled_flag is equal to 1 and one of the following conditions is true, implicitMtsEnabled is set equal to 1:

- IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT

- cu_sbt_flag is equal to 1 and Max( nTbW, nTbH ) is less than or equal to 32

- sps_explicit_mts_intra_enabled_flag and sps_explicit_mts_inter_enabled_flag are both equal to 0 and CuPredMode[ xTbY ][ yTbY ] is equal to MODE_INTRA

- Otherwise, implicitMtsEnabled is set equal to 0.

The variable trTypeHor specifying the horizontal transform kernel and the variable trTypeVer specifying the vertical transform kernel are derived as follows:

- If cIdx is greater than 0, trTypeHor and trTypeVer are set equal to 0.

- Otherwise, if transform_skip_flag is equal to 0, trTypeHor and trTypeVer are set equal to “IDT”.

- Otherwise, if implicitMtsEnabled is equal to 1, the following applies:

- If IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT, trTypeHor and trTypeVer are specified in Table 8-15 depending on intraPredMode.

- Otherwise, if cu_sbt_flag is equal to 1, trTypeHor and trTypeVer are specified in Table 8-14 depending on cu_sbt_horizontal_flag and cu_sbt_pos_flag.

- Otherwise (sps_explicit_mts_intra_enabled_flag and sps_explicit_mts_inter_enabled_flag are equal to 0), trTypeHor and trTypeVer are derived as follows:

trTypeHor = ( nTbW >= 4  &&  nTbW <= 16  &&  nTbW <= nTbH ) ? 1 : 0	(8-1029)

trTypeVer = ( nTbH >= 4  &&  nTbH <= 16  &&  nTbH <= nTbW ) ? 1 : 0	(8-1030)

- Otherwise, trTypeHor and trTypeVer are specified in Table 8-13 depending on tu_mts_idx[ xTbY ][ yTbY ].

// bFlipX derivation for CE8-4.1b

If trTypeX (X=Hor, Ver) is equal to “IDT” and CuPredMode[ xTbY ][ yTbY ] is equal to MODE_INTRA, a variable bFlipX (X=Hor, Ver) specifying the direction of output transformed samples is derived as follows:

bFlipX = 1													(8-X1)

Othwerise, a variable bFlipX(X=Hor, Ver) is derived as follows:

bFlipX = 0													(8-X2)

The variables nonZeroW and nonZeroH are derived as follows:

nonZeroW = Min( nTbW, (trTypeHor==”IDT” ? ( 32: ( trTypeHor > 0 )  ?  16  :  32 ))	(8-1031)

nonZeroH = Min( nTbH, (trTypeVer==”IDT” ? ( 32: ( trTypeVer > 0 )  ?  16  :  32 ))	(8-1032)

The (nTbW)x(nTbH) array r of residual samples is derived as follows:

- When nTbH is greater than 1, each (vertical) column of scaled transform coefficients d[ x ][ y ] with x = 0..nonZeroW − 1, y = 0..nonZeroH − 1 is transformed to e[ x ][ y ] with x = 0..nonZeroW − 1, y = 0..nTbH − 1 by invoking the one-dimensional transformation process as specified in clause 8.7.4.2 for each column x = 0..nonZeroW − 1 with the height of the transform block nTbH, the non-zero height of the scaled transform coefficients nonZeroH, the list d[ x ][ y ] with y = 0..nonZeroH − 1 and the transform type variable trType set equal to trTypeVer as inputs, and the output is the list e[ x ][ y ] with y = 0..nTbH − 1.

- When nTbH and nTbW are both greater than 1, the intermediate sample values g[ x ][ y ] with x = 0..nonZeroW − 1, y = 0..nTbH − 1 are derived as follows:

g[ x ][ y ] = Clip3( CoeffMin, CoeffMax, ( e[ x ][ y ] + 64 )  >>  7 )	(8-1033)

- When nTbW is greater than 1, each (horizontal) row of the resulting array g[ x ][ y ] with x = 0..nonZeroW − 1, y = 0..nTbH − 1 is transformed to r[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 by invoking the one-dimensional transformation process as specified in clause 8.7.4.2 for each row y = 0..nTbH − 1 with the width of the transform block nTbW, the non-zero width of the resulting array g[ x ][ y ] nonZeroW, the list g[ x ][ y ] with x = 0..nonZeroW − 1 and the transform type variable trType set equal to trTypeHor as inputs, and the output is the list r[ x ][ y ] with x = 0..nTbW − 1.

… skipped…

- 8.7.4.2 Transformation process

Inputs to this process are:

- a variable nTbS specifying the horizontal sample size of transformed samples,

- a variable nonZeroS specifying the horizontal sample size of non-zero scaled transform coefficients,

- a list of scaled transform coefficients x[ j ] with j = 0..nonZeroS − 1,

- a transform kernel type variable trType.

- a variable bFlip specifying the direction of output transformed samples.

Output of this process is the list of transformed samples y[ i ] with i = 0..nTbS − 1.

The transformation matrix derivation process as specified in clause 8.7.4.3 in invoked with the transform size nTbS and the transform kernel Type trType as inputs, and the transformation maxtrix transMatrix as output.

Depending on the value of trType, the following applies:, the list of transformed samples y[ i ] with i = 0..nTbS − 1 is derived as follows:

- If trType is equal to 0, the following transform matrix multiplication applies:

y[i]= j=0nonZeroS-1transMatrix[i][j*26-Log2(nTbS) ]*x[j]  with i = 0..nTbS − 1	(8-1034)

- Otherwise (trType is equal to 1 or trType is equal to 2 or trType is equal to “IDT”), the following transform matrix multiplication applies:

y[bFlip ? (nTbS-1-i) : i]= j=0nonZeroS-1transMatrix[i][j]*x[j]  with i = 0..nTbS − 1	(8-1035)

- 8.7.4.3 Transformation matrix derivation process

Inputs to this process are:

- a variable nTbS specifying the horizontal sample size of scaled transform coefficients,

- the transformation kernel type trType.

Output of this process is the transformation matrix transMatrix.

The transformation matrix transMatrix is derived based on trType and nTbs as follows:

- Otherwise, if trType is equal to “IDT” and nTbs is equal to 4, the following applies:

If m is equal to n, transMatrix[ m ][ n ] = 128   with m=0..3, n=0..3		(8-Y1)

Otherwise (m!=n), transMatrix[ m ][ n ] = 0  with m=0..3, n=0..3			(8-Y2)

- Otherwise, if trType is equal to “IDT” and nTbs is equal to 8, the following applies:

If m is equal to n, transMatrix[ m ][ n ] = 181   with m=0..7, n=0..7		(8-Y3)

Otherwise (m!=n), transMatrix[ m ][ n ] = 0  with m=0..7, n=0..7			(8-Y4)

- Otherwise, if trType is equal to “IDT” and nTbs is equal to 16, the following applies:

If m is equal to n, transMatrix[ m ][ n ] = 256   with m=0..15, n=0..15		(8-Y5)

Otherwise (m!=n), transMatrix[ m ][ n ] = 0  with m=0..15, n=0..15		(8-Y6)

- Otherwise, if trType is equal to “IDT” and nTbs is equal to 32, the following applies:

If m is equal to n, transMatrix[ m ][ n ] = 362   with m=0..31, n=0..31		(8-Y7)

Otherwise (m!=n), transMatrix[ m ][ n ] = 0  with m=0..31, n=0..31		(8-Y8)

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).