| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0130-v1 |
| --- | --- |

| Title: | CE3: Reduced reference samples range for CCLM_L and CCLM_T(Tests 1.3.1, 1.3.2 and 1.3.3) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shuai Wan Junyan Huo Yanzhuo Ma Qihong Ran Xinwei Li Fuzheng Yang YuanFang Yu Yang Liu | Email: | swan@nwpu.edu.cn jyhuo@mail.xidian.edu.cn yzma@mail.xidian.edu.cn qhRan@stu.xidian.edu.cn xwli_xidian@stu.xidian.edu.cn fzhyang@mail.xidian.edu.cn yuyuanfang@oppo.com serena@oppo.com |
| Source: | Northwestern Polytechnical University, Xidian University, Guangdong OPPO Mobile Telecommunications Corp., Ltd |  |  |

_____________________________

# Abstract

This contribution presents the results of CE3 test 1.3.1, test 1.3.2 and test 1.3.3. The three tests skip initial quarter of reference samples for CCLM_L and CCLM_T. In test 1.3.1, the reference samples are limited to the last three quarter for CCLM_L and CCLM_T. In test 1.3.2, test 1.3.1 and test 1.2.1 is combined. After skipping the first quarter of reference samples for CCLM_L and CCLM_T, the number of the reference samples is reduced to 2 or 4 according to the size of the block. In test 1.3.3, test 1.3.1 and test 1.2.2 is combined. After skipping the first quarter of reference samples for CCLM_L and CCLM_T, the number of the reference samples is reduced to 4 for all block sizes.

.

Simulation results reportedly show BD rate on Y, Cb and Cr components and encoding/decoding time over VTM4.0:

- Test 1.3.1, skipping the first quarter reference samples for CCLM_L and CCLM_T:

  - For AI configuration: -0.02%, -0.06%, and -0.13%, with 100% EncT, 99% DecT;

  - For RA configuration: -0.01%, -0.11%, and -0.20%, with 100% EncT, 100% DecT;

- Test 1.3.2, test 1.3.1+test 1.2.1:

  - For AI configuration: -0.02%, 0.02%, and -0.08%, with 100% EncT, 99% DecT;

  - For RA configuration: 0.01%, -0.07%, and -0.18%, with 100% EncT, 100% DecT;

- Test 1.3.3, test 1.3.1+test 1.2.2:

  - For AI configuration: -0.03%, -0.06%, and -0.17%, with 100% EncT, 99% DecT;

  - For RA configuration: -0.01%, -0.23%, and -0.40%, with 100% EncT, 99% DecT.

# Introduction

The current VVC includes CCLM_L and CCLM_T modes. In the CCLM_T mode, only the above template is used to calculate the linear model parameters. To get more samples, the above template is extended to the size of (W+W).

Figure 1: CCLM_T mode

In the CCLM_L mode, only the left template is used to calculate the linear model parameters and the left template is extended to the size of (H+H).

Figure 2: CCLM_L mode

In the current VVC, a simplified line fitting method of the cross-component linear model (CCLM) has been adopted. This method uses two Luma samples and their corresponding chroma samples to obtain the linear model parameters α and β.

A

B

A

B

Figure 3:Illustration of the straight line between minimum and maximum Luma value

The linear model parameters α and β are obtained according to the following equations:

β= ∑Cn-α·∑LnN

α=yB-yAxB-xA                                        （1）

β= yA-αxA                                      （2）

# Proposed method

## 1. Test 1.3.1 skipping initial quarter reference samples for CCLM_L and CCLM_T

In test 1.3.1, the CCLM_L and CCLM_T modes skip the first quarter reference samples. The last three quarters of reference samples are used for CCLM_L and CCLM_T, as shown in Figure 4 and Figure 5. This method can reduce the number of comparisons in the MaxMin method in CCLM.

Figure 4: CCLM_T mode with 3/4 reference samples range

Figure 5: CCLM_L mode with 3/4 reference samples range

## 2. Test 1.3.2 : Test 1.3.1+Test 1.2.1

In test 1.2.1, the number of the reference samples is reduced to 2 or 4 according to the size of the block. The reference samples for the CCLM parameter calculation can be selected in the same way as introduced in JVET-M0211[1]. Figure 6 shows how to select 4 reference samples from a 16-pixel line in JVET-M0211.

Figure 6 The reference samples selection in JVET-M0211

The sampling interval and starting offset are calculated as follows:

-Original reference samples number: L=16

-Reduced reference samples number: N=4

-Sampling interval: ∆=L/N=4

-Start offset = ∆/2 =2

Test 1.3.2 combines the method of test 1.3.1 and test 1.2.1. Specifically, for the CCLM_L and CCLM_T modes, the initial offset which is set as the first quarter of the reference samples are skipped firstly. Then, reference samples are selected according to the method in JVET-M0211. If the number of remaining reference samples is less than 4, 2 samples are selected, otherwise 4 samples are selected.

3. Test 1.3.3 : Test 1.3.1+Test 1.2.2

Test 1.3.3 combines the method of test 1.3.1 and test 1.2.2. Firstly, for the CCLM_L and CCLM_T modes, the initial offset which is set as the first quarter of the reference samples are skipped.  Then, 4 reference samples are selected for CCLM, CCLM_L and CCLM_T for all sizes of blocks. Figure 7 shows how to select 4 reference samples from a 16-pixel line for CCLM_L and CCLM_T.

Figure 7 The reference samples selection in CCLM_L and CCLM_T

# Experimental results

The three tests are implemented based on VTM4.0. The coding performance is provided in Table 1, Table 2 and Table 3.

Test 1.3.1

Anchor: VTM4.0

Tested: CE3-1.3.1

Table 1. The results of test 1.3.1

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.17% | -0.33% | 100% | 101% |
| Class A2 | -0.02% | 0.17% | -0.05% | 100% | 100% |
| Class B | -0.01% | -0.16% | -0.11% | 99% | 96% |
| Class C | -0.02% | -0.08% | -0.18% | 100% | 100% |
| Class E | 0.01% | 0.02% | 0.06% | 100% | 100% |
| Overall | -0.02% | -0.06% | -0.13% | 100% | 99% |
| Class D | 0.00% | -0.08% | -0.20% | 100% | 102% |
| Class F | 0.01% | 0.00% | -0.03% | 100% | 102% |
|  |  |  | RandomAccess |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.26% | -0.13% | 100% | 100% |
| Class A2 | -0.03% | -0.02% | 0.05% | 100% | 101% |
| Class B | 0.00% | -0.19% | -0.39% | 100% | 100% |
| Class C | -0.01% | 0.04% | -0.19% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.11% | -0.20% | 100% | 100% |
| Class D | -0.04% | -0.16% | -0.05% | 100% | 100% |
| Class F | 0.02% | -0.07% | -0.11% | 100% | 100% |

Test 1.3.2

Anchor: VTM3.0

Tested: CE3-1.3.2

Table 2. The results of test 1.3.2

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | 0.07% | -0.33% | 100% | 100% |
| Class A2 | 0.02% | 0.35% | 0.21% | 99% | 98% |
| Class B | -0.01% | -0.31% | -0.24% | 100% | 98% |
| Class C | -0.04% | 0.01% | -0.02% | 100% | 99% |
| Class E | -0.01% | 0.18% | 0.05% | 100% | 97% |
| Overall | -0.02% | 0.02% | -0.08% | 100% | 99% |
| Class D | -0.02% | 0.22% | 0.17% | 100% | 100% |
| Class F | 0.01% | 0.17% | -0.03% | 100% | 98% |
|  |  |  | RandomAccess |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.06% | -0.15% | 100% | 100% |
| Class A2 | 0.02% | 0.25% | 0.11% | 100% | 100% |
| Class B | 0.01% | -0.46% | -0.53% | 100% | 100% |
| Class C | 0.02% | 0.18% | 0.01% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.07% | -0.18% | 100% | 100% |
| Class D | 0.02% | -0.08% | 0.30% | 100% | 102% |
| Class F | 0.03% | 0.19% | 0.00% | 100% | 100% |

Test 1.3.3

Anchor: VTM4.0

Tested: CE3-1.3.3

Table 3. The results of test 1.3.3

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | 0.04% | -0.36% | 100% | 100% |
| Class A2 | 0.01% | 0.22% | 0.10% | 100% | 99% |
| Class B | -0.02% | -0.32% | -0.31% | 100% | 99% |
| Class C | -0.05% | -0.17% | -0.15% | 99% | 99% |
| Class E | -0.02% | 0.11% | -0.03% | 100% | 98% |
| Overall | -0.03% | -0.06% | -0.17% | 100% | 99% |
| Class D | -0.03% | -0.16% | -0.14% | 99% | 99% |
| Class F | -0.02% | -0.04% | -0.37% | 99% | 97% |
|  |  |  | RandomAccess |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.04% | -0.25% | 100% | 100% |
| Class A2 | 0.02% | 0.16% | 0.06% | 99% | 99% |
| Class B | -0.01% | -0.55% | -0.72% | 100% | 100% |
| Class C | -0.01% | -0.25% | -0.46% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.23% | -0.40% | 100% | 100% |
| Class D | -0.04% | -0.17% | -0.13% | 100% | 103% |
| Class F | -0.02% | -0.22% | -0.34% | 100% | 100% |

# Conclusions

This contribution presents three tests about simplified CCLM method to reduce the number of reference samples. It can be seen that: for test 1.3.1, skipping initial quarter reference samples for CCLM_L and CCLM_T, can get -0.02%, -0.06%, and -0.13% BD-bitrate gain with 99% decoding time; test 1.3.2, which combines the method of test 1.3.1 and test 1.2.1, can get -0.02%, 0.02%, and -0.08% BD-bitrate gain with 99% decoding time; test 1.3.3, which combines the method of test 1.3.1 and test 1.2.2, achieves -0.03%, -0.06%, and -0.17% BD-bitrate gain with 99% decoding time.

- References

- J.-Y. Huo, X.-W. Li, X.-Y. Chai, J.-L. Wang, Y.-Z. Ma, F.-Z. Yang, S. Wan, Y.-F. Yu, Y. Liu, JVET-M0211, “CE3-related: Fixed Reference Samples Design for CCLM”, Jan. 2019.

# Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Northwestern Polytechnical University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).