| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0138 |
| --- | --- |

| Title: | AHG17: Carriage of reshaper model parameters |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Woong Il Choi Kwang Pyo Choi Kiho Choi Seoul R&D Campus 34, Seongchon-gil, Seocho-gu Seoul, 06765, Korea | Email: | woongil.choi@samsung.com kp5.choi@samsung.com kiho14.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd |  |  |

_____________________________

- Abstract

This contribution proposes extended usage of adaptation parameter set (APS) for carriage of reshaper model parameters as well as ALF parameters. In the last meeting, it is decided for ALF parameters to be carried by APS instead of tile group header for coding efficiency improvement by avoiding unnecessary redundant signaling of parameters in multiple tile groups. Based on the same reason, it is proposed to carry reshaper model parameters by APS instead of tile group header. To identify the type of parameters in the APS (whether ALF or reshaper model at least), APS type information is required in APS syntax as well as APS ID.

Introduction

In the last meeting, it is decided ALF parameters is carried by adaptation parameter set (APS) instead of tile group header due to avoid redundant transmission on multiple tile groups within a picture. It is introduced the key advantage of using APS is not only coding efficiency improvement by efficient signaling of parameters, but also support optimal delivery of VVC video in streaming applications. From the same motivation on this, we propose to carry reshaper model parameters by APS instead of embedded in tile group headers. Since the overhead bit of reshaper model parameters is about 100bits which is relatively not small, we believe APS signaling of those parameters should have impact on the coding efficiency.

Proposal

We propose the reshaper model parameters is signaled by APS. Since ALF parameters are also signaled by APS, APS type information is needed to identify the type of parameters in APS. When reshaper model parameters should be signaled by tile group header (tile_group_reshaper_model_present_flag is equal to 1), APS ID which carries reshaper model parameters is signaled in tile group header. The syntax and semantics of APS and tile group header for the proposed design are described as follows.

- Adaptation parameter set syntax and semantics

| adaptation_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| adaptation_parameter_set_id | u(5) |
| adaptation_parameter_set_type | u(1) |
| if( adaptation_parameter_set_type == 0 ) |  |
| alf_data( ) |  |
| else |  |
| reshaper_model( ) |  |
| aps_extension_flag | u(1) |
| if( aps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| aps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

adaptation_parameter_set_type identifies the type of parameters in APS. The value of adaptation_parameter_set_type shall be in the range of 0 to 1, inclusive. If adaptation_parameter_set_type is equal to 0, the ALF parameters are signaled. Otherwise, reshaper model parameters are signaled.

- General tile group header syntax and semantics

| tile_group_header( ) { | Descriptor |
| --- | --- |
| ... |  |
| if ( sps_reshaper_enabled_flag ) { |  |
| tile_group_reshaper_model_present_flag | u(1) |
| if ( tile_group_reshaper_model_present_flag ) |  |
| tile_group_reshaper_model ( )tile_group_aps_id |  |
| tile_group_reshaper_enable_flag | u(1) |
| if ( tile_group_reshaper_enable_flag && (!( qtbtt_dual_tree_intra_flag && tile_group_type == I ) ) ) |  |
| tile_group_reshaper_chroma_residual_scale_flag | u(1) |
| } |  |
| ... |  |
| } |  |

Patent rights declaration(s)

Samsung Electronics Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).