| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0153 |
| --- | --- |

| Title: | Non-CE9: On DMVR and GBI |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ruoyang Yu Du Liu | Email: | ruoyang.yu@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

In VTM-4.0, the decoder motion vector refinement (DMVR) uses bilateral matching method to refine bi-predictive motion vectors. It is reported that during the DMVR search, the generalized-bi prediction (GBi) weighting factor is not considered when measuring the differences between L0 prediction and L1 prediction. In other words, the L0 prediction and L1 prediction are still treated equally during the search when unequal GBi weights is used. The contribution proposes to disable DMVR for a CU when the CU uses unequal GBi weights. The method was tested under the VTM-4.0 random access (RA) common test configuration. The overall BD-rate results are reported to be 0.00%. For class F, which is not included in calculating the overall BD-rate results, the BD rate results are reported to be -0.10%.

# Background

## DMVR

The DMVR is applied when a current CU when the following criterions are fulfilled:

- The CU uses non-subblock merge with bi-predictive MVs.

- The number of luma samples in the CU is larger than 64. The CU height is larger than 8 luma samples.

- One reference picture has a lower POC value than the current picture, the other reference picture has a higher POC value than the current picture.

- Equal POC distances from both reference picture to the current picture.

When DMVR is applied, it searches around the initial MVs (MV0 and MV1) within a window. The window defines a set of MV offset. For each MV offset, a mirroring rule is followed for generating a new pair of MV0 and MV1, as shown in Figure 1. In other words, the following equations are applied.

| MV0’ = MV0 + MV_offset |  |
| --- | --- |
| MV1’ = MV0 – MV_offset |  |

The MV offset that gives the smallest difference between L0 prediction and L1 prediction will be chosen for generating the final MVs that used for motion compensation.

Figure 1. An example of DMVR search process

## GBi

GBi applies different weighting factor when combining L0 prediction and L1 prediction, i.e.

| P’(x, y) = ( (8 – w)* P0 (x, y) + w * P1(x ,y) + 4 ) >> 3 |  |
| --- | --- |

For non-low delay pictures, three weights {3, 4, 5} are allowed, which corresponds to three (L0, L1) weighting pair i.e. (3, 5), (4, 4) and (5, 3).

# Proposal

The current DMVR method treats the L0 prediction and L1 prediction equally during the search even when unequal GBi weights is used.

To keep the current search scheme unmodified, it is proposed to disable DMVR for a CU when the CU uses unequal GBi weights.

# Results

The proposed method was implemented in the VTM-4.0 reference software [1] and tested using the VTM common test configuration [2].

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.05% | 0.06% | 98% | 99% |
| Class A2 | 0.02% | 0.10% | 0.10% | 98% | 100% |
| Class B | -0.01% | 0.03% | -0.01% | 100% | 97% |
| Class C | 0.00% | -0.01% | -0.03% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.04% | 0.02% | 99% | 99% |
| Class D | 0.02% | -0.09% | -0.08% | 100% | 100% |
| Class F | -0.10% | -0.07% | -0.08% | 89% | 100% |

# Conclusion

The results indicate that DMVR does not bring benefits when unequal GBi weights is used. By disabling DMVR for CUs that use unequal GBi weights, -0.1% BD rate impact is noticed for class F.

It is proposed to adopt the proposal to the next version of VVC draft and VTM software.

# Reference

[1]. VTM-4.0 reference software, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-4.0

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-M1010.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).