| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0155 |
| --- | --- |

| Title: | Non-CE3: Chroma Intra Default Modes Modification Based on Statistics of Luminance Modes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Feiyang Zeng Dong Jiang Jucai Lin Cheng Fang | Email: | jiang_dong@dahuatech.com lin_jucai@dahuatech.com zeng_feiyang@dahuatech.com fang_cheng1@dahuatech.com |
| Source: | Dahua(ZHEJIANG DAHUA TECHNOLOGY CO.,LTD) |  |  |

_____________________________

- Abstract

This contribution selects the prediction modes of the corresponding luma block to replace the chroma default prediction modes. The simulation results show 0.07%, 0.58% and 0.59% coding gain in terms of average BD-rate of Y, Cb and Cr in all intra configuration with the same encoder complexity.

Introduction

The chroma intra mode candidate list in VTM-4.0[1] includes cross-component linear mode (CCLM) , DM and 4 default prediction modes Planar, Vertical, Horizontal and DC. If the DM is the same as the mode in default modes, the mode in default modes will be replaced by vertical diagonal mode. Table 1 lists the values of the candidate modes for different values of the DM.

Table 1: Chroma prediction default modes in VTM 4.0 of VVC.

| Chroma prediction modes | DM = PLANAR_IDX | DM=VER_IDX | DM=HOR_IDX | DM=DC_IDX | DM = Angular (except HOR_IDX and VER_IDX) |
| --- | --- | --- | --- | --- | --- |
| modelist [0] | VDIA_IDX | PLANAR_IDX | PLANAR_IDX | PLANAR_IDX | PLANAR_IDX |
| modelist [1] | VER_IDX | VDIA_IDX | VER_IDX | VER_IDX | VER_IDX |
| modelist [2] | HOR_IDX | HOR_IDX | VDIA_IDX | HOR_IDX | HOR_IDX |
| modelist [3] | DC_IDX | DC_IDX | DC_IDX | VDIA_IDX | DC_IDX |
| modelist [4] | LM | LM | LM | LM | LM |
| modelist [5] | MDLM_L | MDLM_L | MDLM_L | MDLM_L | MDLM_L |
| modelist [6] | MDLM_T | MDLM_T | MDLM_T | MDLM_T | MDLM_T |
| modelist [7] | DM | DM | DM | DM | DM |

Algorithm description

With the separate tree method, a chroma block can correspond to multiple luma blocks. This contribution proposes to use modes of corresponding luma blocks to replace the default modes Vertical , Horizontal and DC modes.

Firstly, obtain the 8 prediction modes of the luma blocks where the proposed positions are located. The 8 proposed luma prediction mode acquisition positions are TL1, TR1, BL1, BR1 and TL, TR, BL, BR, shown in Figure 1.

Figure 2. Proposed positions in the corresponding luma block

Obtain the 8 prediction modes of the luma blocks where the proposed positions are located. Combine the same prediction modes and count the number of occurrences of each mode. Select the three modes that have the most occurrences and satisfy the conditions as the replacement modes. The condition to be met is that the selected modes cannot be the same as the modelist[0] and modelist[7], nor can it be the same.

Replace the modelist[1]、modelist[2] and modelist[3] respectively after selecting three replacement modes. If there is mode A in the replacement modes is the same as the modelist[1] or modelist[2] or modelist[3], the same mode as A in the default modes cannot be replaced, and mode A no longer replaces the other default modes.

If there is only one replacement mode that satisfies the condition, only modelist[1] is replaced. And if there are two replacement modes that satisfies the condition, modelist[1] and modelist[2] are replaced.

Experimental results

The proposed modification is implemented on VTM-4.0 reference software. The simulations are performed on Linux cluster using VTM AI configurations under the common test condition described in JVET-M1010 [2].

Table 1. Performance of the proposed method

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.22% | -0.30% | 100% | 100% |
| Class A2 | -0.06% | -0.26% | -0.34% | 100% | 100% |
| Class B | -0.06% | -0.63% | -0.48% | 100% | 100% |
| Class C | -0.07% | -0.59% | -0.64% | 100% | 99% |
| Class E | -0.12% | -1.14% | -1.25% | 100% | 98% |
| Overall | -0.07% | -0.58% | -0.59% | 100% | 100% |
| Class D | -0.03% | -0.31% | -0.65% | 100% | 97% |
| Class F | -0.07% | -0.36% | -0.34% | 100% | 100% |

Conclusion

The chroma intra default modes modification is implemented on VTM-4.0 reference software. The experimental results reportedly show that under AI configuration , it can achieve 0.07%, 0.58%and 0.59% coding gain in terms of the average BD-rate of Y, Cb and Cr with 100% enc-time.

References

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Suehring “JVET common test conditions and software reference configurations for SDR video,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-M1010, Marrakech, MA, 9–18 Jan. 2019.

Patent rights declaration(s)

Dahua may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).