| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0158-v2 |
| --- | --- |

| Title: | CE9-related: Alternative method of SAD based early termination for BDOF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kyohei Unno Kei Kawamura Sei Naito 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81-70-4560-2752 ky-unno@kddi-research.jp ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

In this contribution, it is proposed that an alternative method of SAD based early termination for BDOF (Bi-Directional Optical-Flow). The proposed method reuses SADs calculated at DMVR (Decoder-side Motion Vector Refinement) stage instead of SADs calculated at BDOF stage for early termination. The proposed method has only one additional comparison operation (thresholding) to CE9-2.1 (Disable SAD based early termination). BD-rate for luma is 0.01% (RA) in comparison with VTM-4.0. Decoding time is 2% faster than VTM-4.0 and 7% faster than CE9-2.1.

Introduction

SAD based early termination is included to BDOF (Bi-Directional Optical-Flow) at VVC WD4 [1] and VTM-4.0. That is effective to reduce software decoding time. On the other hand, disabling SAD based early termination is proposed from hardware implementation aspect as CE9-2.1 [2]. By disabling SAD based early termination, a buffer latency issue would be solved but software decoding time would increase from VTM-4.0. In this contribution, it is proposed that an alternative method of SAD based early termination for BDOF to solve the latency issue on hardware without increasing software decoding time. The remainder of this contribution is divided into two parts;

- Proposed method, and

- Experimental results under the CTC.

Proposed Method

The proposed method is an alternative method of SAD based early termination for BDOF. Details are as follows.

- SADs are not calculated in the BDOF stage for early termination (same as CE9-2.1).

- SADs and MVDs calculated in the DMVR stage are reused for early termination.

- At a block not applied DMVR, no early terminations are applied.

- Because BDOF is processed by sub-PU for a block applied DMVR, early termination is processed by sub-PU of DMVR.

- If the conditions described as follows are true, BDOF is not applied to the sub-PU.

Conditions for Test 1

If the following condition is true, BDOF is not applied to the sub-PU.

- The smallest SAD at integer distance search stage in DMVR is less than the threshold.

The threshold is defined as 16 * W * (H / 2). Here, W means width of the sub-PU, and H means height of the sub-PU. Because SADs are calculated for every other row in DMVR, the height of sub-PU is divided by 2.

Conditions for Test 2

If all of the following conditions are true, BDOF is not applied to the sub-PU.

- The MVD is (0, 0). MVD means motion vector difference between before DMVR and after DMVR.

- The SAD at initial MV of DMVR (= resulting MV of DMVR) is less than the threshold.

The threshold is same as Test 1.

As described above, the proposed method has only a few additional comparisons on top of CE9-2.1.

Experimental Results

The proposed method is implemented on top of the CE9-2.1 software based on VTM-4.0. In order to verify the performance of the proposed method, we have run simulations according to the common test conditions of JVET-M1010 [3].

Results under the common test conditions

Computing platforms

The software was compiled by the same compiler which is GCC 4.8.5 on 64-bit CentOS Linux on the different cluster PCs. Since the same coding condition utilizes the same cluster PCs, running time can be comparable within the same condition. It is noted that we disabled the function of Hyperthreading and TurboBoost for getting accurate running times.

Test results

Table 1 shows the experimental results of Test 1. BD-rate for luma is 0.01% for RA in comparison with VTM-4.0. Decoding time is 2% faster than VTM-4.0 and 7% faster than CE9-2.1 (shown in Table 3).

Table 1 Experimental results of Test 1 for RA condition.

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.08% | 0.01% | 100% | 98% |
| Class A2 | 0.02% | 0.03% | 0.05% | 100% | 98% |
| Class B | 0.01% | 0.03% | -0.02% | 100% | 99% |
| Class C | 0.00% | 0.02% | -0.03% | 100% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.04% | 0.00% | 100% | 98% |
| Class D | 0.02% | -0.03% | -0.08% | 100% | 100% |
| Class F | 0.01% | 0.01% | 0.00% | 100% | 98% |

Table 2 shows the experimental results of Test 2. BD-rate for luma is 0.01% for RA in comparison with VTM-4.0. Decoding time is 3% slower than VTM-4.0 and 2% faster than CE9-2.1.

Table 2 Experimental results of Test 2 for RA condition.

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.02% | 0.02% | 100% | 103% |
| Class A2 | -0.02% | 0.01% | 0.06% | 100% | 103% |
| Class B | -0.01% | 0.02% | -0.03% | 101% | 103% |
| Class C | -0.01% | -0.06% | -0.06% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.01% | -0.01% | 100% | 103% |
| Class D | 0.00% | -0.01% | -0.02% | 100% | 102% |
| Class F | -0.02% | -0.03% | -0.01% | 100% | 100% |

Table 3 shows the experimental results of CE9-2.1 done by KDDI.

Table 3 Experimental results of CE9-2.1 (done by KDDI) for RA condition.

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.04% | 0.02% | 101% | 105% |
| Class A2 | -0.04% | 0.02% | 0.05% | 101% | 106% |
| Class B | -0.02% | -0.03% | 0.01% | 101% | 106% |
| Class C | -0.01% | -0.04% | -0.05% | 101% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.01% | 0.00% | 101% | 105% |
| Class D | -0.02% | -0.01% | -0.07% | 101% | 105% |
| Class F | -0.01% | 0.00% | 0.04% | 101% | 111% |

Conclusion

In this contribution, it was proposed that an alternative method of SAD based early termination for BDOF (Bi-Directional Optical-Flow). The proposed method reused SADs calculated at DMVR (Decoder-side Motion Vector Refinement) stage instead of SADs calculated at BDOF stage for early termination. The proposed method had only one additional comparison operation (thresholding) to CE9-2.1 (Disable SAD based early termination). BD-rate for luma was 0.01% (RA) in comparison with VTM-4.0. Decoding time was 2% faster than VTM-4.0 and 7% faster than CE9-2.1.

- Acknowledgment

We would like to thank SHARP for doing crosschecking of our proposal.

References

- B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 4)”, JVET-M1001, Marrakech, MA, Jan. 2019.

- S. Esenlik, B. Wang, A. M. Kotra, H. Gao, J. Chen, K. Kondo, M. Ikeda and T. Suzuki, “CE9: Disabling SAD based early termination of BDOF (CE9-2.1)”, JVET-N0177, Geneva, CH, Mar. 2019.

- F. Bossen, J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-M1010, Marrakech, MA, Jan. 2019.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).