| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0176-v2 |
| --- | --- |

| Title: | Non-CE8: IBC Merge List Simplification |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Gao Semih Esenlik Biao Wang Anand Meher Kotra Jianle Chen | Tel: Email: | han.gao@huawei.com semih.esenlik@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co.,Ltd. |  |  |

_____________________________

# Abstract

In VVC Draft 4.0, the IBC merge list is constructed in a similar way as the regular merge with maximum 9 pruning. The proposed methods simplify the spatial candidate pruning and HMVP candidate pruning for IBC merge list construction. The proposed methods can reduce the pruning from 9 to 3 with minor coding efficiency different for both CTC ClassF and TGM sequences.

# Introduction

In VTM4.0, the IBC merge list is constructed with spatial candidates, HMVP candidates and pairwise candidate.

The special candidates are possible pruned maximum 5 times with A1-B1, B0-B1, A0-A1, B2-A1, B2-B1.

The HMVP candidates are possible pruned maximum 4 times with Hk - A1, Hk-B1, Hk-1 - A1, Hk-1 - B1, where Hk is the last entry in the HMVP list and Hk-1 is the second last entry in the HMVP list.

# Problem Statement

Since IBC mode is considered as a separated prediction mode from inter and intra prediction, in the worst case 9 pruning are complicated for the IBC merge list construction.

# Proposed Method

## Reducing spatial merging candidate pruning

### Alternative 1

In the alternative 1, the spatial candidate pruning are removed except A1-B1 pruning for IBC merge list construction. In other words, the maximal spatial neighboring candidate pruning is reduced from 5 to 1, the candidates B0, A0 and B2 are inserted into merge list without pruning if they are available.

### Alternative 2

Since the A0, B0 and B2 candidates are not so frequently used, in the alternative 2, the spatial candidates A0, B0 and B2 are removed completely. In other word, the maximal spatial neighboring candidate pruning is reduced from 5 to 1, since only A1 and B1 spatial candidates are used in merge list construction.

## Reducing HMVP candidate pruning

### Alternative 1

In the alternative 1, the number of HMVP candidate pruning for IBC merge list construction is limited as 2. In other words,

- If both A1 and B1 are already in the list, Hk is pruned with A1 and B1

- Otherwise if A1 is in the list and B1 is not in the list, Hk and Hk-1 is pruned with A1

- Otherwise if B1 is in the list and A1 is not in the list, Hk and Hk-1 is pruned with B1

- Otherwise, no HMVP pruning

In this method, the worst case HMVP candidate pruning is reduced from 4 to 2

### Alternative 2

In the alternative 2, the number of HMVP candidate is only pruned with the first spatial candidate for IBC merge list construction. In other words,

- If A1 is in the list, Hk and Hk-1 is pruned with A1

- Otherwise if B1 is in the list and A1 is not in the list, Hk and Hk-1 is pruned with B1

- Otherwise, no HMVP pruning

In this method, the worst case HMVP candidate pruning is reduced from 4 to 2

# Simulation results

Tables below shows the simulation results using VTM 4.0, both in the anchor and tests with IBC and HashME on for all sequence.

Table 1, first alternative of spatial candidate pruning reduction, removing spatial pruning expect A1-B1

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | -0.05% | 99% | 100% |
| Class A2 | 0.01% | 0.05% | 0.03% | 99% | 99% |
| Class B | 0.00% | 0.00% | -0.03% | 99% | 100% |
| Class C | 0.01% | 0.03% | 0.06% | 99% | 99% |
| Class E | 0.02% | -0.07% | 0.04% | 99% | 99% |
| Overall | 0.01% | 0.00% | 0.01% | 99% | 100% |
| Class D | 0.02% | -0.04% | 0.07% | 99% | 98% |
| Class F | 0.08% | -0.07% | -0.08% | 99% | 99% |
| TGM | 0.28% | 0.21% | 0.21% | 100% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | 0.01% | -0.14% | -0.22% | 100% | 100% |
| Class C | 0.00% | 0.07% | -0.06% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | 0.01% | -0.08% | 0.01% | 100% | 99% |
| Class F | 0.00% | -0.03% | -0.09% | 99% | 97% |
| TGM | 0.16% | 0.20% | 0.13% | 100% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class E | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| TGM | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

Table 2, second alternative of spatial candidate pruning reduction, removing spatial candidate except A1 and B1

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | -0.02% | 99% | 100% |
| Class A2 | 0.01% | 0.00% | 0.00% | 99% | 100% |
| Class B | -0.01% | -0.03% | 0.00% | 99% | 101% |
| Class C | 0.00% | 0.07% | -0.02% | 99% | 99% |
| Class E | -0.01% | -0.08% | 0.03% | 99% | 101% |
| Overall | 0.00% | -0.01% | 0.00% | 99% | 100% |
| Class D | 0.00% | -0.04% | 0.05% | 99% | 99% |
| Class F | 0.02% | 0.01% | -0.06% | 99% | 99% |
| TGM | 0.01% | -0.01% | -0.02% | 99% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.04% | -0.02% | 101% | 100% |
| Class A2 | -0.02% | 0.00% | -0.01% | 101% | 100% |
| Class B | 0.00% | -0.15% | -0.12% | 100% | 100% |
| Class C | -0.02% | 0.12% | -0.05% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.03% | -0.06% | 100% | 100% |
| Class D | 0.02% | -0.05% | 0.19% | 100% | 100% |
| Class F | -0.02% | -0.10% | -0.17% | 100% | 100% |
| TGM | 0.01% | 0.02% | -0.04% | 100% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -0.04% | -0.19% | 0.02% | 100% | 99% |
| Class E | 0.06% | -0.49% | 0.10% | 101% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.05% | 1.12% | -0.30% | 101% | 102% |
| Class F | -0.15% | -0.12% | -0.62% | 101% | 101% |
| TGM | 0.03% | -0.02% | -0.04% | 101% | 99% |

Table 3, first alternative of HMVP candidate pruning reduction, maximum 2 HMVP pruning

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | -0.02% | 99% | 100% |
| Class A2 | 0.00% | -0.04% | -0.03% | 99% | 100% |
| Class B | 0.00% | 0.03% | -0.03% | 99% | 100% |
| Class C | 0.00% | 0.01% | -0.04% | 99% | 100% |
| Class E | 0.00% | 0.02% | 0.03% | 99% | 100% |
| Overall | 0.00% | 0.01% | -0.02% | 99% | 100% |
| Class D | 0.00% | -0.01% | 0.00% | 100% | 99% |
| Class F | 0.03% | 0.07% | 0.06% | 99% | 99% |
| TGM | 0.08% | 0.04% | 0.03% | 100% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | 0.01% | 100% | 100% |
| Class A2 | 0.02% | 0.03% | -0.05% | 100% | 100% |
| Class B | 0.02% | -0.02% | -0.02% | 100% | 100% |
| Class C | -0.02% | 0.07% | -0.02% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.02% | -0.02% | 100% | 100% |
| Class D | 0.00% | 0.18% | -0.02% | 100% | 99% |
| Class F | 0.00% | -0.04% | -0.13% | 100% | 100% |
| TGM | 0.08% | 0.14% | 0.05% | 100% | 100% |

Table 4, second alternative of HMVP candidate pruning reduction, Hk and Hk-1 pruned with first spatial candidate

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class B | 0.00% | 0.01% | 0.00% | 99% | 100% |
| Class C | 0.00% | 0.01% | -0.05% | 99% | 99% |
| Class E | 0.01% | -0.02% | 0.02% | 98% | 96% |
| Overall | 0.00% | 0.00% | -0.01% | 99% | 99% |
| Class D | 0.00% | -0.01% | 0.01% | 99% | 96% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| TGM | 0.10% | 0.03% | 0.05% | 99% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | 0.01% | -0.02% | -0.16% | 99% | 99% |
| Class C | -0.02% | 0.12% | -0.01% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | 0.01% | 0.13% | -0.05% | 100% | 99% |
| Class F | 0.00% | -0.06% | -0.08% | 99% | 100% |
| TGM | 0.05% | 0.05% | 0.05% | 100% | 100% |

# Draft specification changes

The draft text changes are based on [1], the proposed method is highlighted as follow:

## Method 1

#### 8.6.3.3 Derivation process for spatial merging candidates

…

For the derivation of availableFlagB0 and mvB0 the following applies:

- The luma location ( xNbB0, yNbB0 ) inside the neighbouring luma coding block is set equal to ( xCb + cbWidth, yCb − 1 ).

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbB0, yNbB0 ) as inputs, and the output is assigned to the block availability flag availableB0.

- The variables availableFlagB0 and mvB0 are derived as follows:

  - If one or more of the following conditions are true, availableFlagB0 is set equal to 0 and both components of   mvB0 are set equal to 0:

  - availableB0 is equal to FALSE.

  - availableB1 is equal to TRUE and the luma locations ( xNbB1, yNbB1 ) and ( xNbB0, yNbB0 ) have the same motion vectors.

  - availableA1 is equal to TRUE and the luma locations ( xNbA1, yNbA1 ) and ( xNbB0, yNbB0 ) have the same motion vectors.

- Otherwise, availableFlagB0 is set equal to 1 and the following assignments are made:

mvB0 = MvL0[ xNbB0 ][ yNbB0 ]		(8-981)

For the derivation of availableFlagA0 and mvA0 the following applies:

- The luma location ( xNbA0, yNbA0 ) inside the neighbouring luma coding block is set equal to ( xCb − 1,  yCb + cbWidth ).

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbA0, yNbA0 ) as inputs, and the output is assigned to the block availability flag availableA0.

- The variables availableFlagA0 and mvA0 are derived as follows:

- If one or more of the following conditions are true, availableFlagA0 is set equal to 0 and both components of  mvA0 are set equal to 0:

  - availableA0 is equal to FALSE.

  - availableA1 is equal to TRUE and the luma locations ( xNbA1, yNbA1 ) and ( xNbA0, yNbA0 ) have the same motion vectors.

  - availableB1 is equal to TRUE, the luma locations ( xNbB1, yNbB1 ) and ( xNbA0, yNbA0 ) have the same motion vectors.

  - availableB0 is equal to TRUE, the luma locations ( xNbB0, yNbB0 ) and ( xNbA0, yNbA0 ) have the same motion vectors.

- Otherwise, availableFlagA0 is set equal to 1 and the following assignments are made:

mvA0 = MvL0[ xNbA0 ][ yNbA0 ]		(8-982)

For the derivation of availableFlagB2 and mvB2 the following applies:

- The luma location ( xNbB2, yNbB2 ) inside the neighbouring luma coding block is set equal to ( xCb − 1, yCb − 1 ).

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbB2, yNbB2 ) as inputs, and the output is assigned to the block availability flag availableB2.

- The variables availableFlagB2 and mvB2 are derived as follows:

- If one or more of the following conditions are true, availableFlagB2 is set equal to 0 and both components of  mvB2 are set equal to 0:

  - availableB2 is equal to FALSE.

  - availableA1 is equal to TRUE and the luma locations ( xNbA1, yNbA1 ) and ( xNbB2, yNbB2 ) have the same motion vectors.

  - availableB1 is equal to TRUE and the luma locations ( xNbB1, yNbB1 ) and ( xNbB2, yNbB2 ) have the same motion vectors.

  - availableB0 is equal to TRUE and the luma locations ( xNbB0, yNbB0 ) and ( xNbB2, yNbB2 ) have the same motion vectors.

  - availableA0 is equal to TRUE and the luma locations ( xNbA0, yNbA0 ) and ( xNbB2, yNbB2 ) have the same motion vectors.

- Otherwise, availableFlagB2 is set equal to 1 and the following assignments are made:

mvB2 = MvL0[ xNbB2 ][ yNbB2 ]		(8-983)

## Method 2

#### 8.6.3.3 Derivation process for spatial merging candidates

…

For the derivation of availableFlagB0 and mvB0 the following applies:

- The luma location ( xNbB0, yNbB0 ) inside the neighbouring luma coding block is set equal to ( xCb + cbWidth, yCb − 1 ).

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbB0, yNbB0 ) as inputs, and the output is assigned to the block availability flag availableB0.

- The variables availableFlagB0 and mvB0 are derived as follows:

  - If one or more of the following conditions are true, availableFlagB0 is set equal to 0 and both components of   mvB0 are set equal to 0:

  - availableB0 is equal to FALSE.

  - availableB1 is equal to TRUE and the luma locations ( xNbB1, yNbB1 ) and ( xNbB0, yNbB0 ) have the same motion vectors.

  - availableA1 is equal to TRUE and the luma locations ( xNbA1, yNbA1 ) and ( xNbB0, yNbB0 ) have the same motion vectors.

- Otherwise, availableFlagB0 is set equal to 1 and the following assignments are made:

mvB0 = MvL0[ xNbB0 ][ yNbB0 ]		(8-981)

For the derivation of availableFlagA0 and mvA0 the following applies:

- The luma location ( xNbA0, yNbA0 ) inside the neighbouring luma coding block is set equal to ( xCb − 1,  yCb + cbWidth ).

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbA0, yNbA0 ) as inputs, and the output is assigned to the block availability flag availableA0.

- The variables availableFlagA0 and mvA0 are derived as follows:

- If one or more of the following conditions are true, availableFlagA0 is set equal to 0 and both components of  mvA0 are set equal to 0:

  - availableA0 is equal to FALSE.

  - availableA1 is equal to TRUE and the luma locations ( xNbA1, yNbA1 ) and ( xNbA0, yNbA0 ) have the same motion vectors.

  - availableB1 is equal to TRUE, the luma locations ( xNbB1, yNbB1 ) and ( xNbA0, yNbA0 ) have the same motion vectors.

  - availableB0 is equal to TRUE, the luma locations ( xNbB0, yNbB0 ) and ( xNbA0, yNbA0 ) have the same motion vectors.

- Otherwise, availableFlagA0 is set equal to 1 and the following assignments are made:

mvA0 = MvL0[ xNbA0 ][ yNbA0 ]		(8-982)

For the derivation of availableFlagB2 and mvB2 the following applies:

- The luma location ( xNbB2, yNbB2 ) inside the neighbouring luma coding block is set equal to ( xCb − 1, yCb − 1 ).

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbB2, yNbB2 ) as inputs, and the output is assigned to the block availability flag availableB2.

- The variables availableFlagB2 and mvB2 are derived as follows:

- If one or more of the following conditions are true, availableFlagB2 is set equal to 0 and both components of  mvB2 are set equal to 0:

  - availableB2 is equal to FALSE.

  - availableA1 is equal to TRUE and the luma locations ( xNbA1, yNbA1 ) and ( xNbB2, yNbB2 ) have the same motion vectors.

  - availableB1 is equal to TRUE and the luma locations ( xNbB1, yNbB1 ) and ( xNbB2, yNbB2 ) have the same motion vectors.

  - availableB0 is equal to TRUE and the luma locations ( xNbB0, yNbB0 ) and ( xNbB2, yNbB2 ) have the same motion vectors.

  - availableA0 is equal to TRUE and the luma locations ( xNbA0, yNbA0 ) and ( xNbB2, yNbB2 ) have the same motion vectors.

- Otherwise, availableFlagB2 is set equal to 1 and the following assignments are made:

mvB2 = MvL0[ xNbB2 ][ yNbB2 ]		(8-983)

## Method 3

#### 8.6.3.5 Derivation process for history-based merging candidates

…

The variables isPrunedA1 and isPrunedB1 are set both equal to FALSE.

The variable numHmvpPrune is set equal to 1 if avalibleA1 and avalbile B1 are TRUE and merge canddiate A1 is not equal to B1, otherwise variable numHmvpPrune is set equal to 2

For each candidate in HmvpIbcCandList[ hMvpIdx ] with index hMvpIdx = 1..NumHmvpIbcCand, the following ordered steps are repeated until numCurrMergeCand is equal to ( MaxNumMergeCand − 1):

- The variable sameMotion is derived as follows:

  - If all of the following conditions are true for any merging candidate N with N being A1 or B1, sameMotion and isPrunedN are both set equal to TRUE:

  - hMvpIdx is less than or equal to 2numHmvpPrune.

  - The candidate HmvpIbcCandList[NumHmvpIbcCand − hMvpIdx] is equal to the merging candidate N.

  - isPrunedN is equal to FALSE.

  - Otherwise, sameMotion is set equal to FALSE.

## Method 4

#### 8.6.3.5 Derivation process for history-based merging candidates

…

The variables isPrunedA1 and isPrunedB1 are set both equal to FALSE.

For each candidate in HmvpIbcCandList[ hMvpIdx ] with index hMvpIdx = 1..NumHmvpIbcCand, the following ordered steps are repeated until numCurrMergeCand is equal to ( MaxNumMergeCand − 1):

- The variable sameMotion is derived as follows:

  - If all of the following conditions are true for any merging candidate A1 N with N being A1 or B1, sameMotion and isPrunedA1N are both set equal to TRUE:

  - hMvpIdx is less than or equal to 2.

  - The candidate HmvpIbcCandList[NumHmvpIbcCand − hMvpIdx] is equal to the merging candidate A1N.

  - isPrunedA1N is equal to FALSE.

  - available A1 is TRUE

  - Otherwise If all of the following conditions are true for any merging candidate B1, sameMotion and isPrunedB1 are both set equal to TRUE:

  - hMvpIdx is less than or equal to 2.

  - The candidate HmvpIbcCandList[NumHmvpIbcCand − hMvpIdx] is equal to the merging candidate B1.

  - isPrunedB1 is equal to FALSE.

  - Otherwise, sameMotion is set equal to FALSE.

# Conclusion

The proposed methods reduce the IBC merge list pruning from 9 to 3 without significant penalty in the coding efficiency. The methods are proposed to be included in the next version of VVC draft.

# References

- “Versatile Video Coding (Draft 4)”, JVET-M1001, B. Bross, J. Chen, S. Liu

- “JVET common test conditions and software reference configurations for SDR video ”, JVET-M1010, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring

- “Description of Core Experiment 8 (CE8): Screen Content Coding Tools ”, JVET-M1028, X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu

- “CE4.3.1: Shared merging candidate list”, JVET-M0170, C.-C. Chen, Y.-C. Lin, M.-S. Chiang, C.-W. Hsu, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei

# Patent rights declaration(s)

Huawei Technologies Co.,Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).