| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0179-v2 |
| --- | --- |

| Title: | Non-CE9: Reducing bi-linear interpolation buffer requirement for DMVR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Semih Esenlik Han Gao Anand Meher Kotra Biao Wang Jianle Chen | Tel: Email: | 0049 162 210 6260 semih.esenlik@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

A new row sub-sampling method is proposed for the SAD computation of DMVR. According to the proposed method, the even and odd sample rows are used in an alternating manner depending on the vertical coordinate of the search point in DMVR search space. Accordingly approximately half of the rows of the prediction generated by bilinear interpolation is never used by SAD calculation, hence need not be generated or stored. It is asserted that the above modification reduces the necessary memory to store bilinear interpolation result, and the necessity to perform vertical interpolation by approximately 50%.

Negligible degradation in BD-rate performance is observed. The modification is proposed to be included in next VVC specification draft.

# Introduction

In VTM4.0, the SAD computation for MV refinement search is performed based on sub-sampled rows, which means every second row of samples in a block are used to perform SAD computation. The sub-sampling of the 8x8 block is shown as an example in the below figure:

Figure 1: Subsampling of samples in VTM4.0 for SAD computation

# Problem Definition

In VTM4.0 the maximum block size applying DMVR is restricted to 16x16 samples, where a coding block is implicitly divided into 16x16 sub-PUs if it is bigger. In order obtain the refined motion vectors for the 16x16 block, bilinear interpolation need to be performed to obtain prediction of 20x20 reference blocks. In other words in order to perform motion vector refinement for a block with a size of 16x16 samples, 20x20 samples need to be generated using bilinear interpolation.

# Proposed Solution

According to the proposed solution, the rows that are used in SAD computation are selected in an alternating manner, depending on the search point in the search space. The search points that are in the first, third and fifth row of the search space utilize the row sub-sampling pattern with odd lines (right sub-sampling pattern in the below figure). On the other hand the search points that are in the second and fourth rows of the search space utilize the row sub-sampling pattern with even lines (left sub-sampling pattern in the below figure).

Figure 2: Proposed subsampling patterns based on search point coordinate in the search space

Accordingly approximately half of the rows of the prediction generated by bilinear interpolation is never used by SAD calculation, hence need not be generated or stored. In the case of 16x16 maximum DMVR block size, 20x11 samples need to be generated by bilinear interpolation instead of 20x20 samples in VTM4.0.

It is asserted that the above modification reduces the necessary memory to store bilinear interpolation result, as well as the necessity to perform vertical interpolation by approximately 50%.

## 2-D bilinear interpolation without intermediate output

In the case of 2-D bilinear interpolation with fractional sample position in both the horizontal and vertical directions, the currently adopted DMVR performs the interpolation in 2 stages. The horizontal interpolation outputs are first produced. The vertical interpolation outputs are produced from the horizontally interpolated output values. This was being done as the horizontal interpolation outputs on a given row can be used across the vertical interpolation output for 2 different output rows.

However, with the proposed vertical decimation by a factor of 2 while producing the output, the horizontal interpolation of every pair of rows is required only for producing one vertically interpolated output row. Hence, it is further proposed to perform the 2-D bilinear interpolation as a joint interpolation without any intermediate normative output. This will help to eliminate the line buffers (2 lines x 2 references) associated with the intermediate output in a hardware implementation.

In addition, up to input bit-depths of 10-bits, it is desirable to fit the intermediate outputs of the interpolation within 16-bits. Currently, the fractional sample positions are maintained in 4-bit precision. With this, the interpolated output before final right shift can go up to 18-bits precision. Hence, it is further suggested to reduce the bit-depths of the fractional sample positions to 3-bits before the interpolation. In other words, the fractional sample positions are converted to 1/8th of a pixel accuracy when 2-D interpolation is required.

If  and  are the fractional sample offsets from the integer sample grid, the proposed single stage 2-D interpolated value val for a fractional sample position with the integer sample values a, b, c, and d to its top-left, top-right, bottom-left, and bottom-right respectively is given by:

val = ((8 – )*(8 – )*a + *(8 – )*b +  (8 – )**c +  **d + 32) >> (IBD-4)

The output of the interpolation filtering is 10 bits, irrespective of the internal bit-depth.

# Experimental Results

The simulations are performed according to CTC described in [1].

|  |  |  | Over VTM-4.0 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.02% | -0.02% | 101% | 101% |
| Class A2 | 0.01% | -0.01% | 0.02% | 101% | 100% |
| Class B | 0.03% | 0.00% | -0.02% | 101% | 102% |
| Class C | 0.01% | -0.03% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.01% | -0.01% | 101% | 101% |
| Class D | 0.00% | -0.07% | -0.08% | 101% | 102% |
| Class F | -0.01% | 0.01% | -0.01% | 100% | 101% |

Table 1: Simulation results according to random access configuration

## Experimental results for 2-D bilinear interpolation without intermediate output

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM4.0 |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |
| Class A1 | -0.01% |  | -0.02% |  | 0.05% |  | 99% |  | 100% |
| Class A2 | -0.02% |  | -0.01% |  | -0.02% |  | 100% |  | 100% |
| Class B | -0.02% |  | 0.00% |  | -0.01% |  | 99% |  | 98% |
| Class C | -0.02% |  | -0.08% |  | -0.05% |  | 100% |  | 100% |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.02% |  | -0.03% |  | -0.01% |  | 100% |  | 99% |
| Class D | -0.03% |  | -0.07% |  | -0.08% |  | 100% |  | 99% |
| Class F (optional) | -0.02% |  | -0.02% |  | -0.03% |  | 100% |  | 99% |

# Conclusion

A new row sub-sampling method is proposed for the SAD computation of DMVR. According to the proposed method the even and odd sample rows are used in an alternating manner depending on the vertical position of the search point in DMVR search space. Accordingly approximately half of the rows of the prediction generated by bilinear interpolation is never used by SAD calculation, hence need not be generated or stored. It is asserted that the above modification reduces the necessary memory to store bilinear interpolation result, and the necessity to perform vertical interpolation by approximately 50%.

0.02% increase in luma BD-rate is observed for RA configuration. The modification is proposed to be included in next VVC specification draft.

# References

- “JVET common test conditions and software reference configurations”, JVET-M1010, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).