| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0194 |
| --- | --- |

| Title: | CE6-related: Context selection of last non-zero coefficient position coding based on reduced TU size (related to JVET-M0297 and JVET-M0251/M0257) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Moonmo Koo Jungah Choi Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Mehdi Salehifar Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +1-858-635-5226 moonmo.koo@lge.com jungah.choi@lge.com jaehyun.lim@lge.com mehdi.salehifar@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In 13th JVET meeting, reduced 32-point MTS (RMTS32) based on skipping high frequency coefficients was adopted in order to reduce computational complexity of 32-point DST-7/DCT-8. Moreover, it was also agreed that the proposed residual coding change should be accompanied considering all types of zero-out (those of JVET-M0297 and JVET-M0251/M0257).

During in-between VVC working draft editing period, all the related proponents provided their spec text where contexts of last non-zero coefficient position coding is selected based on original TU size, which is aligned with the current VTM4.0 SW. But, it was questioned if this context selection should be performed based on reduced TU size, which was commented to be more consistent. Therefore, it was proposed that it could be decided at the next meeting based on experimental results which TU size should be involved during the context selection.

In this contribution, the experimental results based on reduced TU size are presented. It is reportedly shown that luma BD-rate/encoding time/decoding time changes of all configurations are 0.10%/101%/103% (AI), 0.XX%/XXX%/XXX% (RA), and 0.XX%/XXX%/XXX% (LD), respectively, but BD-rate loss for high resolution sequences is substantial (up-to 0.24%).

Proposed Method

  - Context selection based on reduced TU size (VVC Working Draft v6):

In the latest VVC working draft document [1], TU size is reduced by changing two variables, log2TbWidth and log2TbHeight, as in Table 1 for residual_coding syntax parsing.

Table 1  The excerpted syntax table for residual_coding illustrating TU size change

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |  |
| --- | --- | --- |
| if( ( tu_mts_idx[ x0 ][ y0 ] > 0 ¦ ¦ ( cu_sbt_flag && log2TbWidth < 6 && log2TbHeight < 6 ) ) && cIdx = = 0 && log2TbWidth > 4 ) |  |  |
| log2TbWidth = 4 |  |  |
| else |  |  |
| log2TbWidth = Min( log2TbWidth, 5 ) |  |  |
| if( tu_mts_idx[ x0 ][ y0 ] > 0 ¦ ¦ ( cu_sbt_flag && log2TbWidth < 6 && log2TbHeight < 6 ) ) && cIdx = = 0 && log2TbHeight > 4 ) |  |  |
| log2TbHeight = 4 |  |  |
| else |  |  |
| log2TbHeight = Min( log2TbHeight, 5 ) |  |  |
| if( log2TbWidth > 0 ) |  |  |
| last_sig_coeff_x_prefix | ae(v) |
| if( log2TbHeight > 0 ) |  |
| last_sig_coeff_y_prefix | ae(v) |
| if( last_sig_coeff_x_prefix > 3 ) |  |
| last_sig_coeff_x_suffix | ae(v) |
| if( last_sig_coeff_y_prefix > 3 ) |  |
| last_sig_coeff_y_suffix | ae(v) |
| …… |  |

The changed two variables affect context selection of last_sig_coeff_x_prefix and last sig_coeff_y_prefix in accordance with clause 9.5.4.2.4 (Table 2).

Table 2  The clause 9.5.4.2.4 in [1]

9.5.4.2.4 Derivation process of ctxInc for the syntax elements last_sig_coeff_x_prefix and last_sig_coeff_y_prefix Inputs to this process are the variable binIdx, the colour component index cIdx, the binary logarithm of the transform block width log2TbWidth and the transform block height log2TbHeight. Output of this process is the variable ctxInc. The variable log2TbSize is derived as follows: - If the syntax element to be parsed is last_sig_coeff_x_prefix, log2TbSize is set equal to log2TbWidth. - Otherwise (the syntax element to be parsed is last_sig_coeff_y_prefix), log2TbSize is set equal to log2TbHeight. The variables ctxOffset and ctxShift are derived as follows: - If cIdx is equal to 0, ctxOffset is set equal to 3 * ( log2TbSize − 2 ) + ( ( log2TbSize − 1 ) >> 2 ) and ctxShift is set equal to ( log2TbSize + 1 ) >> 2. - Otherwise (cIdx is greater than 0), ctxOffset is set equal to 21 and ctxShift is set equal to Clip3( 0, 2, 2log2TbSize >> 3 ). The variable ctxInc is derived as follows: ctxInc = ( binIdx >> ctxShift ) + ctxOffset (9-31)

The binarization of last_sig_coeff_x_prefix and last_sig_coeff_y_prefix is also adjusted considering reduced TU size as shown in Table 3.

Table 3  The TU binarization of last_sig_coeff_x_prefix and last_sig_coeff_y_prefix with modified TU size

| residual_coding( ) | last_sig_coeff_x_prefix | TR | cMax = ( log2TbWidth << 1 ) − 1, cRiceParam = 0 |
| --- | --- | --- | --- |
|  | last_sig_coeff_y_prefix | TR | cMax = ( log2TbHeight << 1 ) − 1, cRiceParam = 0 |
|  | …… | …… | …… |

  - Context selection based on original TU size (VTM 4.0):

In VTM 4.0, CABAC contexts for the two syntax elements, last_sig_coeff_x_prefix and last_sig_coeff_y_prefix, are designated based on original TU size, which can be described with slight modifications from [1] as presented in Table 4.

Table 4  The modifications for allowing context selection of last_sig_coeff_x_prefix and last_sig_coeff_y_coeff based on original TU size

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |  |
| --- | --- | --- |
| if( ( tu_mts_idx[ x0 ][ y0 ] > 0 ¦ ¦ ( cu_sbt_flag && log2TbWidth < 6 && log2TbHeight < 6 ) ) && cIdx = = 0 && log2TbWidth > 4 ) |  |  |
| log2ZoTbWidth = 4 |  |  |
| else |  |  |
| log2ZoTbWidth = Min( log2TbWidth, 5 ) |  |  |
| if( tu_mts_idx[ x0 ][ y0 ] > 0 ¦ ¦ ( cu_sbt_flag && log2TbWidth < 6 && log2TbHeight < 6 ) ) && cIdx = = 0 && log2TbHeight > 4 ) |  |  |
| log2ZoTbHeight = 4 |  |  |
| else |  |  |
| log2ZoTbHeight = Min( log2TbHeight, 5 ) |  |  |
| if( log2TbWidth > 0 ) |  |  |
| last_sig_coeff_x_prefix | ae(v) |
| if( log2TbHeight > 0 ) |  |
| last_sig_coeff_y_prefix | ae(v) |
| if( last_sig_coeff_x_prefix > 3 ) |  |
| last_sig_coeff_x_suffix | ae(v) |
| if( last_sig_coeff_y_prefix > 3 ) |  |
| last_sig_coeff_y_suffix | ae(v) |
| log2TbWidth = log2ZoTbWidth |  |
| log2TbHeight = log2ZoTbHeight |  |
| …… |  |

In Table [4], two new variables for indicating shortened width and height, log2ZoTbWidth and log2ZoTbHeight, are employed and TU size is finally updated with the two variables after parsing syntax elements regarding last non-zero coefficient position. The clause for context selection of last_sig_coeff_x_prefix and last_sig_coeff_y_prefix, i.e. 9.2.4.5.2 in Table 2, is the same as that of 1.1), but the difference is that log2TbWidth and log2TbHeight are not changed here so as to choose CABAC contexts based on original TU size.

In VTM 4.0, cMax values of last_sig_coeff_x_prefix and last_sig_coeff_y_prefix are decided according to reduced TU size as 1.1), which can be realized as in Table 5.

Table 5  The TU binarization adjustment of last_sig_coeff_x_prefix and last_sig_coeff_y_prefix in VTM 4.0 with newly employed variables, log2ZoTbWidth and log2ZoTbHeight

| residual_coding( ) | last_sig_coeff_x_prefix | TR | cMax = ( log2ZoTbWidth << 1 ) − 1, cRiceParam = 0 |
| --- | --- | --- | --- |
|  | last_sig_coeff_y_prefix | TR | cMax = ( log2ZoTbHeight << 1 ) − 1, cRiceParam = 0 |
|  | …… | …… | …… |

Also, the semantics part of last_sig_coeff_x_prefix and last_sig_coeff_y_prefix needs to be slightly modified with log2ZoTbWidth and log2ZoTbHeight accordingly, as presented in Table 6.

Table 6  The modified semantics part of last_sig_coeff_x and last_sig_coeff_y with log2ZoTbWidth and log2ZoTbHeight

- 7.4.7.11 Residual coding semantics …… last_sig_coeff_x_prefix specifies the prefix of the column position of the last significant coefficient in scanning order within a transform block. The values of last_sig_coeff_x_prefix shall be in the range of 0 to ( log2ZoTbWidth << 1 ) − 1, inclusive. When last_sig_coeff_x_prefix is not present, it is inferred to be 0. last_sig_coeff_y_prefix specifies the prefix of the row position of the last significant coefficient in scanning order within a transform block. The values of last_sig_coeff_y_prefix shall be in the range of 0 to ( log2ZoTbHeight << 1 ) − 1, inclusive. When last_sig_coeff_y_prefix is not present, it is inferred to be 0. ……

Experimental results

The context selection method based on reduced TU size introduced in 1.1), has been implemented based on VTM 4.0 reference software according to common test conditions defined in JVET-M1010 [5] and Core Experiment description in JVET-M1026 [6].

  - Results with inter MTS off  (CTC)

TBD

  - Results with inter MTS on

TBD

Conclusion

It is reportedly observed that Luma BD-rate is degraded by 0.10%, 0.XX%, and 0.XX% from the context selection based on the original TU size for AI, RA, and LD configurations, respectively. Furthermore, coding performance becomes worse for high resolution sequences (especially, Class A1/A2). Therefore, if the coding loss is thought to be more critical over TU size consistency, it is recommended that the current working draft text should be aligned with VTM 4.0 implementation.

Reference

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 4)”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-M1001, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019.

- M. Koo et al., “CE6-related: 32 point MTS based on skipping high frequency coefficients”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-M0297, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019.

- J. Choi et al., “Non-CE7: Last position coding for large block-size transforms”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-M0251, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019.

- M. Coban and M. Karczewicz, “CE7-related: coefficient scanning and last position coding for TUs of greater than 32 width or height”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-M0257, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019.

- F. Bossen, J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-M1010, 13th JVET meeting, Marrakech, MA, 9–18 Jan. 2019.

- X. Zhao and H. E. Egilmez, “Description of Core Experiment 6 (CE6): Transforms and transform signalling”, JVET-M1026, 13th JVET meeting, Marrakech, MA, 9–18 Jan. 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).