| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0200-v1 |
| --- | --- |

| Title: | Non-CE4: Simplification of inter MVP list generation for 4x4 block |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jingya Li Che-Wei Kuo Chong Soon Lim 202 Bedok South Ave 1 #02-11 Singapore 469332 | Tel: Email: | +65 6552 5423 jingya.li@sg.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution proposes to further reduce number of cycles for normal inter and IBC inter MVP candidate list construction process for 4x4 block by not adding HMVP candidate into candidate list and skipping look up table updating. It is reported that BD rate changes are -0.01% and 0.00% in RA and LDB configurations by applying the simplification to normal inter MVP candidate list generation for 4x4 block, and 0.**% and 0.**% in RA and LDB configurations by applying the simplification to IBC inter MVP candidate list generation for 4x4 block.

# Introduction

In current VTM4.0, a coding block as small as 4x4 has a tight budget in terms of number of cycles to avoid latency issues when generating motion vector candidate list. Such problem has been addressed for merge list generation by using MER and shared list as proposed in M0170. However for normal inter and IBC inter MVP list generation, since MER and shared list are not applied, number of cycles used for small blocks can still be further simplified. Examples of virtual cycle counts for normal inter and IBC inter MVP list generations are given in Figure 1 and Figure 2. Note that actual cycle cost may be different from the illustration, e.g., generating TMVP predictor may cost more than 3 cycles; spatial and temporal predictor may also not be generated in parallel.

Figure 1: Virtual cycle counts for normal inter MVP list generation

Figure 2: Virtual cycle counts for IBC inter MVP list generation

# Proposal

The proposed method simplifies normal inter and IBC inter MVP candidate list generation by not adding HMVP candidate and skipping look up table updating for 4x4 block. By using this method, number of cycles used for normal inter and IBC inter MVP candidate list generation are further reduced.

Figure 3: proposed simplification of normal inter MVP list generation for 4x4 blocks

Figure 4: proposed simplification of IBC inter MVP list generation for 4x4 blocks

# Experimental results

Tests were carried out by using the VTM4.0 reference software. The CTC (common test conditions) defined in [1] was followed.

Table 1: simulation results

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.02% | 0.02% | 99% | #DIV/0! |
| Class A2 | -0.03% | -0.08% | -0.03% | 100% | #DIV/0! |
| Class B | 0.00% | -0.09% | -0.05% | 103% | #DIV/0! |
| Class C | -0.01% | -0.04% | -0.05% | 100% | #DIV/0! |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.05% | -0.03% | 101% | #DIV/0! |
| Class D | 0.00% | -0.12% | 0.00% | 100% | #DIV/0! |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.09% | -0.16% | 100% | #DIV/0! |
| Class C | -0.01% | 0.05% | 0.04% | 100% | #DIV/0! |
| Class E | 0.03% | -0.06% | -0.14% | 99% | #DIV/0! |
| Overall | 0.00% | -0.03% | -0.09% | 100% | #DIV/0! |
| Class D | -0.07% | -0.30% | -0.35% | 100% | #DIV/0! |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

# Discussions

This contribution proposed a method of further simplification of normal inter and IBC inter MVP list to reduce number of cycles for 4x4 blocks by not adding HMVP candidates and skipping HMVP look up table updating. Simulation results show that there is no coding performance impact for inter MVP list generation, and 0.**%, 0.**% impact for RA and LDB configurations for classF. It is suggested to adopt proposed method to the next VVC working draft.

# References

[1] J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-M1010, the 13th JVET meeting, Jan. 2019, Marrakesh, MA

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Text

#### Updating process for the history-based motion vector predictor candidate list

Inputs to this process are:

- luma motion vectors in 1/16 fractional-sample accuracy mvL0 and mvL1,

- reference indices refIdxL0 and refIdxL1,

- prediction list utilization flags predFlagL0 and predFlagL1,

- bi-prediction weight index gbiIdx.

The MVP candidate hMvpCand consists of the luma motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, the prediction list utilization flags predFlagL0 and predFlagL1, and the bi-prediction weight index gbiIdx.

The candidate list HmvpCandList is modified using the candidate hMvpCand by the following ordered steps:

- The variable identicalCandExist is set equal to FALSE and the variable removeIdx is set equal to 0.

- When NumHmvpCand is greater than 0, for each index hMvpIdx with hMvpIdx = 0..NumHmvpCand − 1, the following steps apply until identicalCandExist is equal to TRUE:

  - When hMvpCand is equal to HmvpCandList[ hMvpIdx ], identicalCandExist is set equal to TRUE and removeIdx is set equal to hMvpIdx.

- The candidate list HmvpCandList is updated as follows:

  - If identicalCandExist is equal to TRUE or NumHmvpCand is equal to MaxNumMergeCand − 1, the following applies:

  - For each index i with i = ( removeIdx + 1 )..( NumHmvpCand − 1 ), HmvpCandList[ i − 1] is set equal to HmvpCandList[ i ].

  - HmvpCandList[ NumHmvpCand − 1 ] is set equal to mvCand.

  - Otherwise (identicalCandExist is equal to FALSE and NumHmvpCand is less than MaxNumMergeCand − 1), the following applies:

  - HmvpCandList[ NumHmvpCand++ ] is set equal to mvCand.

#### Updating process for the history-based motion vector predictor candidate list is not invoked when cbWidth* cbHeight<=16

#### Updating process for the history-based motion vector predictor candidate list

Inputs to this process are:

- luma motion vector mvL in 1/16 fractional-sample accuracy.

The candidate list HmvpIbcCandList is modified by the following ordered steps:

- The variable identicalCandExist is set equal to FALSE and the variable removeIdx is set equal to 0.

- When NumHmvpIbcCand is greater than 0, for each index hMvpIdx with hMvpIdx = 0..NumHmvpIbcCand − 1, the following steps apply until identicalCandExist is equal to TRUE:

  - When hMvpCand is equal to HmvpIbcCandList[ hMvpIdx ], identicalCandExist is set equal to TRUE and removeIdx is set equal to hMvpIdx.

- The candidate list HmvpIbcCandList is updated as follows:

  - If identicalCandExist is equal to TRUE or NumHmvpIbcCand  is equal to MaxNumMergeCand − 1, the following applies:

  - For each index i with i = ( removeIdx + 1 )..( NumHmvpIbcCand  − 1 ), HmvpIbcCandList[ i − 1] is set equal to HmvpIbcCandList [ i ].

  - HmvpIbcCandList[ NumHmvpIbcCand − 1 ] is set equal to mvCand.

  - Otherwise (identicalCandExist is equal to FALSE and NumHmvpIbcCand  is less than MaxNumMergeCand − 1), the following applies:

  - HmvpIbcCandList[ NumHmvpIbcCand ++ ] is set equal to mvCand.

#### Updating process for the ibc history-based motion vector predictor candidate list is not invoked when cbWidth* cbHeight<=16

#### Derivation process for motion vector predictor candidate list

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

Output of this process is motion vector predictor candidate list mvpList in 1/16 fractional-sample accuracy.

The motion vector predictor candidate list mvpList is derived in the following ordered steps:

- The derivation process for spatial motion vector predictor candidates from neighbouring coding unit partitions as specified in clause 8.6.2.8 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth and the luma coding block height cbHeight as inputs, and the availability flags availableFlagN and the motion vectors mvN, with N being replaced by A or B, as outputs.

- The rounding process for motion vectors as specified in clause 8.5.2.14 is invoked with mvX set equal to mvN, with N being replaced by A or B, rightShift set equal to MvShift + 2, and leftShift set equal to MvShift + 2 as inputs and the rounded mvN, with N being replaced by A or B, as outputs.

- The motion vector predictor candidate list, mvpList, is constructed as follows:

numCurrMvpCand = 0
if( availableFlagA ) {
	mvpList[ numCurrMvpCand++ ] = mvA
	if( availableFlagB  &&  ( mvA  !=  mvB ) )
		mvpList[ numCurrMvpCand++ ] = mvB		(8-989)
} else if( availableFlagB )
	mvpList[ numCurrMvpCand++ ] = mvB

- When numCurrMvpCand is less than 2 and NumHmvpIbcCand is greater than 0 and cbWidth* cbHeight<=16, the following applies for i= 1..Min( 4, NumHmvpIbcCand ) until numCurrMvpCand is equal to 2:

- The rounding process for motion vectors as specified in clause 8.5.2.14 is invoked with mvX set equal to the motion vector of the candidate HmvpIbcCandList[ i − 1 ], rightShift set equal to MvShift + 2, and leftShift set equal to MvShift + 2 as inputs and the rounded motion vector of the candidate HmvpIbcCandList[ i − 1 ] as output is assigned to mvpList[ numCurrMvpCand++ ].

- When numCurrMvpCand is less than 2, the following applies for until numCurrMvpCand is equal to 2:

- mvpList[ numCurrMvpCand ][ 0 ] is set equal to 0.

- mvpList[ numCurrMvpCand ][ 1 ] is set equal to 0.

- numCurrMvpCand is increased by 1.