| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0220 |
| --- | --- |

| Title: | AHG16: Simplification of Reshaper Implementation |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Taoran Lu, Fangjun Pu, Peng Yin, Sean McCarthy, Walt Husak, Tao Chen 432 Lakeside Drive, Sunnyvale, CA, 94085 US | Tel: Email: | {pyin, tlu, fpu}@dolby.com |  |
| Source: | Dolby Laboratories, Inc. |  |  |  |

_____________________________

# Abstract

This contribution presents four normative changes to simplify VTM4 LMCS implementation:

- reduction of the pipeline delay for computing average luma for chroma residue scaling;

- reduction of the size of the local buffer needed to store chroma residue samples;

- unification of the fixed-point precision used in luma mapping and chroma residue scaling;

- unification of the method of calculating chroma residue scale with the method of calculating the luma inverse scale, and removal of the pre-computed LUT.

The combined simplification test results for SDR CTC over the VTM4.0 anchor are {BDRate for Y, U, V}:

- AI:     0.00%,  0.00%,  0.00%;

- RA:    0.01%, -0.05%,  -0.01%;

- LDB:  0.01%, -0.09%, -0.42%;

- LDP: -0.01%, -0.19%, -0.07%.

The encoding and decoding times are found to be similar to VTM4.0.

This contribution also presents some non-CTC results for three non-normative topics:

- results for a VTM non-normative modification to enable support of InternalBitDepth other than 10;

- results for a test for interactions between LMCS and local QP adjustment. The test results indicate that LMCS and local QP adjustment are independent and the respective coding gains are additive. Test results for RA Class B/C/D/F are {average BDRate for Y,U,V}:

  - local QP only:         -0.48%, -0.91%, -0.96%;

  - LMCS only:         -1.48%,   2.85%,  2.85%;

  - both:                        -1.94%,  1.76%,  1.85%;

  - diff (c – (a + b)):      0.02%, -0.18%, -0.04%;

- test results for SCC TGM test sequences, (average BDRate for Y,U,V)

  - AI:     -1.36%,  0.00%,  0.23%;

  - RA:    -1.44%, -0.15%,  0.09%;

  - LDB:  -1.67%, -0.10%, -0.19%;

  - LDP:  -2.12%, -0.40%, -041%.

# Introduction

Luma mapping with chroma scaling (LMCS) [1][2][3] was added in VTM 4.0 as a new processing block before the loop filters. It consists of two main components: 1) in-loop mapping of the luma component based on adaptive piecewise linear models; 2) luma-dependent chroma residual scaling for the chroma components.

Figure 1 depicts LMCS architecture from the decoder’s perspective. The light-blue shaded blocks in Figure 1 indicate where the processing is applied in the mapped domain; and these include the inverse quantization, inverse transform, luma intra prediction and adding of the luma prediction together with the luma residual. The unshaded blocks in Figure 1 indicate where the processing is applied in the original (i.e., non-mapped) domain; and these include loop filters such as deblocking, ALF, and SAO, motion compensated prediction, chroma intra prediction, adding of the chroma prediction together with the chroma residual, and the storage of decoded pictures in DPB. The light-yellow shaded blocks in Figure 1 are the new LMCS functional blocks, including forward and inverse mapping of the luma signal and a luma-dependent chroma scaling process.

As adopted in VTM4, chroma residual scaling depends on the average value of the corresponding luma prediction block (for both intra- and inter-coded blocks). Denote avgY' as the average of the luma prediction block. The value of CScaleInv is computed in the following steps:

- Find the index YIdx of the piecewise linear model to which avgY' belongs based on the InvMap function.

- CScaleInv  = cScaleInv[YIdx], where cScaleInv[] is a pre-computed 16-piece LUT.

If the current block is coded as intra, CIIP, or intra block copy (IBC, a.k.a. current picture referencing or CPR) modes, avgY' is computed as the average of the intra-, CIIP-, or IBC- predicted luma values; otherwise, avgY' is computed as the average of the forward mapped inter predicted luma values (Y'pred in Figure 1). Unlike luma mapping, which is performed on the sample basis, CScaleInv is a constant value for the entire chroma block. With CScaleInv , chroma residual scaling is applied as follows:

Encoder side: CResScale=CRes*CScale=CRes/CScaleInv

Decoder side: CRes=CResScale/CScale=CResScale*CScaleInv

In VTM4, chroma residual scaling is disabled for dual-tree intra and disabled for chroma TU size <= 4.

Figure 1 – Luma mapping with chroma scaling (LMCS) architecture

# Proposed normative simplifications

This contribution includes four normative simplifications for LMCS implementation.

## Simplification 1: Reduction of pipeline delay in chroma residue scaling

As adopted in VTM4, when chroma residue scaling is applied, the chroma residue scale factor for each chroma TU depends on the average value of the corresponding luma prediction block (avgY'). The operation to compute avgY' thus requires access of all the samples of the corresponding luma prediction block, which introduces pipeline delay for the reconstruction process of chroma components. For example, for a large TU such as 64x64, decoding pipeline would need to finish the 64x64 luma prediction first and then process chroma residue scaling. Consequently, luma and chroma reconstruction cannot be performed timely in parallel because chroma processing would need to wait for the average of the whole luma block.

As a simplification, we propose to use the first (upper-left) sample value of the corresponding luma prediction block to serve as the average value of the block. Hence, the chroma residue scale factor for the chroma TU can be computed immediately after the first corresponding luma prediction sample is available.

Simulation results over VTM4.0 CTC anchors (Table 1) show that the change in overall BD-rate is very small.

Table 1 results of simplification 1

|  | All Intra Main10 |  |  | Random access Main10 |  |  | Low delay B Main10 |  |  | Low delay P Main10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Class A1 | 0.00% | 0.00% | 0.00% | 0.00% | -0.07% | 0.00% |  |  |  |  |  |  |
| Class A2 | 0.00% | 0.00% | 0.00% | -0.03% | -0.05% | -0.03% |  |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 0.01% | -0.06% | -0.02% | 0.00% | -0.07% | 0.08% | 0.00% | -0.14% | 0.16% |
| Class C | 0.00% | 0.00% | 0.00% | 0.02% | 0.02% | -0.18% | -0.03% | 0.46% | -0.07% | 0.01% | -0.54% | -0.30% |
| Class E | 0.00% | 0.00% | 0.00% |  |  |  | -0.05% | 0.01% | -0.63% | 0.00% | -0.26% | 0.00% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | -0.04% | -0.06% | -0.02% | 0.13% | -0.15% | 0.00% | -0.30% | -0.03% |
| Class D | 0.00% | 0.00% | 0.00% | 0.01% | -0.03% | 0.15% | -0.03% | -0.41% | -0.71% | 0.05% | -0.20% | -0.02% |
| Class F | 0.00% | 0.00% | 0.00% | -0.01% | 0.05% | 0.00% | -0.16% | -0.36% | 0.00% | 0.01% | 0.15% | 0.18% |

## Simplification 2: Reduction of hardware local buffer size

It is reported by hardware implementers that, based on VTM4 design, chroma residue needs to be kept in high-precision before it is scaled (e.g., from 11-bit storage to 16-bit storage for 10-bit video). This can cost approximately 2.5 Kbytes additional local buffer size (double buffer, i.e. 5-bit *32*32*2*2/8 = 2560 bytes) for a 64x64 decoder pipeline.  It is desirable to minimize the expense for memory.

We propose to add a normative clipping in the chroma residue scaling process. For each chroma residue sample to be scaled, clip it within the range of the valid residue range given the bitdepth of the chroma components before chroma residue scaling:

CRes= Clip3(- (1<< BitDepthC), 1<< BitDepthC - 1, CRes)

For example, for 10-bit coding, the chroma residues are clipped in range [-1024 1023] before applying chroma residue scaling. The normative clipping reduces the local buffer size.

Simulation results over VTM4.0 CTC anchors (Table 2) show that there is no measurable change in BD-rate.

Table 2 results of simplification 2

|  | All Intra Main10 |  |  | Random access Main10 |  |  | Low delay B Main10 |  |  | Low delay P Main10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Class A1 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |  |  |  |  |  |  |
| Class A2 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |  |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class E | 0.00% | 0.00% | 0.00% |  |  |  | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |

## Simplification 3: Unification of fixed-point precision

In VTM4, luma mapping and chroma residue scaling are implemented with fixed-point integer operations. For chroma residue scaling, the precision (shiftC) used to represent the fractional bits is 11. For luma mapping, the precision (shiftY) used to represent the fractional bits is 14. We propose to unify the precision to be 11 for both luma mapping and chroma residue scaling.

Simulation results over VTM4.0 CTC anchors (Table 3) show that the change in overall BD-rate is very small.

Table 3 results of simplification 3

|  | All Intra Main10 |  |  | Random access Main10 |  |  | Low delay B Main10 |  |  | Low delay P Main10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Class A1 | 0.00% | 0.00% | -0.01% | 0.00% | -0.03% | -0.03% |  |  |  |  |  |  |
| Class A2 | 0.00% | 0.00% | 0.00% | -0.02% | -0.06% | 0.00% |  |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | -0.01% | -0.04% | -0.04% | -0.01% | -0.20% | -0.26% | -0.02% | -0.18% | 0.15% |
| Class C | 0.00% | 0.00% | 0.00% | 0.02% | -0.04% | -0.04% | -0.01% | 0.14% | 0.24% | 0.00% | -0.43% | -0.35% |
| Class E | 0.00% | 0.01% | 0.00% |  |  |  | -0.04% | -0.15% | -0.31% | 0.05% | -0.24% | 0.07% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | -0.04% | -0.03% | -0.02% | -0.07% | -0.10% | 0.01% | -0.28% | -0.04% |
| Class D | 0.00% | 0.00% | 0.00% | -0.02% | -0.09% | -0.10% | -0.09% | 0.65% | -0.48% | 0.05% | -0.09% | 0.17% |
| Class F | 0.00% | 0.00% | -0.01% | -0.02% | 0.00% | -0.02% | -0.06% | -0.03% | 0.24% | 0.04% | -0.06% | 0.08% |

## Simplification 4: Unification of chroma residue scaling factor and luma inverse scaling factor

In VTM4, the value of chroma residue scale is calculated using a pre-computed 64-entry codewords-to-scale mapping table (chromaResidualScaleLut), which needs to be stored in the decoder to compute the chroma scale:

chromaResidualScaleLut[64] = {16384, 16384, 16384, 16384, 16384, 16384, 16384, 8192, 8192, 8192, 8192, 5461, 5461, 5461, 5461, 4096, 4096, 4096, 4096, 3277, 3277, 3277, 3277, 2731, 2731, 2731, 2731, 2341, 2341, 2341, 2048, 2048, 2048, 1820, 1820, 1820, 1638, 1638, 1638, 1638, 1489, 1489, 1489, 1489, 1365, 1365, 1365, 1365, 1260, 1260, 1260, 1260, 1170, 1170, 1170, 1170, 1092, 1092, 1092, 1092, 1024, 1024, 1024, 1024};

The chroma scale ChromaScaleCoef[ i ] for i-th luma PWL piece is calculated as:

binCW = BitDepthY > 10  ?  ( rspCW[ i ] >> (BitDepthY – 10)) : BitDepthY < 10 ? ( rspCW[ i ] << ( 10 – BitDepthY) ): rspCW[ i ]

ChromaScaleCoef[ i ] = chromaResidualScaleLut[ Clip3(1, 64, binCW >> 1 ) – 1 ]

where rspCW[ i ] the mapped codewords in the i-th luma piece of the PWL luma mapping model.

We propose to remove the pre-computed table chromaResidualScaleLut[] and calculate the chroma residue scale in the same way that inverse luma scale is calculated (InvScaleCoeff[ i ] = OrgCW * (1 << shiftY) / binCW [ i ]):

ChromaScaleCoef[ i ] = OrgCW * (1 << shiftC) / binCW [ i ]

where OrgCW is the original codewords in each equal piece of the PWL luma mapping model (for 10-bit, OrgCW=1024/16 = 64), and shiftC = 11.

Simulation results over VTM4.0 CTC anchors (Table 4) show that the overall change in BD-rate change is very small.

Table 4 results of simplification 4

|  | All Intra Main10 |  |  | Random access Main10 |  |  | Low delay B Main10 |  |  | Low delay P Main10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Class A1 | 0.00% | 0.00% | 0.00% | -0.01% | 0.34% | 0.24% |  |  |  |  |  |  |
| Class A2 | 0.00% | 0.00% | 0.00% | 0.03% | -0.49% | -0.46% |  |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 0.01% | -0.15% | 0.01% | 0.01% | -0.68% | -0.55% | -0.01% | -0.72% | -0.40% |
| Class C | 0.00% | 0.00% | 0.00% | -0.02% | 0.08% | -0.03% | -0.02% | -0.34% | -0.42% | 0.04% | -0.53% | -0.42% |
| Class E | 0.00% | 0.00% | 0.00% |  |  |  | -0.03% | 0.96% | 0.49% | -0.16% | 0.95% | 1.20% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | -0.06% | -0.05% | -0.01% | -0.16% | -0.24% | -0.03% | -0.24% | -0.01% |
| Class D | 0.00% | 0.00% | 0.00% | -0.02% | -0.31% | -0.29% | -0.01% | -0.43% | -0.80% | 0.08% | -0.53% | -0.85% |
| Class F | 0.00% | 0.00% | 0.00% | -0.03% | 0.21% | 0.19% | 0.04% | -0.06% | 0.11% | 0.01% | 0.10% | 0.03% |

When both simplification 3 (shiftY = shiftC = 11) and 4 are enabled, there is no need to calculate ChromaScaleCoef[ i ] for valid pieces. Instead, ChromaScaleCoef[ i ] is equal to the inverse luma scale of the i-th PWL luma mapping model: ChromaScaleCoef[ i ]  = InvScaleCoeff[ i ].

The results of combining simplification 3 and 4 is shown in Table 5. The change in BDrate over VTM4.0 is very small.

Table 5 results of combination of simplification 3 and 4 over VTM4.0

|  | All Intra Main10 |  |  | Random access Main10 |  |  | Low delay B Main10 |  |  | Low delay P Main10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Class A1 | 0.00% | 0.00% | -0.01% | -0.02% | 0.34% | 0.22% |  |  |  |  |  |  |
| Class A2 | 0.00% | 0.00% | 0.00% | 0.04% | -0.38% | -0.41% |  |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 0.01% | -0.18% | 0.02% | 0.00% | -0.66% | -0.48% | 0.02% | -0.68% | -0.50% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.07% | 0.04% | 0.00% | -0.07% | -0.34% | 0.04% | -0.52% | -0.64% |
| Class E | 0.00% | 0.01% | 0.00% |  |  |  | -0.03% | 1.28% | 0.39% | -0.15% | 1.04% | 1.37% |
| Overall | 0.00% | 0.00% | 0.00% | 0.01% | -0.05% | -0.02% | -0.01% | 0.02% | -0.22% | -0.02% | -0.20% | -0.08% |
| Class D | 0.00% | 0.00% | 0.00% | -0.04% | -0.27% | -0.19% | 0.01% | -1.56% | -1.35% | 0.06% | -0.75% | -0.87% |
| Class F | 0.00% | 0.00% | -0.01% | -0.04% | 0.25% | 0.17% | -0.05% | -0.04% | -0.04% | 0.08% | -0.02% | 0.03% |

## Results of combined simplifications

The combined tests results of all the four simplifications are shown in Table 6. The overall change in BD-rate is very small. The encoding/decoding timing is quite close to VTM4.0.

Table 6 results of all simplifications over VTM4.0

|  | All Intra Main10 |  |  | Random access Main10 |  |  | Low delay B Main10 |  |  | Low delay P Main10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Class A1 | 0.00% | 0.00% | -0.01% | 0.00% | 0.35% | 0.36% |  |  |  |  |  |  |
| Class A2 | 0.00% | 0.00% | 0.00% | 0.03% | -0.38% | -0.44% |  |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 0.01% | -0.17% | -0.06% | 0.04% | -0.64% | -0.92% | 0.02% | -0.72% | -0.21% |
| Class C | 0.00% | 0.00% | 0.00% | -0.02% | 0.03% | 0.12% | -0.02% | -0.20% | -0.40% | 0.03% | -0.56% | -0.64% |
| Class E | 0.00% | 0.01% | 0.00% |  |  |  | 0.02% | 0.96% | 0.36% | -0.10% | 1.17% | 0.92% |
| Overall | 0.00% | 0.00% | 0.00% | 0.01% | -0.05% | -0.01% | 0.01% | -0.09% | -0.42% | -0.01% | -0.19% | -0.07% |
| Class D | 0.00% | 0.00% | 0.00% | -0.01% | -0.38% | -0.13% | -0.05% | -0.93% | -0.94% | 0.03% | -1.17% | -0.65% |
| Class F | 0.00% | 0.00% | -0.01% | -0.04% | 0.22% | 0.20% | -0.03% | -0.20% | -0.05% | -0.01% | 0.09% | -0.04% |

Additional test results of the simplifications on HDR reshaping are presented in Table 7. The tests compare against HDR reshaping with VTM4.0 configured with lumaDQP disabled and luma reshaping enabled with signal type set to HDR-PQ. The results suggest that the proposed simplifications do not introduce significant changes to HDR coding performance.

Table 7 results of all simplifications over VTM4.0 HDR reshaping (LumaDQP=0, LumaReshape=1, SignalType=HDR-PQ)

|  | All Intra: BD-rate |  |  | Random Access: BD-rate |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | wPsnrY | DE100 | PSNRL100 | wPsnrY | DE100 | PSNRL100 |
| Average HDR-B | 0.00% | 0.01% | 0.01% | 0.06% | -0.18% | 0.06% |

## Specification changes

The specification changes of the normative simplifications are presented in attached specification document with tracked changes.

# Non-CTC non-normative topics

## LMCS for bitdepths other than 10b

JVET CTC uses InternalBitDepth = 10 for all simulations. During the software development of VTM4, LMCS implementation has been further improved to support non-CTC InternalBitDepth values other than 10. For the decoder, support is straightforward. The encoder requires some simple adjustments of the existing 10b algorithm, as described below.

For SDR, the encoder re-uses the 10b codeword allocation scheme by re-normalizing picture statistics to 10b, then shifts codewords for each piece to the targeted InternalBitDepth.

Table 8 shows RA results for InternalBitDepth=8.  Table 9 shows RA results for InternalBitDepth=12.  For both Table 8 and Table 9, the results are for LMCS=1 versus LMCS=0 in VTM4.0 (with the fix from VTM4.0.1 for 12b under RA).  For Table 8, the results are for 8-bit sequences only. For Table 9, the results are for all test sequences.

Table 8 BDRate of LMCS with InternalBitDepth=8 for 8bit content (Class C/D/F) under RA

|  | Random Access Main 8 |  |  |
| --- | --- | --- | --- |
|  | LMCS=1 Over LMCS=0 |  |  |
|  | Y | U | V |
| Class C | -0.60% | -0.04% | 0.03% |
| Class D | -0.50% | -1.81% | -1.25% |
| Class F | -0.79% | 0.65% | 0.78% |

Table 9 BDRate of LMCS with InternalBitDepth=12 under RA

|  | Random Access Main 12 |  |  |
| --- | --- | --- | --- |
|  | LMCS=1 Over LMCS=0 |  |  |
|  | Y | U | V |
| Class A1 | -1.08% | 3.69% | 3.50% |
| Class A2 | -1.37% | 3.30% | 3.40% |
| Class B | -1.70% | 1.59% | 1.28% |
| Class C | -1.51% | 4.51% | 4.41% |
| Class E |  |  |  |
| Overall | -1.46% | 3.13% | 2.98% |
| Class D | -1.45% | 4.02% | 4.51% |
| Class F | -1.78% | 2.58% | 2.53% |

For HDR in VTM4.0.1, bitdepths other than 10b are currently disabled because of the following two issues: 1) VTM4.0 for HDR LMCS curve derivation uses fixed point implementation for 10b (the source code does not work for the 12b case); and 2) lumaDQP table in the HDR anchor is implemented only for 10b case.

For LMCS, we propose to change the source code to floating point implementation to support any bitdepth. (Use of floating point also reduces the number of lines of the code.) The 10b floating point results are not measurably different from the fixed point results. For the lumaDQP table, we propose shifting the luma value range from 10b to the targeted bitdepth by reusing the same dQP value corresponding to 10b. The code fix is submitted in merge requests # 364 and #365.

Table 10 shows HDR RA 12b simulation results for comparison of LMCS=1 with the HDR anchor.

Table 10 BDRate of LMCS with InternalBitDepth=12 under RA for HDR PQ

|  | vtm4 reshape1 lqp0 12b Over vtm4 reshape0 lqp1 12b |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra: BD-rate |  |  | Random Access: BD-rate |  |  |
|  | DE100 | PSNRL100 | wPsnrY | DE100 | PSNRL100 | wPsnrY |
| Average HDR-B | 9.8% | -2.3% | -1.9% | 8.6% | -2.1% | -1.9% |

## LMCS and local QP adjustment

We tested the hypothesis that the coding performance of LMCS, which is a fine-granularity pixel-level process, is independent from local QP adjustment, which is a coarser block-level transform-domain process.

To test the hypothesis, we measured coding gains, relative to VTM4.0, of LMCS and of rate-distortion optimized local QP selection (e.g., testing +/1 values of QP) independently and in combination. If LMCS and local QP adjustment are independent, the coding gains should be additive.

Local QP adjustment is enabled by setting MaxDeltaQP=1 in VTM4.0 software. When enabling MaxDeltaQP=1, the encoding time is about 2.5x of VTM4.0 anchor. Therefore, we only run simulation for ClassB/C/D/F for RA case.

Table 11 shows results for the following test cases:

- “MaxDeltaQP=1 and LMCS=0” versus “MaxDeltaQP=0 and LMCS=0”;

- “MaxDeltaQP=0 and LMCS=1” versus “MaxDeltaQP=0 and LMCS=0”;

- “MaxDeltaQP=1 and LMCS=1” versus “MaxDeltaQP=0 and LMCS=0”.

Table 11 BDRate of LMCS and local QP adjustment individually and combined under RA (Class B/C/D/F)

|  | RA10 |  |  |  |
| --- | --- | --- | --- | --- |
|  | "MaxDeltaQP=1, LMCS=0" vs " MaxDeltaQP=0, LMCS=0" |  |  |  |
|  | Y | U | V | EncT |
| Class B | -0.52% | -1.22% | -1.26% | 249% |
| Class C | -0.39% | -0.89% | -1.15% | 251% |
| Class D | -0.33% | -0.71% | -0.58% | 265% |
| Class F | -0.68% | -0.80% | -0.85% | 244% |
| Average | -0.48% | -0.91% | -0.96% | 252% |
|  | RA10 |  |  |  |
|  | " MaxDeltaQP=0, LMCS=1" vs " MaxDeltaQP=0, LMCS=0" |  |  |  |
|  | Y | U | V | EncT |
| Class B | -1.61% | 1.03% | 0.77% | 107% |
| Class C | -1.44% | 3.86% | 4.01% | 104% |
| Class D | -1.20% | 3.98% | 4.18% | 110% |
| Class F | -1.67% | 2.53% | 2.45% | 103% |
| Average | -1.48% | 2.85% | 2.85% | 106% |
|  | RA10 |  |  |  |
|  | " MaxDeltaQP=1, LMCS=1" vs " MaxDeltaQP=0, LMCS=0" |  |  |  |
|  | Y | U | V | EncT |
| Class B | -2.11% | -0.40% | -0.60% | 269% |
| Class C | -1.80% | 2.74% | 2.89% | 252% |
| Class D | -1.50% | 3.01% | 3.39% | 287% |
| Class F | -2.36% | 1.70% | 1.71% | 256% |
| Average | -1.94% | 1.76% | 1.85% | 266% |

Table 12 Difference of BDRate of combined LMCS and local QP adjustment and sum of LMCS and local QP adjustment individually under RA (Class B/C/D/F)

|  | RA10 |  |  |
| --- | --- | --- | --- |
|  | Difference |  |  |
|  | Y | U | V |
| Class B | 0.02% | -0.21% | -0.11% |
| Class C | 0.03% | -0.23% | 0.03% |
| Class D | 0.03% | -0.26% | -0.21% |
| Class F | -0.01% | -0.03% | 0.11% |
| Average | 0.02% | -0.18% | -0.04% |

Note that the coding gains of “MaxDeltaQP=1 and LMCS=1” are very similar to the sum of the coding gains of “MaxDeltaQP=0 and LMCS=1” and “MaxDeltaQP=1 and LMCS=0”, as shown in Table 12.

The results shown in Tables 11 and 12 indicate that the coding gains from the LMCS and from local QP adjustment are independent and additive.

## SCC TGM

We tested VTM 4.0 LMCS for SCC TGM content.  The results are shown in Table 13.

Table 13 BDRate for SCC TGM under RA

|  | VTM4.0 over LMCS=0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SCC TGM | Y | U | V | EncT | DecT |
| AI 10 | -1.36% | 0.00% | -0.23% | 92% | 100% |
| RA 10 | -1.44% | 0.15% | 0.09% | 101% | 104% |
| LDB 10 | -1.67% | -0.10% | -0.19% | 101% | 105% |
| LDP 10 | -2.12% | -0.40% | -0.41% | 95% | 104% |

# Conclusion

This contribution proposes four normative simplifications of LMCS implementations. The performance of the simplified implementations are individually and collectively very close to the VTM4.0 anchor.

This contribution also describes: 1) a non-normative change to support bitdepths in addition to 10b in VTM; 2) informative results on the independent additive gains of LMCS and rate-distortion optimized local QP adjustment; and 3) LMCS performance results for SCC TGM test content.

It is suggested that all four normative simplifications of LMCS implementations be adopted.

# Acknowledgements

We would like to thank Technicolor for crosschecking results.

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5)”, JVET-M1001-v5, Marrakech, MA, 9-18 Jan. 2019.

- J. Chen, Y. Ye, S. Kim, “Algorithm description for Versatile Video Coding and Test Model 4 (VTM 4)”, JVET-M1002, Marrakech, MA, 9-18 Jan. 2019.

- T. Lu, F. Pu, P. Yin, S. McCarthy, W. Husak, T. Chen, “CE12: Mapping functions (test CE12-1 and CE12-2)”, JVET-M0427, Marrakech, MA, 9-18 Jan. 2019

- A. Segall, E. François, S. Iwamura, D. Rusanovskyy, “JVET common test conditions and evaluation procedures for HDR/WCG video”, JVET-L1011, Macao, CN, 3–12 Oct. 2018.

# Patent rights declaration(s)

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).