| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0249_v2 |
| --- | --- |

| Title: | Non-CE8: an alternative search area for IBC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jizheng Xu, Li Zhang, Kai Zhang, Hongbin Liu, Yue Wang 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email:; +1-619-436-7216 xujizheng@bytedance.com. |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This document presents an alternative search area setting for intra block copy when CTU size is 128x128. For class F/4:2:0 TGM, the BD-rate numbers of the proposed setting are -0.90%/-2.12% for AI, -0.74%/-1.74% for RA.

# Introduction

Intra block copy, i.e. IBC (or current picture referencing, i.e. CPR previously) coding mode, was adopted into VTM at the 12th JVET meeting. It was realized that IBC cannot directly use the reconstrued buffer. Instead, a dedicated buffer needs to be maintained. To restrict the extra on-chip memory for the buffer, IBC employs a reference area of one CTU. When CTU size is 128x128, currently the reference area is shown in Fig. 1.

Figure 1. Reference area defined in VTM-4.0, where a block denotes a 64x64 area.

# Proposed scheme

To further improve coding efficiency, we propose an new search area configuration, shown in Figure 2 and figure 3. For the 3rd and 4th 64x64 blocks in the current CTU, the reference is the same as the current design. For the 1st and 2nd 64x64 blocks in the current CTU, the reference area contains 64x64 reference reference blocks that are closer to the current 64x64 blocks, compared with the current design.

Figure 2. Proposed reference area when the 1st partitioning for the current CTU is not vertical BT.

Figure 3. Proposed reference area when the 1st partitioning is for the current CTU vertical BT.

# Experimental results

The proposed scheme is compared with VTM-4.0 with IBC on for all sequences. Table I shows the results.

Table I. The proposed vs. VTM-4.0, IBC is on for all sequences.

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 w/ IBC on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.03% | -0.03% | 121% | 100% |
| Class A2 | 0.00% | -0.03% | -0.01% | 122% | 100% |
| Class B | -0.03% | -0.06% | -0.02% | 122% | 102% |
| Class C | -0.01% | 0.05% | -0.04% | 121% | 108% |
| Class E | -0.16% | -0.27% | -0.01% | 124% | 102% |
| Overall | -0.04% | -0.06% | -0.02% | 122% | 103% |
| Class D | -0.03% | -0.02% | 0.08% | 118% | 95% |
| Class F | -0.90% | -0.89% | -0.86% | 105% | 105% |
| 4:2:0 TGM | -2.12% | -2.19% | -2.18% | 95% | 100% |
|  | Random Access |  |  |  |  |
|  | Over VTM-4.0 w/ IBC on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.07% | -0.06% | 106% | 100% |
| Class A2 | -0.03% | -0.10% | -0.01% | 107% | 100% |
| Class B | -0.01% | -0.17% | -0.42% | 108% | 101% |
| Class C | 0.00% | 0.12% | 0.11% | 112% | 107% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.06% | -0.13% | 108% | 102% |
| Class D | 0.00% | 0.04% | 0.06% | 112% | 105% |
| Class F | -0.74% | -0.78% | -0.76% | 105% | 102% |
| 4:2:0 TGM | -1.74% | -1.60% | -1.66% | 103% | 102% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-4.0 w/ IBC on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | 0.12% | -0.15% | 111% | #NUM! |
| Class C | -0.04% | -0.37% | 0.04% | 115% | 101% |
| Class E | -0.05% | 0.41% | 1.22% | 104% | #NUM! |
| Overall | -0.04% | 0.03% | 0.26% | 111% | #NUM! |
| Class D | -0.10% | 0.88% | -0.17% | 116% | 102% |
| Class F | -0.68% | -0.37% | -0.03% | 109% | #NUM! |
| 4:2:0 TGM | -1.45% | -1.37% | -1.33% | 104% | 103% |

# Text change

- When ( xCb + ( mvL[ 0 ] >> 4 ) ) >> CtbLog2SizeY is equal to ( xCb >> CtbLog2SizeY ) − 1, the derivation process for block availability as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( ( ( xCb + ( mvL[ 0 ] >> 4 ) + CtbSizeY ) >> ( CtbLog2SizeY − 1 ) ) << ( CtbLog2SizeY − 1 ), ( ( yCb + ( mvL[ 1 ] >> 4 ) ) >> ( CtbLog2SizeY − 1 ) ) << ( CtbLog2SizeY − 1 ) ) as inputs, and the output shall be equal to FALSE.

- When xCb >> (CtbLog2SizeY – 1) & 1 = 0 and yCb >> (CtbLog2SizeY – 1) & 1 = 0, the following conditions shall not be all true:

  - (xCb + (mvL[0] >> 4)) >> (CtbLog2SizeY – 1) & 1 is equal to 0.

  - (yCb + (mvL[1] >> 4) + cbHeight − 1) >> (CtbLog2SizeY – 1) & 1 is equal to 1.

- When xCb >> (CtbLog2SizeY – 1) & 1 = 1 and yCb >> (CtbLog2SizeY – 1) & 1 = 1, ( xCb + ( mvL[ 0 ] >> 4 ) ) >> CtbLog2SizeY shall be equal to xCb >> CtbLog2SizeY.

- When xCb >> (CtbLog2SizeY – 1) & 1 = 1 and yCb >> (CtbLog2SizeY – 1) & 1 = 0, one of the following conditions shall be true:

  - (xCb + (mvL[0] >> 4)) >> (CtbLog2SizeY – 1) & 1 is equal to 1.

  - ( xCb + ( mvL[ 0 ] >> 4 ) ) >> CtbLog2SizeY is equal to xCb >> CtbLog2SizeY.

- When xCb >> (CtbLog2SizeY – 1) & 1 = 0 and yCb >> (CtbLog2SizeY – 1) & 1 = 1, ( xCb + ( mvL[ 0 ] >> 4 ) ) >> CtbLog2SizeY shall be equal to xCb >> CtbLog2SizeY or the following conditions shall not be all true:

  - (xCb + (mvL[0] >> 4)) >> (CtbLog2SizeY – 1) & 1 is equal to 1.

  - (yCb + (mvL[1] >> 4)) >> (CtbLog2SizeY – 1) & 1 is equal to 1.

- When xCb >> (CtbLog2SizeY – 1) & 1 = 0 and yCb >> (CtbLog2SizeY – 1) & 1 = 1, one of the following conditions shall be true:

  - (xCb + (mvL[0] >> 4)) >> (CtbLog2SizeY – 1) & 1 is equal to 1.

  - ( xCb + ( mvL[ 0 ] >> 4 ) ) >> CtbLog2SizeY is equal to xCb >> CtbLog2SizeY.

- When xCb >> (CtbLog2SizeY – 1) & 1 = 1 and yCb >> (CtbLog2SizeY – 1) & 1 = 0, ( xCb + ( mvL[ 0 ] >> 4 ) ) >> CtbLog2SizeY shall be equal to xCb >> CtbLog2SizeY or the following conditions shall all be true:

  - (xCb + (mvL[0] >> 4)) >> (CtbLog2SizeY – 1) & 1 is equal to 1.

  - ( yCb + ( mvL[ 0 ] >> 4 ) + cbHeight − 1 ) >> (CtbLog2SizeY – 1) & 1 is equal to 0.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).