| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0253 |
| --- | --- |

| Title: | CE2-4.7: Motion compensation with padded samples for small coding units |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongbin Liu, Jaehong Chon, Hsiao Chiang Chuang, Li Zhang, Kai Zhang, Jizheng Xu, Yue Wang Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190 Beijing, China | Tel: Email: | +86 13911971613 liuhongbin.01@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In current VVC [1], bi-predicted 4*16, 16*4, 4*8 and 8*4 coding units (CU) and uni-predicted 4*4 CUs require much higher memory bandwidth than 8*8 bi-predicted CUs. This contribution proposes to fetch only some of the reference pixels and pad other reference pixels in the motion compensation process to reduce the worst-case memory bandwidth. It is asserted that the proposed method can reduce the worst-case memory bandwidth significantly with negligible impact on coding performance.

# Introduction

In current VVC, bi-predicted 4*16, 16*4, 4*8 and 8*4 coding unit (CU) and uni-predicted 4*4 CU require higher memory bandwidth than 8*8 bi-predicted CU. Tool provided in JVET-M0862 [2] is used to analyze the memory bandwidth required by these CUs, and the results are summarized in Table 1 for YCbCr 4:2:0 case. Both luma and chroma components are considered in the analysis.

Compared with HEVC wherein 8*8 bi-predicted CU requires the highest memory bandwidth, the worst-case memory bandwidth of VVC is increased by around 46% and 100% for 2*1 memory pattern and 16*2 memory pattern respectively.

Table 1: Memory bandwidth comparison of different block sizes.

| CUs | 2*1 memory pattern |  | 16*2 memory pattern |  |
| --- | --- | --- | --- | --- |
|  | Memory bandwidth | Bandwidth / bandwidth of 8*8 bi-predicted CUs | Memory bandwidth | Bandwidth / bandwidth of 8*8 bi-predicted CUs |
| 8*8 Bi | 20.19 | 100% | 36.00 | 100% |
| 4*8 Bi | 29.38 | 146% | 72.00 | 200% |
| 8*4 Bi | 29.38 | 146% | 54.00 | 150% |
| 4*16 Bi | 22.69 | 112% | 54.00 | 150% |
| 16*4 Bi | 22.69 | 112% | 36.00 | 100% |
| 4*4 Uni | 21.38 | 106% | 54.00 | 150% |

# Proposed method

To suppress the problem mentioned above, this contribution proposes to fetch only some of the reference pixels and pad other pixels in the motion compensation process [3]. The proposed method is only applied to luma samples, and the specific number of padded rows and columns are summarized in Table 2.

The padding method applies when both horizonal and vertical MV components of a block are fractional, no matter whether the block is uni-predicted or bi-predicted. With config #1, the worst-case memory bandwidth of VVC is reduced to 100% and 133% of that of the 8*8 bi-predicted CU, for 2*1 memory pattern and 16*2 memory pattern respectively, as shown in Table 3. With config #2, the worst-case memory bandwidth of VVC is reduced to 110% and 133% of that of the 8*8 bi-predicted CU, for 2*1 memory pattern and 16*2 memory pattern respectively, as shown in Table 4.

Table 2: padded luma rows or columns of different CUs.

| CUs | Config #1 |  | Config #2 |  |
| --- | --- | --- | --- | --- |
|  | Padded columns | Padded rows | Padded columns | Padded rows |
| 4*8 Bi | 2 | 4 | 2 | 3 |
| 8*4 Bi | 2 | 3 | 2 | 3 |
| 4*16 Bi | 2 | 3 | 2 | 3 |
| 16*4 Bi | 2 | 3 | 2 | 3 |
| 4*4 Uni | 3 | 4 | 2 | 3 |

Table 3: memory bandwidth comparison of different CUs with config #1.

| CUs | 2*1 memory pattern |  | 16*2 memory pattern |  |
| --- | --- | --- | --- | --- |
|  | Memory bandwidth | Bandwidth / bandwidth of 8*8 bi-predicted CUs | Memory bandwidth | Bandwidth / bandwidth of 8*8 bi-predicted CUs |
| 8*8 Bi | 20.19 | 100% | 36.00 | 100% |
| 4*8 Bi | 19.75 | 98% | 48.00 | 133% |
| 8*4 Bi | 20.13 | 100% | 42.00 | 117% |
| 4*16 Bi | 18.13 | 90% | 38.00 | 106% |
| 16*4 Bi | 17.38 | 86% | 28.00 | 78% |
| 4*4 Uni | 15.25 | 76% | 34.00 | 94% |

Table 4: memory bandwidth comparison of different CUs with config #2.

| CUs | 2*1 memory pattern |  | 16*2 memory pattern |  |
| --- | --- | --- | --- | --- |
|  | Memory bandwidth | Bandwidth / bandwidth of 8*8 bi-predicted CUs | Memory bandwidth | Bandwidth / bandwidth of 8*8 bi-predicted CUs |
| 8*8 Bi | 20.19 | 100% | 36.00 | 100% |
| 4*8 Bi | 22.25 | 110% | 48.00 | 133% |
| 8*4 Bi | 21.75 | 108% | 42.00 | 117% |
| 4*16 Bi | 18.13 | 90% | 38.00 | 106% |
| 16*4 Bi | 17.38 | 86% | 28.00 | 78% |
| 4*4 Uni | 15.25 | 76% | 34.00 | 94% |

# Experimental results

The proposed method was implemented on top of reference software VTM-4.0 [4], and simulations were performed following the common test conditions [5]. The results are shown in Table 5 and Table 6. As can be seen, proposed method has negligible impact on compression performance.

Table 5: Coding performance of config #1.

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.04% | 0.01% | 99% | 100% |  |  |  |  |  |
| Class A2 | -0.01% | -0.01% | -0.05% | 99% | 101% |  |  |  |  |  |
| Class B | 0.02% | -0.07% | -0.07% | 98% | 100% | 0.01% | -0.10% | -0.21% | 100% | 100% |
| Class C | 0.04% | -0.04% | -0.17% | 97% | 97% | -0.02% | 0.20% | 0.18% | 99% | 98% |
| Class E |  |  |  |  |  | 0.01% | -0.30% | 0.07% | 100% | 98% |
| Overall | 0.01% | -0.04% | -0.08% | 98% | 99% | 0.00% | -0.05% | -0.01% | 99% | 99% |
| Class D | 0.03% | 0.04% | -0.02% | 96% | 97% | 0.00% | 0.20% | -1.00% | 100% | 99% |
| Class F | 0.00% | 0.01% | -0.02% | 98% | 97% | -0.03% | -0.34% | -0.12% | 98% | 99% |

Table 6: Coding performance of config #2.

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.04% | 0.04% | 99% | 100% |  |  |  |  |  |
| Class A2 | -0.01% | -0.02% | 0.06% | 100% | 101% |  |  |  |  |  |
| Class B | 0.01% | -0.10% | -0.10% | 98% | 100% | -0.01% | -0.04% | 0.17% | 101% | 102% |
| Class C | 0.02% | -0.02% | -0.22% | 97% | 97% | 0.00% | 0.00% | -0.08% | 98% | 100% |
| Class E |  |  |  |  |  | -0.03% | 0.37% | -0.39% | 100% | 97% |
| Overall | 0.00% | -0.05% | -0.07% | 99% | 99% | -0.01% | 0.07% | -0.05% | 99% | 100% |
| Class D | -0.03% | -0.03% | 0.14% | 97% | 97% | -0.02% | 0.16% | -0.63% | 99% | 98% |
| Class F | 0.01% | 0.01% | -0.07% | 98% | 98% | -0.09% | -0.20% | 0.00% | 98% | 99% |

# Conclusions

This contribution proposes a method to reduce the memory bandwidth required by small CUs. With proposed method, the worst-case memory bandwidth of VVC reduced dramatically. Meanwhile, there is negligible impact on compression performance. It is recommended to adopt it into VVC.

# References

- “Versatile Video Coding (Draft 4)”, B. Bross, J. Chen, S. Liu, Doc. JVET-M1001, Jan. 2019.

- “BoG report on CE2 related contributions”, Y. He, Doc. JVET-M0862, Jan. 2019.

- “AHG16: Motion compensation with padded samples for small coding units”, H. Liu, J. Chon, H.-C. Chuang, L. Zhang, K. Zhang, J. Xu, Doc. JVET-M0248, Jan. 2019.

- VTM-4.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-4.0

- “JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Suehring, Doc. JVET-M1010, Jan. 2019.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).