| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0253_r1 |
| --- | --- |

| Title: | CE2-4.7: Motion compensation with padded samples for small coding units |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongbin Liu, Jaehong Chon, Hsiao Chiang Chuang, Li Zhang, Kai Zhang, Jizheng Xu, Yue Wang Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190 Beijing, China | Tel: Email: | +86 13911971613 liuhongbin.01@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In current VVC [1], bi-predicted 4*16, 16*4, 4*8 and 8*4 coding units (CU) and uni-predicted 4*4 CUs require much higher memory bandwidth than 8*8 bi-predicted CUs. This contribution proposes to fetch only some of the reference pixels and pad other reference pixels in the motion compensation process to reduce the worst-case memory bandwidth. It is asserted that the proposed method can reduce the worst-case memory bandwidth significantly with negligible impact on coding performance.

# Introduction

In current VVC, bi-predicted 4*16, 16*4, 4*8 and 8*4 coding unit (CU) and uni-predicted 4*4 CU require higher memory bandwidth than 8*8 bi-predicted CU. Tool provided in JVET-M0862 [2] is used to analyze the memory bandwidth required by these CUs, and the results are summarized in Table 1 for YCbCr 4:2:0 case. Both luma and chroma components are considered in the analysis.

Compared with HEVC wherein 8*8 bi-predicted CU requires the highest memory bandwidth, the worst-case memory bandwidth of VVC is increased by around 46% and 100% for 2*1 memory pattern and 16*2 memory pattern respectively.

Table 1: Memory bandwidth comparison of different block sizes.

| CUs | 2*1 memory pattern |  | 16*2 memory pattern |  |
| --- | --- | --- | --- | --- |
|  | Memory bandwidth | Bandwidth / bandwidth of 8*8 bi-predicted CUs | Memory bandwidth | Bandwidth / bandwidth of 8*8 bi-predicted CUs |
| 8*8 Bi | 20.19 | 100% | 36.00 | 100% |
| 4*8 Bi | 29.38 | 146% | 72.00 | 200% |
| 8*4 Bi | 29.38 | 146% | 54.00 | 150% |
| 4*16 Bi | 22.69 | 112% | 54.00 | 150% |
| 16*4 Bi | 22.69 | 112% | 36.00 | 100% |
| 4*4 Uni | 21.38 | 106% | 54.00 | 150% |

# Proposed method

To suppress the problem mentioned above, this contribution proposes to fetch only some of the reference pixels and pad other pixels in the motion compensation process [3]. The proposed method is only applied to luma samples, and the specific number of padded rows and columns are summarized in Table 2.

The padding method applies when both horizonal and vertical MV components of a block are fractional, no matter whether the block is uni-predicted or bi-predicted. With config #1, the worst-case memory bandwidth of VVC is reduced to 100% and 133% of that of the 8*8 bi-predicted CU, for 2*1 memory pattern and 16*2 memory pattern respectively, as shown in Table 3. With config #2, the worst-case memory bandwidth of VVC is reduced to 110% and 133% of that of the 8*8 bi-predicted CU, for 2*1 memory pattern and 16*2 memory pattern respectively, as shown in Table 4.

Table 2: padded luma rows or columns of different CUs.

| CUs | Config #1 |  | Config #2 |  |
| --- | --- | --- | --- | --- |
|  | Padded columns | Padded rows | Padded columns | Padded rows |
| 4*8 Bi | 2 | 4 | 2 | 3 |
| 8*4 Bi | 2 | 3 | 2 | 3 |
| 4*16 Bi | 2 | 3 | 2 | 3 |
| 16*4 Bi | 2 | 3 | 2 | 3 |
| 4*4 Uni | 3 | 4 | 2 | 3 |

Table 3: memory bandwidth comparison of different CUs with config #1.

| CUs | 2*1 memory pattern |  | 16*2 memory pattern |  |
| --- | --- | --- | --- | --- |
|  | Memory bandwidth | Bandwidth / bandwidth of 8*8 bi-predicted CUs | Memory bandwidth | Bandwidth / bandwidth of 8*8 bi-predicted CUs |
| 8*8 Bi | 20.19 | 100% | 36.00 | 100% |
| 4*8 Bi | 19.75 | 98% | 48.00 | 133% |
| 8*4 Bi | 20.13 | 100% | 42.00 | 117% |
| 4*16 Bi | 18.13 | 90% | 38.00 | 106% |
| 16*4 Bi | 17.38 | 86% | 28.00 | 78% |
| 4*4 Uni | 15.25 | 76% | 34.00 | 94% |

Table 4: memory bandwidth comparison of different CUs with config #2.

| CUs | 2*1 memory pattern |  | 16*2 memory pattern |  |
| --- | --- | --- | --- | --- |
|  | Memory bandwidth | Bandwidth / bandwidth of 8*8 bi-predicted CUs | Memory bandwidth | Bandwidth / bandwidth of 8*8 bi-predicted CUs |
| 8*8 Bi | 20.19 | 100% | 36.00 | 100% |
| 4*8 Bi | 22.25 | 110% | 48.00 | 133% |
| 8*4 Bi | 21.75 | 108% | 42.00 | 117% |
| 4*16 Bi | 18.13 | 90% | 38.00 | 106% |
| 16*4 Bi | 17.38 | 86% | 28.00 | 78% |
| 4*4 Uni | 15.25 | 76% | 34.00 | 94% |

# Working draft

The working draft of config #1 is described as follows, which is based on JVET-M1001-v6 [1]. The newly added parts are highlighted in yellow, and the deleted part are highlighted in red strikethrough.

#### Fractional sample interpolation process

##### 8.5.7.3.1	General

Inputs to this process are:

- a luma location ( xSb, ySb ) specifying the top-left sample of the current coding subblock relative to the top-left luma sample of the current picture,

- a variable sbWidth specifying the width of the current coding subblock,

- a variable sbHeight specifying the height of the current coding subblock,

- a motion vector offset mvOffset,

- a refined motion vector refMvLX,

- the selected reference picture sample array refPicLX,

- the bidirectional optical flow flag bdofFlag,

- a variable cIdx specifying the colour component index of the current block.

Outputs of this process are:

- an (sbWidth +bdofOffset)x(sbHeight +bdofOffset) array predSamplesLX of prediction sample values.

The bidirectional optical flow boundary offset bdofOffset is derived as follows:

bdofOffset = bdofFlag  ?  2  :  0		(8-811)

- If cIdx is equal to 0, the following applies:

  - Let ( xIntL, yIntL ) be a luma location given in full-sample units and ( xFracL, yFracL ) be an offset given in 1/16-sample units. These variables are used only in this clause for specifying fractional-sample locations inside the reference sample arrays refPicLX.

  - For each luma sample location ( xL = 0..sbWidth − 1 +bdofOffset, yL = 0..sbHeight − 1 +bdofOffset ) inside the prediction luma sample array predSamplesLX, the corresponding prediction luma sample value predSamplesLX[ xL ][ yL ] is derived as follows:

- The variables xIntL, yIntL, xFracL and yFracL are derived as follows:

xIntL = xSb + ( refMvLX[ 0 ]  >>  4 ) + xL		(8-812)

yIntL = ySb + ( refMvLX[ 1 ]  >>  4 ) + yL		(8-813)

xFracL = refMvLX[ 0 ] & 15		(8-814)

yFracL = refMvLX[ 1 ] & 15		(8-815)

  - If bdofFlag is equal to TRUE and one or more of the following conditions are true, the prediction luma sample value predSamplesLX[ xL ][ yL ] is derived by invoking the luma integer sample fetching process as specified in clause 8.5.7.3.3 with ( xIntL, yIntL ), ( xFracL, yFracL ) and refPicLX as inputs:

    - bdofFlag is equal to TRUE.

    - xL is equal to 0.

    - xL is equal to sbWidth + 1.

    - yL is equal to 0.

    - yL is equal to sbHeight + .

  - Otherwise, the following applies:

    - The motion vector mvLX is set equal to ( refMvLX − mvOffset ).

    - If dmvrFlag is equal to 0 and xFracL is not equal to 0 and yFracL is not equal to 0, padVal is derived as follows:

      - If sbWidth is equal to 4 and sbHeight is equal to 4

padVal[0] = 4

padVal[1] = 5

      - If sbWidth is equal to 4 and sbHeight is equal to 8

padVal[0] = 5

padVal[1] = 6

      - If sbWidth is equal to 8 and sbHeight is equal to 4

padVal[0] = 4

padVal[1] = 6

      - If sbWidth is equal to 4 and sbHeight is equal to 16

padVal[0] = 4

padVal[1] = 5

      - If sbWidth is equal to 16 and sbHeight is equal to 4

padVal[0] = 4

padVal[1] = 5

The prediction luma sample value predSamplesLX[ xL ][ yL ] is derived by invoking the luma sample 8-tap interpolation filtering process as specified in clause 8.5.7.3.2 with ( xIntL, yIntL ), ( xFracL, yFracL ), refPicLX, and padVal as inputs.

- Otherwise (cIdx is not equal to 0), the following applies:

  - Let ( xIntC, yIntC ) be a chroma location given in full-sample units and ( xFracC, yFracC ) be an offset given in 1/32 sample units. These variables are used only in this clause for specifying general fractional-sample locations inside the reference sample arrays refPicLX.

  - For each chroma sample location ( xC = 0..sbWidth − 1, yC = 0.. sbHeight − 1 ) inside the prediction chroma sample arrays predSamplesLX, the corresponding prediction chroma sample value predSamplesLX[ xC ][ yC ] is derived as follows:

- The variables xIntC, yIntC, xFracC and yFracC are derived as follows: [Ed. (SL): Shall we make it consistent: /2 or /SubWidthC and /SubHeightC?]

xIntC = ( xSb / SubWidthC ) + ( mvLX[ 0 ]  >>  5 ) + xC	(8-816)

yIntC = ( ySb / SubHeightC ) + ( mvLX[ 1 ]  >>  5 ) + yC	(8-817)

xFracC = mvLX[ 0 ] & 31		(8-818)

yFracC = mvLX[ 1 ] & 31		(8-819)

  - The motion vector mvLX is set equal to ( refMvLX − mvOffset ).

  - The list padVal[ dir ] is derived as follows for dir = 0..1:

    - The variable disp is derived as follows:

disp = ( refMvLX[ dir ]  >>  4) − ( mvLX[ dir ]  >>  4) + ( dir = = 0 ? xC : yC)	(8-820)

    - If disp is less than 0, padVal[ dir ] is set equal to disp.

    - Otherwise, if disp is greater than ( dir  = = 0  ?  sbWidth / SubWidthC : sbHeight / SubWidthC ) − 1, padVal[ dir ] is set equal to disp − ( ( dir  = =  0  ?  sbWidth / SubWidthC : sbHeight / SubWidthC ) − 1).

    - Otherwise, padVal[ dir ] is set equal to 0.

  - The prediction sample value predSamplesLX[ xC ][ yC ] is derived by invoking the process specified in clause 8.5.7.3.4 with ( xIntC, yIntC ), ( xFracC, yFracC ), refPicLX, and padVal as inputs.

##### 8.5.7.3.2	Luma sample 8-tap interpolation filtering process

Inputs to this process are:

–	a luma location in full-sample units ( xIntL, yIntL ),

–	a luma location in fractional-sample units ( xFracL, yFracL ),

–	the luma reference sample array refPicLXL,

–	a list padVal[ dir ] with dir = 0,1 specifying reference sample padding direction and amount.

Output of this process is a predicted luma sample value predSampleLXL

The variables shift1, shift2 and shift3 are derived as follows:

–	The variable shift1 is set equal to Min( 4, BitDepthY − 8 ), the variable shift2 is set equal to 6 and the variable shift3 is set equal to Max( 2, 14 − BitDepthY ).

–	The variable picW is set equal to pic_width_in_luma_samples and the variable picH is set equal to pic_height_in_luma_samples.

The luma interpolation filter coefficients fL[ p ] for each 1/16 fractional sample position p equal to xFracL or  yFracL are specified in Table 8-9.

The luma interpolation filter coefficients fPadL[ 0 ] are set equal to fL[ xFracL ] and fPadL[ 1 ] are set equal to fL[ yFracL ] and modified depending on padVal[ dir ] as follows for dir being equal to 0 and 1:

–	If padVal[ dir ] is equal to − 2, fPadL[ dir ] is modified as follows:

fPadL[ dir ][ 2 ] = fPadL[ xFracL ][ 2 ] + fPadL[ xFracL ][ 1 ] + fPadL[ xFracL ][ 0 ]	(8-821)

fPadL[ dir ][ 0 ] = 0		(8-822)

fPadL[ dir ][ 1 ] = 0		(8-823)

–	Otherwise, if padVal[ dir ] is equal to − 1, fPadL[ dir ] is modified as follows:

fPadL[ dir ][ 1 ] = fPadL[ dir ][ 1 ] + fPadL[ dir ][ 0 ]		(8-824)

fPadL[ dir ][ 0 ] = 0		(8-825)

–	Otherwise, if padVal[ dir ] is equal to 1, fPadL[ dir ] is modified as follows:

fPadL[ dir ][ 6 ] = fPadL[ dir ][ 6 ] + fPadL[ dir ][ 7 ]		(8-826)

fPadL[ dir ][ 7 ] = 0		(8-827)

–	Otherwise, if padVal[ dir ] is equal to 2, fPadL[ dir ] is modified as follows:

fPadL[ dir ][ 5 ] = fPadL[ dir ][ 5 ] + fPadL[ dir ][ 6 ] + fPadL[ dir ][ 7 ] 	(8-828)

fPadL[ dir ][ 6 ] = 0		(8-829)

fPadL[ dir ][ 7 ] = 0		(8-830)

–	Otherwise, if padVal[ dir ] is equal to 4, fPadL[ dir ] is modified as follows:

fPadL[ dir ][ 1 ] = fPadL[ xFracL ][ 1 ] + fPadL[ xFracL ][ 0 ]	(8-831)

fPadL[ dir ][ 0 ] = 0		(8-832)

fPadL[ dir ][ 6 ] = fPadL[ xFracL ][ 6 ] + fPadL[ xFracL ][ 7 ]	(8-833)

fPadL[ dir ][ 7 ] = 0		(8-834)

–	Otherwise, if padVal[ dir ] is equal to 5, fPadL[ dir ] is modified as follows:

fPadL[ dir ][ 1 ] = fPadL[ dir ][ 1 ] + fPadL[ dir ][ 0 ]		(8-835)

fPadL[ dir ][ 0 ] = 0		(8-836)

fPadL[ dir ][ 5 ] = fPadL[ dir ][ 5 ] + fPadL[ dir ][ 6 ] + fPadL[ dir ][ 7 ] 	(8-837)

fPadL[ dir ][ 6 ] = 0		(8-838)

fPadL[ dir ][ 7 ] = 0		(8-839)

–	Otherwise, if padVal[ dir ] is equal to 6, fPadL[ dir ] is modified as follows:

fPadL[ dir ][ 2 ] = fPadL[ xFracL ][ 2 ] + fPadL[ xFracL ][ 1 ] + fPadL[ xFracL ][ 0 ]	(8-840)

fPadL[ dir ][ 0 ] = 0		(8-841)

fPadL[ dir ][ 1 ] = 0		(8-842)

fPadL[ dir ][ 5 ] = fPadL[ dir ][ 5 ] + fPadL[ dir ][ 6 ] + fPadL[ dir ][ 7 ] 	(8-843)

fPadL[ dir ][ 6 ] = 0		(8-844)

fPadL[ dir ][ 7 ] = 0		(8-845)

The luma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..7:

xInti = sps_ref_wraparound_enabled_flag ?
	ClipH( ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, picW, xIntL + i − 3 ) :	(8-831) 
	Clip3( 0, picW − 1, xIntL + i − 3 )

yInti = Clip3( 0, picH − 1, yIntL + i − 3 )		(8-832)

The predicted luma sample value predSampleLXL is derived as follows:

–	If both xFracLand yFracL are equal to 0, the value of predSampleLXL is derived as follows:

predSampleLXL = refPicLXL[ xInt3 ][ yInt3 ] << shift3	(8-833)

–	Otherwise, if xFracL is not equal to 0 and yFracL is equal to 0, the value of predSampleLXL is derived as follows:

predSampleLXL =  i=07fPadL[ 0 ][ i ] * refPicLXL[ xInti ][ yInt3 ]   >>  shift1	(8-834)

–	Otherwise, if xFracL is equal to 0 and yFracL is not equal to 0, the value of predSampleLXL is derived as follows:

predSampleLXL =  i=07fPadL[ 1 ][ i ] * refPicLXL[ xInt3 ][ yInti ]   >>  shift1	(8-835)

–	Otherwise, if xFracL is not equal to 0 and yFracL is not equal to 0, the value of predSampleLXL is derived as follows:

- The sample array temp[ n ] with n = 0..7, is derived as follows:

temp[ n ] =  i=07fPadL[ 0 ][ i ] * refPicLXL[ xInti ][ yIntn ]   >>  shift1	(8-836)

- The predicted luma sample value predSampleLXL is derived as follows:

predSampleLXL =  i=07fPadL[ 1 ][ i ] * temp[ i ]   >>  shift2	(8-837)

# Experimental results

The proposed method was implemented on top of reference software VTM-4.0 [4], and simulations were performed following the common test conditions [5]. The results are shown in Table 5 and Table 6. As can be seen, proposed method has negligible impact on compression performance.

Table 5: Coding performance of config #1.

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.04% | 0.01% | 99% | 100% |  |  |  |  |  |
| Class A2 | -0.01% | -0.01% | -0.05% | 99% | 101% |  |  |  |  |  |
| Class B | 0.02% | -0.07% | -0.07% | 98% | 100% | 0.01% | -0.10% | -0.21% | 100% | 100% |
| Class C | 0.04% | -0.04% | -0.17% | 97% | 97% | -0.02% | 0.20% | 0.18% | 99% | 98% |
| Class E |  |  |  |  |  | 0.01% | -0.30% | 0.07% | 100% | 98% |
| Overall | 0.01% | -0.04% | -0.08% | 98% | 99% | 0.00% | -0.05% | -0.01% | 99% | 99% |
| Class D | 0.03% | 0.04% | -0.02% | 96% | 97% | 0.00% | 0.20% | -1.00% | 100% | 99% |
| Class F | 0.00% | 0.01% | -0.02% | 98% | 97% | -0.03% | -0.34% | -0.12% | 98% | 99% |

Table 6: Coding performance of config #2.

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.04% | 0.04% | 99% | 100% |  |  |  |  |  |
| Class A2 | -0.01% | -0.02% | 0.06% | 100% | 101% |  |  |  |  |  |
| Class B | 0.01% | -0.10% | -0.10% | 98% | 100% | -0.01% | -0.04% | 0.17% | 101% | 102% |
| Class C | 0.02% | -0.02% | -0.22% | 97% | 97% | 0.00% | 0.00% | -0.08% | 98% | 100% |
| Class E |  |  |  |  |  | -0.03% | 0.37% | -0.39% | 100% | 97% |
| Overall | 0.00% | -0.05% | -0.07% | 99% | 99% | -0.01% | 0.07% | -0.05% | 99% | 100% |
| Class D | -0.03% | -0.03% | 0.14% | 97% | 97% | -0.02% | 0.16% | -0.63% | 99% | 98% |
| Class F | 0.01% | 0.01% | -0.07% | 98% | 98% | -0.09% | -0.20% | 0.00% | 98% | 99% |

# Conclusions

This contribution proposes a method to reduce the memory bandwidth required by small CUs. With proposed method, the worst-case memory bandwidth of VVC reduced dramatically. Meanwhile, there is negligible impact on compression performance. It is recommended to adopt it into VVC.

# References

- “Versatile Video Coding (Draft 4)”, B. Bross, J. Chen, S. Liu, Doc. JVET-M1001, Jan. 2019.

- “BoG report on CE2 related contributions”, Y. He, Doc. JVET-M0862, Jan. 2019.

- “AHG16: Motion compensation with padded samples for small coding units”, H. Liu, J. Chon, H.-C. Chuang, L. Zhang, K. Zhang, J. Xu, Doc. JVET-M0248, Jan. 2019.

- VTM-4.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-4.0

- “JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Suehring, Doc. JVET-M1010, Jan. 2019.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).