| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0271-v2 |
| --- | --- |

| Title: | CE3-1.5: CCLM derived with four neighbouring samples |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Meng Wang, Kai Zhang, Li Zhang, Hongbin Liu, Jizheng Xu, Shiqi Wang (Bytedance) Jurun Li, Shanshe Wang, Wen Gao (Peking University) | Tel: Email: | +1-858-380-8522 wangmeng.kids@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. and Peking University. |  |  |

_____________________________

# Abstract

In this contribution, it is proposed to derive the CCLM parameters with at most four neighbouring chroma samples and their corresponding down-sampled luma samples. It is asserted that the number of luma down-sampling is reduced from 64 to 4, and the number of comparisons is reduced from 128 to 4, in the CCLM parameter derivation for a 32×32 chroma block. Simulation results reportedly show -0.05% and -0.05% BD-rate changing under AI and RA configurations, respectively, with almost no change on encoding/decoding time.

# Introduction

In VTM-4.0, CCLM parameters are derived with a max-min method. With LM mode, the minimum and maximum down-sampled neighbouring luma samples denoted as xA and xB are found by performing (W+H-1) comparisons on all down-sampled neighbouring luma samples. With LM-A or LM-L modes, xA and xB are found by performing 2W or 2H comparisons on all extended above or left down-sampled neighbouring luma samples. Suppose corresponding chroma sample values are yA and yB, then the linear model parameters α and β are derived as:β= ∑Cn-α·∑LnN

.

The derivation of CCLM parameters involves quite a lot of comparison operations, which are not desirable in hardware or software design.

# Proposed methods

In JVET-M0274 [1], it is proposed to derive the CCLM parameters with at most four neighbouring chroma samples and their corresponding down-sampled luma samples.

Suppose the current chroma block dimensions are W×H, then W’ and H’ are set as

- W’=W, H’=H when LM mode is applied;

- W’=W+H when LM-A mode is applied;

- H’=H+W when LM-L mode is applied;

The above neighbouring positions are denoted as S[0, -1]…S[W’-1, -1] and the the left neighbouring positions are denoted as S[-1, 0]…S[-1, H’-1]. Then the four samples are selected as

- S[W’/4, -1], S[3W’/4, -1], S[-1, H’/4], S[-1, 3H’/4] when LM mode is applied and both above and left neighbouring samples are available;

- S[W’/8, -1], S[3W’/8, -1], S[5W’/8, -1], S[7W’/8, -1] when LM-A mode is applied or only the above neighbouring samples are available;

- S[-1, H’/8], S[-1, 3H’/8], S[-1, 5H’/8], S[-1, 7H’/8] when LM-L mode is applied or only the left neighbouring samples are available;

The four neighbouring luma samples at the selected positions are down-sampled and compared four times to find two smaller values: x0A and x1A, and two larger values: x0B and x1B. Their corresponding chroma sample values are denoted as y0A, y1A, y0B and y1B. Then xA, xB, yA and yB are derived as:

xA=(x0A + x1A +1)>>1; xB=(x0B + x1B +1)>>1; yA=(y0A + y1A +1)>>1; yB=(y0B + y1B +1)>>1.

Table 1 summarizes the complexity comparison between VTM-4.0 and the proposed method to derive the CCLM parameters.

Table 1: Complexity comparison for CCLM parameter derivation on a 32×32 chroma block.

|  | Luma sample down-sampling | Comparison between luma samples |
| --- | --- | --- |
| VTM-4.0 | 64 | 128 |
| Proposed | 4 | 4 |
| Proposed/VTM-4.0 | 6% | 3% |

# Experimental Results

The proposed method has been implemented on top of VTM-4.0. Results under AI/RA configurations are summarized in Table 2. The simulations were performed following the JVET CTC [2]. More detail results can be found in the attached spreadsheets.

Table 2: Results for the CE3-1.5

|  | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.17% | -0.16% | -0.46% | 96% | 100% | -0.16% | -0.06% | -0.40% | 100% | 99% |
| Class A2 | -0.03% | -0.04% | -0.08% | 100% | 101% | -0.04% | 0.08% | -0.06% | 100% | 98% |
| Class B | -0.02% | -0.59% | -0.70% | 99% | 100% | 0.00% | -0.82% | -1.13% | 99% | 99% |
| Class C | -0.02% | -0.47% | -0.33% | 100% | 101% | -0.03% | -0.32% | -0.59% | 101% | 101% |
| Class E | -0.02% | -0.01% | -0.15% | 101% | 100% |  |  |  |  |  |
| Overall | -0.05% | -0.30% | -0.38% | 99% | 100% | -0.05% | -0.36% | -0.63% | 100% | 100% |
| Class D | -0.01% | -0.21% | -0.34% | 99% | 97% | -0.02% | -0.18% | -0.32% | 99% | 98% |
| Class F | 0.01% | -0.04% | -0.21% | 100% | 103% | 0.00% | -0.09% | -0.21% | 100% | 104% |

# Conclusion

Because the proposed method can significantly reduce the number of down-sampling and comparison on neighbouring luma samples to derive the CCLM parameters while improving the coding performance slightly, it is recommended to adopt CE3-1.5 into VVC.

# References

- M. Wang, K. Zhang, L. Zhang, H. Liu, J. Xu, S. Wang, J. Li, S. Wang, W. Gao, “CE3-related: Modified linear model derivation for CCLM modes,” JVET-M0274, Jan. 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, Jan. 2019.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Peking Unviersity may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).