| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0294-v2 |
| --- | --- |

| Title: | CE9-related: Adaptive search pattern for DMVR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sriram Sethuraman | Tel: Email: | +91-8066601077 Sriram.sethuraman@ittiam.com |
| Source: | Ittiam Systems Pvt. Ltd. |  |  |

_____________________________

# Abstract

DMVR was adopted at the last meeting into VTM with an exhaustive search over all 25 search points in a +/-2 refinement range with the cost comparisons happening in a raster order. The core experiments related to DMVR had all been done with an adaptive search pattern. In this proposal, the CTC results of the exhaustive method are compared against those with the adaptive search pattern. The results show that the exhaustive search has a small (~0.04%) BDRATE gain over the adaptive search pattern, while the adaptive search pattern offers several software and hardware implementation benefits. A new adaptive search pattern is proposed that retains most of the advantages of the earlier adaptive search pattern while bridging most of the BDRATE gap with the exhaustive search. Given the software and hardware implementation flexibilities offered by the adaptive search pattern, it is requested that the new adaptive search pattern be considered for adoption into the next VTM.

# Introduction

The complexity associated with DMVR has two major components. The first component is associated with the bilinear interpolation that is performed to obtain the predicted sample values to use for refinement. The second component is associated with the search complexity. The currently adopted DMVR employs an exhaustive 25-point search that requires sum of absolute differences of predicted samples to be evaluated. While the exhaustive evaluation is straight-forward to understand, it results in a higher average decoding time for software implementations and offers less flexibility to hardware designs. For example, if a design wanted to have only 9 concurrent SAD computation units, the 25 cost evaluations will require 3 stages to complete all evaluations. On the other hand, if a 2-stage cost evaluation is required, then 13 concurernt SAD evaluation units will be required.

Adaptive search pattern methods studied in CE9 over the past 3 meetings do not preclude exhaustive evaluation of the SAD costs at all 25 positions needed to achieve the lowest timing in hardware. However, they offer additional flexibility for hardware designers either to reduce the number of SAD computation units or to reduce the number of stages required with a given number of SAD computation units to achieve the best timing. Also, they offer reduced average decoding time in software implementations.

In this proposal, the adaptive search pattern studied in JVET-M0147 is described again along with its advantages. Also, a new variant of this adaptive search pattern is presented. This method retains the advantages of the earlier adaptive search pattern while bridging most of the BDRATE gap with respective to the exhaustive 25-point search.

# Search Pattern in DMVR

## Adaptive search pattern used in JVET-M0147

VTM4.0 includes this adaptive search pattern, while subsequent merge request has changed the search pattern to exhaustive 25-point square shaped search space corresponding to +/-2 refinement in both the horizontal and vertical directions.

The adaptive search pattern first evaluates the SAD cost at 5 search positions corresponding to the center of the refinement and its 4-connected 1-pixel neighbors that are to the left, top, right, and bottom of the center position. The lowest matching cost and the second lowest matching cost positions among these 5 positions are used for selecting the subsequent positions to search. Specifically, the position with the lower cost among the left and right neighbors to the center position and the position with lower cost among the top and bottom neighbors to the center position are used to select one out of the 4 diagonal search points. In the event that the lowest matching cost point corresponds to the center position, the second stage search space includes this additional diagonal point and its 4-connected 1-pixel left, top, right, and bottom neighbors at which matching costs have not been evaluated yet. If the lowest matching cost corresponds to a non-center position, the second stage search space of points will then include the 4-connected 1-pixel left, top, right and bottom neighbors of the lowest matching cost search point and the additional diagonal point at which matching costs have not been evaluated yet. This constitutes a maximum of 5 new search points. Based on the matching costs at the second search space points and the first 5 search points, one additional search point is chosen as a 3rd stage search point. Alternatively, two additional search points can be included in the second search space itself taking the total of additional search points to 8.

2

1

1

1

1

1

2

2

2

2

2

2

2

Fig.1 illustrates an example where position (1,0) corresponds to the lowest cost search point among the first 5 search points evaluated and the position (0,-1) corresponds to the lower cost position among (0,-1) and (0,1). The second search space is illustrated with all the additional positions marked with a 2 below them.

Figure 1: Adaptive search pattern with 5 stage-1 and 7 stage-2 search points

The above method can be realized in software implementation with SAD evaluations only at 9 search points. In hardware designs that can have up to 4 stages, the maximum number of concurrent SAD evaluation units can be maintained at 5. When 2 stages are allowed, the maximum number of concurrent SAD evaluation units can be maintained at 8. At the same time, it does not preclude a one-shot evaluation of all 25 matching costs as long as the order of cost comparisons can be performed in the specified order.

## New adaptive search pattern

Instead of selecting one diagonal search point in the above method based on the lower cost search points in the horizontal non-center and vertical non-center positions, this method selects two diagonal search points for the first 5 search points that are in the 4-conencted 1-pixel neighborhood of the lowest cost non-center search point. Hence the second search space has 2 additional search points. Based on the lowest cost search point among the 7 search points, up to 5 additional search points are added. Specifically, if one of the 2 diagonal search points corresponds to the lowest matching cost, then the 5 additional search points are chosen to complete the 8-connected neighbors of that point. On the other hand, if the non-diagonal search point corresponds to the lowest matching cost, 3 additional search points to complete the 8-connected neighbors of the lowest matching cost point are added. Hence, this method requires 3 stages of cost computations with a maximum of 5 concurrent SAD units. If a 2-stage cost evaluation is needed from a timing perspective, it can be done by first evaluating the 8 1-connected neighbors of the center search point followed by 3 or 5 search points depending on the same order of cost comparisons. In this case, 9 concurrent SAD evaluation units are required. In software implementations, this method requires SAD costs to be evaluated only at 11 search points.

Figure 2 illustrates this new adaptive search pattern. In the example shown, position (1,0) corresponds to the lowest matching cost position among the first 5 cost evaluations. The position (1,1) is the lowest cost among all the first and second search space points. The position (2,2) emerges as the final lowest cost search point in this example.

1

1

1

1

1

2

3

3

3

3

2

3

# Results

Since VTM4.0 includes the adaptive search pattern described in section 2.1, the results of the exhaustive 25-point search are compared under CTC against VTM4.0 in Table 3.1. The result shows that the exhaustive 25-point search offers a 0.04% BDRATE gain over the adaptive search pattern in section 2.1, while having a 2% increase in decoding time ratio.

Table 3.1: Results for 25-point exhaustive search over VTM4.0

In Table3.2, the results for the new adaptive search pattern are compared against VTM4.0. It can be seen that this new adaptive search pattern bridges most of the gap against the 25-point exhaustive search. It has a slight increase in decoding time over the adaptive search pattern used in VTM4.0.

Table 3.2: Results for new adaptive search pattern over VTM4.0

# Conclusions

In this proposal, the merits of the adaptive search pattern over an exhaustive search method are presented. The method studied in core experiments earlier and that is part of VTM4.0 before the 25-point exhaustive method was committed offers a flexible method that can reduce hardware gate-count and software average decoding times. However, it has a 0.04% BDRATE gap against the 25-point exhaustive search. A new adaptive search pattern that bridges most of this BDRATE gap and offers a 3-stage option in hardware with just 5 SAD computation units is presented. It is requested that the adaptive search pattern be considered for further study in core experiments.

# Patent rights declaration(s)

Ittiam Systems Pvt. Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).