| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, Switzerland, 19–27 Mar. 2019 | Document: JVET-N0319 |
| --- | --- |

| Title: | CE2: Symmetric MVD for affine bi-prediction coding (CE2-1.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hua Yang, Jiancong Luo, Yuwen He 4 Research Way, Princeton, NJ 08540 | Tel: Email: | +1-609-608-0479 Hua.Yang@InterDigital.com Daniel.Luo@InterDigital.com Yuwen.He@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This document reports the results of Core Experiment (CE) 2 test 2.1.2 on symmetric MVD (SMVD) for affine bi-prediction coding. SMVD for affine bi-prediction coding proposed in JVET-M0467 [1], is tested under VTM4.0 [2]. Two tests are reported in this contribution. Test 2.1.2.a is the original SMVD approach signalling MVDs of all the affine control points of List 0. Test 2.1.2.b is an alternative approach only signalling MVD of the top-left (TL) control point. Simulation results reportedly show on average -0.10% Y-BD rate saving with 6% encoding time increase for RA in test 2.1.2a and -0.07% Y-BD rate saving with 5% encoding time increase for RA in test 2.1.2b.

# Introduction

SMVD mode is proposed in JVET-L0370 [3] to improve coding efficiency of bi-prediction via symmetrically mapping coded MVD of List 0 to that of List1. In JVET-M0467 [1], this mode is extended from regular bi-prediction to affine bi-prediction, where further coding gain was achieved.

Regarding SMVD mode for affine bi-prediction, motion information including reference picture indices of both list-0 and list-1 and control point MVDs of list-1 are derived at the decoder.

Firstly, variables BiDirPredFlag at slice level is derived in the same way as JVET-M0444 [4]:

- If mvd_l1_zero_flag is 1, BiDirPredFlag is set equal to 0.

- Otherwise, if the nearst reference picture in list-0 and the nearst reference picture in list-1 form a forward and backward pair of reference pictures or a backward and forward pair of reference pictures, BiDirPredFlag is set to 1. Otherwise BiDirPredFlag is set to 0.

Secondly, a symmetrical mode flag at CU level indicating whether symmetrical mode is used or not is explicitly signalled if the CU is bi-prediction coded and BiDirPredFlag is equal to 1.

When the symmetrical mode flag is true, only MVP index flags mvp_l0_flag, mvp_l1_flag and MVD in list-0 (MVD0) are explicitly signalled. The reference indices for list-0 and list-1 are set equal to the pair of reference pictures, respectively. If the affine mode flag is true, top-left control point MVD of list-1 is set equal to the negative of the top-left control point MVD of list-0. The other control point MVDs of list 1 are set to zero. The final control point motion vectors are derived in below formulas:

For the top-left control point:

mvx0,0, mvy0,0=(mvpx0,0+mvdx0,0,mvpy0,0+mvdy0,0) mvx1,0, mvy1,0=(mvpx1,0-mvdx0,0,mvpy1,0-mvdy0,0)

For the other control points (j = 1 or 2):

mvx0,j, mvy0,j=(mvpx0,j+mvdx0,j+mvdx0,0,mvpy0,j+mvdy0,j+mvdy0,0) mvx1,j, mvy1,j=(mvpx1,j-mvdx0,0,mvpy1,j-mvdx0,0)

where the first dimension of the subscripts indicates the reference list, and the second dimension of the subscripts indicates the control point index.

The original encoding algorithm of affine SMVD from JVET-M0467 [1] involves two iterations of translational motion estimation (ME) and affine ME. In this report, for the 2nd iteration, affine ME is not used for a better trade-off of coding complexity and performance.

In this report, an alternative approach of only signaling MVD of the top-left control point of List 0 is tested in test 2.1.2b to see if better rate-distortion (RD) coding performance can be achieved. The MVs for the other control points (j = 1 or 2)  are derived as:

mvx0,j, mvy0,j=(mvpx0,j+mvdx0,0,mvpy0,j+mvdy0,0) mvx1,j, mvy1,j=(mvpx1,j-mvdx0,0,mvpy1,j-mvdx0,0)

Moreover, the original encoding algorithm of affine SMVD from JVET-M0467 [1] involves two iterations of translational motion estimation (ME) and affine ME. In this report, for the 2nd iteration, affine ME is not used for a better trade-off of coding complexity and performance.

# Simulation results

The test is implemented and conducted under VTM-4.0 [2] with the common test conditions as described in JVET-M1010 [5]. The simulation results reportedly show the original all MVDs signaling approach achieves average Y BD-rate reduction by 0.10% with 106% encoding time for RA, and the alternative TL MVD only signaling achieves 0.07% Y BD-rate reduction with 105% encoding time for RA. Note that only RA tests, but no LD tests, are conducted herein, as SMVD mode is only applicable to bi-prediction coding when the reference pictures coming from two sides (i.e. forward and backward).  The simulation results reportedly show the improved all MVD signaling SMVD approach (test 2.1.2a) achieves average Y BD-rate reduction by 0.10% with 106% encoding time for RA, and the alternative TL MVD only signaling (test 2.1.2b) achieves 0.07% Y BD-rate reduction with 105% encoding time for RA.

Table 1. Simulation results of test 2.1.2a - Signalling MVDs of all control points of List 0

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-4.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.05% | -0.10% | 106% | 100% |
| Class A2 | -0.19% | -0.10% | -0.08% | 107% | 99% |
| Class B | -0.09% | -0.03% | -0.05% | 106% | 100% |
| Class C | -0.03% | -0.07% | -0.05% | 105% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.10% | -0.06% | -0.06% | 106% | 100% |
| Class D | -0.03% | 0.01% | 0.01% | 104% | 100% |
| Class F | -0.08% | -0.03% | -0.06% | 104% | 100% |

Table 2. Simulation results of test 2.1.2b – Signalling MVD of TL control point only of List 0

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | -0.01% | -0.07% | 105% | 100% |
| Class A2 | -0.15% | -0.07% | -0.09% | 106% | 100% |
| Class B | -0.05% | -0.05% | -0.05% | 105% | 100% |
| Class C | -0.01% | -0.06% | 0.01% | 103% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.07% | -0.05% | -0.05% | 105% | 100% |
| Class D | 0.00% | 0.01% | -0.02% | 103% | 101% |
| Class F | -0.04% | -0.02% | -0.03% | 103% | 100% |

# References

- J. Luo, Y. He, “CE2-related: Symmetric MVD for Affine Bi-prediction Coding”, JVET-M0467, the 13th JVET meeting, Jan. 2019, Marrakech.

- VTM-4.0 reference software, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-4.0

- H. Chen, H. Yang, J. Chen, “CE4: Symmetrical MVD mode (Test 4.5.1)”, JVET-L0370, the 12th JVET meeting, Oct. 2018, Macao, CN.

- J. Luo, Y. He, “CE2-related: Simplified symmetric MVD based on CE4.4.3”, JVET-M0444, the 13th JVET meeting, Jan. 2019, Marrakech.

- F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-M1010, the 13th JVET meeting, Jan. 2019, Marrakech.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).