| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0332 |
| --- | --- |

| Title: | Non-CE4: MVD scaling issue for LTRPs |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen Xiaoyu Xiu Tsung-Chuan Ma Xianglin Wang | Email: | yiwenchen@kwai.com xiaoyuxiu@kwai.com tsung-chuanma@kwai.com xianglinwang@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In this proposal, it is proposed to disable the motion vector difference (MVD) scaling process for MMVD when a picture refers to a long-term reference picture as a reference for inter prediction. It is suggested to consider the LTRP in refining the specification for MVD scaling process for MMVD. The simulation results reportedly show that the propose modification introduces no coding performance for VTM-4.0 under CTC.

# Introduction

In current MMVD design, after the MV offset is determined based on the signaled index, the final MVDs are further derived through the following steps:

If the starting MV is a bi-prediction MV, the following applies:

The luma merge motion vector differences mMvdL0 and mMvdL1 are derived as follows:

- If both predFlagL0 and predFlagL1 are equal to 1, the following applies:

currPocDiffL0  =  DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ] )	(8-356)

currPocDiffL1  =  DiffPicOrderCnt( currPic, RefPicList[ 1 ][ refIdxL1 ] )	(8-357)

- If currPocDiffL0 is equal to currPocDiffL1, the following applies:

mMvdL0[ 0 ]  =  MmvdOffset[ xCb ][ yCb ][ 0 ]	(8-358)

mMvdL0[ 1 ]  =  MmvdOffset[ xCb ][ yCb ][ 1 ]	(8-359)

mMvdL1[ 0 ]  =  MmvdOffset[ xCb ][ yCb ][ 0 ]	(8-360)

mMvdL1[ 1 ]  =  MmvdOffset[ xCb ][ yCb ][ 1 ]	(8-361)

- Otherwise, if Abs( currPocDiffL0 ) is greater than or equal to Abs( currPocDiffL1 ), the following applies:

td = Clip3( −128, 127, currPocDiffL0 )		(8-362)

tb = Clip3( −128, 127, currPocDiffL1 )		(8-363)

tx = ( 16384 + ( Abs( td )  >>  1 ) ) / td		(8-364)

distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 )  >>  6 )	(8-365)

mMvdL0[ 0 ]  =  MmvdOffset[ xCb ][ yCb ][ 0 ]	(8-366)

mMvdL0[ 1 ]  =  MmvdOffset[ xCb ][ yCb ][ 1 ]	(8-367)

mMvdL1[ 0 ] = Clip3( −215, 215 − 1, Sign( distScaleFactor * mMvdL0[ 0 ] )  *  	(8-368)
	( ( Abs( distScaleFactor * mMvdL0[ 0 ] ) + 127 )  >>  8 ) )

mMvdL1[ 1 ] = Clip3( −215, 215 − 1, Sign( distScaleFactor * mMvdL0[ 1 ] )  *  	(8-369)
	( ( Abs( distScaleFactor * mMvdL0[ 1 ] ) + 127 )  >>  8 ) )

- Otherwise (Abs( currPocDiffL0 ) is less than Abs( currPocDiffL1 )), the following applies:

td = Clip3( −128, 127, currPocDiffL1 )		(8-370)

tb = Clip3( −128, 127, currPocDiffL0 )		(8-371)

tx = ( 16384 + ( Abs( td )  >>  1 ) ) / td		(8-372)

distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 )  >>  6 )	(8-373)

mMvdL1[ 0 ]  =  MmvdOffset[ xCb ][ yCb ][ 0 ]	(8-374)

mMvdL1[ 1 ]  =  MmvdOffset[ xCb ][ yCb ][ 1 ]	(8-375)

mMvdL0[ 0 ] = Clip3( −215, 215 − 1, Sign( distScaleFactor * mMvdL1[ 0 ] )  *  	(8-376)
	( ( Abs( distScaleFactor * mMvdL1[ 0 ] ) + 127 )  >>  8 ) )

mMvdL0[ 1 ] = Clip3( −215, 215 − 1, Sign( distScaleFactor * mMvdL1[ 1 ] )  *  	(8-377)
	( ( Abs( distScaleFactor * mMvdL1[ 1 ] ) + 127 )  >>  8 ) )

- Otherwise ( predFlagL0 or predFlagL1 are equal to 1 ), the following applies for X being 0 and 1:

mMvdLX[ 0 ] = ( predFlagLX = = 1 )  ?  MmvdOffset[ xCb ][ yCb ][ 0 ]  :  0	(8-378)

mMvdLX[ 1 ] = ( predFlagLX = = 1 )  ?  MmvdOffset[ xCb ][ yCb ][ 1 ]  :  0 	   (8-379)

# Proposed Method

In the MVD scaling process for MMVD, POC-based MVD scaling scheme does not consider long term reference pictures where the delta POC difference of a target picture and a LTRP may be extremely large. In that case, performing POC-based scaling on an MV which points to a long term reference picture may result in the scaled MV having an extremely large or small magnitude. In such a case, the accuracy and efficiency of MV prediction process becomes suboptimal. The proposed constraint considering the LTRP is illustrated as below.

The luma merge motion vector differences mMvdL0 and mMvdL1 are derived as follows:

- If both predFlagL0 and predFlagL1 are equal to 1, the following applies:

currPocDiffL0  =  DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ] )	(8-356)

currPocDiffL1  =  DiffPicOrderCnt( currPic, RefPicList[ 1 ][ refIdxL1 ] )	(8-357)

- If currPocDiffL0 is equal to currPocDiffL1, the following applies:

mMvdL0[ 0 ]  =  MmvdOffset[ xCb ][ yCb ][ 0 ]	(8-358)

mMvdL0[ 1 ]  =  MmvdOffset[ xCb ][ yCb ][ 1 ]	(8-359)

mMvdL1[ 0 ]  =  MmvdOffset[ xCb ][ yCb ][ 0 ]	(8-360)

mMvdL1[ 1 ]  =  MmvdOffset[ xCb ][ yCb ][ 1 ]	(8-361)

- Otherwise, if Abs( currPocDiffL0 ) is greater than or equal to Abs( currPocDiffL1 ), the following applies:

td = Clip3( −128, 127, currPocDiffL0 )		(8-362)

tb = Clip3( −128, 127, currPocDiffL1 )		(8-363)

tx = ( 16384 + ( Abs( td )  >>  1 ) ) / td		(8-364)

distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 )  >>  6 )	(8-365)

mMvdL0[ 0 ]  =  MmvdOffset[ xCb ][ yCb ][ 0 ]	(8-366)

mMvdL0[ 1 ]  =  MmvdOffset[ xCb ][ yCb ][ 1 ]	(8-367)

if RefPicList[ 0 ][ refIdxL0 ] is not a long-term reference picture and RefPicList[ 1 ][ refIdxL1 ] is not a long-term reference picture, the following applies:

mMvdL1[ 0 ] = Clip3( −215, 215 − 1, Sign( distScaleFactor * mMvdL0[ 0 ] )  *  	(8-368)
	( ( Abs( distScaleFactor * mMvdL0[ 0 ] ) + 127 )  >>  8 ) )

mMvdL1[ 1 ] = Clip3( −215, 215 − 1, Sign( distScaleFactor * mMvdL0[ 1 ] )  *  	(8-369)
	( ( Abs( distScaleFactor * mMvdL0[ 1 ] ) + 127 )  >>  8 ) )

Otherwise, the following applies:

mMvdL1[ 0 ] = (currPocDiffL0*currPocDiffL1>0)? mMvdL0[ 0 ]: -mMvdL0[ 0 ];

mMvdL1[ 1 ] = (currPocDiffL0*currPocDiffL1>0)?mMvdL0[ 1 ]: -mMvdL0[ 1 ];

- Otherwise (Abs( currPocDiffL0 ) is less than Abs( currPocDiffL1 )), the following applies:

td = Clip3( −128, 127, currPocDiffL1 )		(8-370)

tb = Clip3( −128, 127, currPocDiffL0 )		(8-371)

tx = ( 16384 + ( Abs( td )  >>  1 ) ) / td		(8-372)

distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 )  >>  6 )	(8-373)

mMvdL1[ 0 ]  =  MmvdOffset[ xCb ][ yCb ][ 0 ]	(8-374)

mMvdL1[ 1 ]  =  MmvdOffset[ xCb ][ yCb ][ 1 ]	(8-375)

if RefPicList[ 0 ][ refIdxL0 ] is not a long-term reference picture and RefPicList[ 1 ][ refIdxL1 ] is not a long-term reference picture

mMvdL0[ 0 ] = Clip3( −215, 215 − 1, Sign( distScaleFactor * mMvdL1[ 0 ] )  *  	(8-376)
	( ( Abs( distScaleFactor * mMvdL1[ 0 ] ) + 127 )  >>  8 ) )

mMvdL0[ 1 ] = Clip3( −215, 215 − 1, Sign( distScaleFactor * mMvdL1[ 1 ] )  *  	(8-377)
	( ( Abs( distScaleFactor * mMvdL1[ 1 ] ) + 127 )  >>  8 ) )

Otherwise, the following applies:

mMvdL0[ 0 ] = (currPocDiffL0*currPocDiffL1>0)?mMvdL1[ 0 ]: -mMvdL1[ 0 ];

mMvdL0[ 1 ] = (currPocDiffL0*currPocDiffL1>0)?mMvdL1[ 1 ]: -mMvdL1[ 1 ];

- Otherwise ( predFlagL0 or predFlagL1 are equal to 1 ), the following applies for X being 0 and 1:

mMvdLX[ 0 ] = ( predFlagLX = = 1 )  ?  MmvdOffset[ xCb ][ yCb ][ 0 ]  :  0	(8-378)

mMvdLX[ 1 ] = ( predFlagLX = = 1 )  ?  MmvdOffset[ xCb ][ yCb ][ 1 ]  :  0	       (8-379)

# Results

The proposed modification is integrated into VTM-4.0 [1] and the results show that the proposed modification introduces no impact on the coding performance under CTC [2] against VTM-4.0.

# Conclusion

This contribution proposes to disable the motion vector difference scaling process for MMVD when a picture refers to a long-term reference picture as a reference for inter prediction. It is suggested that the JVET considers this proposal in refining the specification for MVD scaling process for MMVD.

# Reference

- VTM-4.0 in https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- “JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, doc. JVET-M1010, January 2019.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).