| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0341 |
| --- | --- |

| Title: | On reporting combined YUV BD rates |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Frank Bossen | Tel: Email: | +1 647 745 1691 fbossen@sharplabs.com |
| Source: | Sharp Electronics of Canada Ltd. |  |  |

_____________________________

# Abstract

It is proposed to report combined YUV BD rate numbers in addition to separate Y, U, and V numbers.

# Problem Statement

BD rate results are currently reported separately for luma and chroma channels. While trends are often similar for each channel, it is not uncommon for results to show divergence. Rules of thumb, such as a 90/5/5 weighting of Y, U and V BD rate differences, may be used to determine an overall difference. However, such rules may not be very robust, and it is desirable to have a less ad-hoc approach.

# Proposed solution

It is proposed to include the following additional information in the Excel summary sheet:

- Combined YUV PSNR that is computed based on the MSE across all components and all frames of a test point. Such a YUV PSNR metric is already reported by the VTM software on a per-sequence basis

- BD rate values for the combined YUV PSNR metric

## VTM 4.0 vs VTM 3.2

In the tables below, VTM version 4.0 is compared to VTM version 3.2. This is a clear example where luma and chroma BD rate numbers diverge. The combined YUV numbers are here more useful to measure progress between the two VTM versions.

|  |  |  | All Intra |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.2 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | -2.66% | -3.82% | -4.59% | -1.64% | 128% | 96% |
| Class A2 | -2.65% | -2.97% | -2.75% | -1.69% | 124% | 101% |
| Class B | -2.18% | -1.75% | -2.17% | -1.70% | 122% | 100% |
| Class C | -2.40% | 1.58% | 1.21% | -1.76% | 120% | 113% |
| Class E | -2.45% | 1.57% | 1.27% | -2.02% | 129% | 106% |
| Overall | -2.43% | -1.01% | -1.35% | -1.76% | 124% | 103% |
| Class D | -2.18% | 0.09% | 0.08% | -1.79% | 114% | 110% |
| Class F | -18.38% | -14.24% | -14.34% | -17.60% | 201% | 107% |
|  |  |  | Random Access |  |  |  |
|  |  |  | Over VTM-3.2 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | -5.43% | 6.44% | 3.69% | -3.68% | 145% | 116% |
| Class A2 | -6.83% | 5.50% | 6.18% | -4.17% | 157% | 121% |
| Class B | -5.04% | 5.89% | 5.33% | -3.93% | 154% | 115% |
| Class C | -5.16% | 7.10% | 7.61% | -3.74% | 158% | 120% |
| Class E |  |  |  |  |  |  |
| Overall | -5.51% | 6.25% | 5.78% | -3.88% | 154% | 118% |
| Class D | -4.90% | 6.63% | 6.93% | -3.89% | 169% | 123% |
| Class F | -17.37% | -9.36% | -9.08% | -16.01% | 161% | 119% |
|  |  |  | Low delay B |  |  |  |
|  |  |  | Over VTM-3.2 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -4.15% | 19.69% | 20.48% | -1.91% | 156% | 115% |
| Class C | -4.19% | 14.54% | 15.70% | -2.12% | 154% | 116% |
| Class E | -1.95% | 9.93% | 12.19% | -0.77% | 143% | 114% |
| Overall | -3.61% | 15.53% | 16.81% | -1.69% | 152% | 115% |
| Class D | -4.48% | 15.65% | 18.69% | -2.68% | 170% | 112% |
| Class F | -13.29% | 0.70% | 1.13% | -11.11% | 155% | 112% |
|  |  |  | Low delay P |  |  |  |
|  |  |  | Over VTM-3.2 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -4.04% | 21.25% | 22.53% | -1.71% | 167% | 115% |
| Class C | -4.12% | 15.63% | 16.57% | -1.95% | 171% | 113% |
| Class E | -2.15% | 10.92% | 12.28% | -0.90% | 154% | 107% |
| Overall | -3.60% | 16.80% | 17.98% | -1.59% | 165% | 113% |
| Class D | -4.56% | 16.54% | 19.21% | -2.72% | 185% | 113% |
| Class F | -13.50% | 0.81% | 1.34% | -11.31% | 168% | 111% |

## VTM 4.0 vs HM 16.20

In the tables below, VTM version 4.0 is compared to HM version 16.20. In this case the luma and chroma BD rate numbers are relatively well balanced. The YUV BD rate numbers are thus expectedly close to the luma numbers.

|  |  |  | All Intra |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over HM-16.20 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | -23.74% | -39.31% | -36.64% | -25.30% | 1353% | 167% |
| Class A2 | -22.96% | -25.02% | -19.85% | -21.00% | 2185% | 172% |
| Class B | -19.13% | -24.25% | -31.00% | -19.66% | 2273% | 168% |
| Class C | -19.52% | -21.31% | -24.54% | -19.78% | 2987% | 181% |
| Class E | -22.80% | -23.91% | -27.15% | -23.11% | 1843% | 167% |
| Overall | -21.23% | -26.18% | -28.00% | -21.42% | 2125% | 171% |
| Class D | -16.31% | -18.23% | -19.64% | -16.46% | 3290% | 177% |
| Class F | -33.58% | -36.53% | -38.28% | -34.07% | 3071% | 171% |
|  |  |  | Random Access |  |  |  |
|  |  |  | Over HM-16.20 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | -32.66% | -39.07% | -42.09% | -34.13% | 763% | 180% |
| Class A2 | -37.44% | -35.60% | -29.98% | -36.21% | 872% | 185% |
| Class B | -31.30% | -36.97% | -38.13% | -31.84% | 833% | 172% |
| Class C | -26.51% | -27.41% | -29.17% | -26.45% | 1010% | 179% |
| Class E |  |  |  |  |  |  |
| Overall | -31.52% | -34.57% | -34.90% | -31.74% | 869% | 178% |
| Class D | -25.60% | -25.16% | -26.27% | -25.66% | 1053% | 174% |
| Class F | -36.13% | -38.20% | -39.23% | -36.22% | 519% | 155% |
|  |  |  | Low delay B |  |  |  |
|  |  |  | Over HM-16.20 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -24.96% | -19.21% | -18.67% | -24.12% | 816% | 158% |
| Class C | -21.97% | -16.65% | -17.69% | -21.26% | 936% | 167% |
| Class E | -24.72% | -23.27% | -25.18% | -24.65% | 427% | 144% |
| Overall | -23.90% | -19.37% | -19.97% | -23.30% | 726% | 157% |
| Class D | -21.75% | -11.82% | -12.05% | -20.89% | 885% | 149% |
| Class F | -32.70% | -32.89% | -34.05% | -32.43% | 454% | 136% |
|  |  |  | Low delay P |  |  |  |
|  |  |  | Over HM-16.20 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -29.26% | -22.11% | -22.09% | -28.27% | 715% | 163% |
| Class C | -24.08% | -17.10% | -18.27% | -23.19% | 785% | 162% |
| Class E | -27.71% | -26.43% | -28.61% | -27.70% | 346% | 143% |
| Overall | -27.14% | -21.52% | -22.45% | -26.43% | 615% | 157% |
| Class D | -23.44% | -12.75% | -12.86% | -22.49% | 768% | 152% |
| Class F | -32.32% | -32.18% | -33.35% | -32.03% | 429% | 144% |

## Chroma QP as a function of luma QP

It is suggested to review chroma QP settings with respect to luma QP in light of this combined YUV metric. In VTM 4.0, the chroma QP value is set to the luma QP value plus 1. Additionally, a mapping function decreases the chroma QP value for larger QP values.

It is proposed to remove the offset between luma and chroma QP values because at low QP values there is no reason to use different quantizers. It is further proposed to shift the mapping curve such that chroma QP adjustments kick in at a higher QP value.

More specifically, it is proposed to modify the mapping table from:

Table 8-16 – Specification of QpC as a function of qPi for ChromaArrayType equal to 1

| qPi | < 30 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | > 43 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QpC | = qPi | 29 | 30 | 31 | 32 | 33 | 33 | 34 | 34 | 35 | 35 | 36 | 36 | 37 | 37 | = qPi − 6 |

to:

Table 8-16 – Specification of QpC as a function of qPi for ChromaArrayType equal to 1

| qPi | < 33 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | > 46 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QpC | = qPi | 32 | 33 | 34 | 35 | 36 | 36 | 37 | 37 | 38 | 38 | 39 | 39 | 40 | 40 | = qPi − 6 |

|  |  |  | All Intra |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.2 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.97% | -6.55% | -6.02% | -0.18% | 101% | 100% |
| Class A2 | 1.79% | -7.33% | -7.24% | -0.32% | 102% | 102% |
| Class B | 0.67% | -6.29% | -5.87% | -0.19% | 103% | 102% |
| Class C | 0.63% | -4.40% | -4.18% | -0.18% | 101% | 99% |
| Class E | 0.32% | -4.08% | -3.86% | -0.22% | 101% | 100% |
| Overall | 0.84% | -5.72% | -5.41% | -0.21% | 102% | 101% |
| Class D | 0.57% | -4.29% | -4.03% | -0.17% | 102% | 100% |
| Class F | 0.73% | -3.47% | -3.39% | -0.09% | 100% | 100% |
|  |  |  | Random Access |  |  |  |
|  |  |  | Over VTM-3.2 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | -0.41% | -3.56% | -4.56% | -0.31% | 102% | 100% |
| Class A2 | 0.23% | -7.31% | -7.22% | -0.88% | 100% | 101% |
| Class B | 0.26% | -8.54% | -8.31% | -0.55% | 100% | 101% |
| Class C | 0.20% | -5.93% | -5.71% | -0.48% | 100% | 100% |
| Class E |  |  |  |  |  |  |
| Overall | 0.10% | -6.60% | -6.65% | -0.55% | 100% | 100% |
| Class D | 0.19% | -6.77% | -5.97% | -0.43% | 101% | 101% |
| Class F | 0.45% | -4.89% | -4.85% | -0.60% | 101% | 100% |
|  |  |  | Low delay B |  |  |  |
|  |  |  | Over VTM-3.2 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | 0.61% | -15.65% | -15.38% | -0.95% | 99% | 99% |
| Class C | 0.66% | -11.47% | -11.20% | -0.71% | 101% | 101% |
| Class E | 0.25% | -11.47% | -12.52% | -1.00% | 100% | 101% |
| Overall | 0.54% | -13.21% | -13.27% | -0.88% | 100% | 100% |
| Class D | 0.55% | -12.71% | -13.08% | -0.75% | 102% | 101% |
| Class F | 0.90% | -8.00% | -7.14% | -0.57% | 99% | 100% |
|  |  |  | Low delay P |  |  |  |
|  |  |  | Over VTM-3.2 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | 0.61% | -16.20% | -16.02% | -0.97% | 101% | 99% |
| Class C | 0.77% | -12.22% | -12.06% | -0.71% | 102% | 103% |
| Class E | 0.23% | -12.44% | -12.82% | -1.09% | 100% | 102% |
| Overall | 0.57% | -13.93% | -13.90% | -0.91% | 101% | 101% |
| Class D | 0.63% | -13.73% | -13.33% | -0.72% | 100% | 100% |
| Class F | 0.81% | -8.46% | -8.49% | -0.68% | 100% | 102% |

The tables below show the coding results w.r.t. HM version 16.20. It should be noted that the result for luma is improved for the random access case w.r.t. results for HM 4.0 (31.55% improvement vs 31.52%) while the results for chroma are improved more significantly (e.g., 38.45% vs 34.57%).

|  |  |  | All Intra |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over HM-16.20 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | -23.06% | -42.28% | -39.54% | -25.47% | 1371% | 168% |
| Class A2 | -21.83% | -29.73% | -24.96% | -21.25% | 2223% | 176% |
| Class B | -18.64% | -28.80% | -34.52% | -19.82% | 2331% | 172% |
| Class C | -19.07% | -24.92% | -27.72% | -19.94% | 3028% | 179% |
| Class E | -22.61% | -26.80% | -29.71% | -23.29% | 1854% | 166% |
| Overall | -20.66% | -30.01% | -31.45% | -21.61% | 2159% | 173% |
| Class D | -15.89% | -21.67% | -22.61% | -16.62% | 3347% | 177% |
| Class F | -33.17% | -38.90% | -40.67% | -34.17% | 3078% | 171% |
|  |  |  | Random Access |  |  |  |
|  |  |  | Over HM-16.20 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | -33.05% | -40.41% | -44.03% | -34.29% | 778% | 180% |
| Class A2 | -37.51% | -39.73% | -34.60% | -36.74% | 868% | 186% |
| Class B | -31.18% | -41.91% | -42.91% | -32.19% | 833% | 173% |
| Class C | -26.41% | -31.67% | -33.14% | -26.80% | 1008% | 179% |
| Class E |  |  |  |  |  |  |
| Overall | -31.55% | -38.45% | -38.87% | -32.08% | 872% | 179% |
| Class D | -25.50% | -30.12% | -30.46% | -25.96% | 1058% | 175% |
| Class F | -35.95% | -41.16% | -42.06% | -36.62% | 523% | 155% |
|  |  |  | Low delay B |  |  |  |
|  |  |  | Over HM-16.20 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -24.55% | -31.63% | -30.89% | -24.82% | 809% | 156% |
| Class C | -21.49% | -26.04% | -26.83% | -21.82% | 944% | 169% |
| Class E | -24.53% | -31.58% | -34.01% | -25.36% | 425% | 145% |
| Overall | -23.53% | -29.76% | -30.32% | -23.96% | 725% | 157% |
| Class D | -21.34% | -22.87% | -23.26% | -21.47% | 905% | 151% |
| Class F | -32.14% | -38.49% | -39.04% | -32.85% | 450% | 136% |
|  |  |  | Low delay P |  |  |  |
|  |  |  | Over HM-16.20 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -28.87% | -34.52% | -34.07% | -28.93% | 723% | 162% |
| Class C | -23.53% | -27.01% | -28.01% | -23.73% | 799% | 166% |
| Class E | -27.55% | -34.96% | -37.16% | -28.41% | 347% | 147% |
| Overall | -26.76% | -32.13% | -32.82% | -27.07% | 622% | 159% |
| Class D | -22.97% | -24.48% | -23.99% | -23.03% | 771% | 153% |
| Class F | -31.81% | -38.26% | -39.38% | -32.52% | 431% | 147% |

# Patent rights declaration(s)

Sharp Labs of America may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).