| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–29 March. 2019 | Document: JVET-N0342 |
| --- | --- |

| Title: | Non-CE: Decoder-side Intra Mode Derivation with Prediction Fusion |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mohsen Abdoli Elie Mora Thomas Guionnet Mickael Raulet 12D Rue de Patis Tatelin 35700 Rennes, France | Email: | m.abdoli@ateme.com e.mora@ateme.com t.guionnet@ateme.com m.raulet@ateme.com |
| Source: | ATEME |  |  |

_____________________________

# Abstract

This contribution presents a modified version of the Decoder-side Intra Mode Derivation (DIMD) on top of the previously proposed contribution JVET-M0094 [1]. The proposed DIMD skips the intra mode transmission and performs a texture gradient processing to derive the best mode, instead. Compared to the previous version, the main modification is a “prediction fusion” step which uses up to three intra modes, implicitly derived from the texture. Like the previous version, the second and the third intra modes for prediction fusion are also derived by the texture gradient processing step. The results show a 0.41% gain on average in AI for 125% encoding time and 110% decoding time.

# Introduction

The concept of Decoder-side Intra Mode Derivation (DIMD) was first introduced in JVET-L0164 [2]. DIMD is an intra coding tool wherein the luma intra prediction mode (IPM) is not transmitted via the bitstream. Instead, it is derived using previously encoded/decoded pixels, in an identical fashion at the encoder and at the decoder. The selection of DIMD is signalled in the bitstream for intra coded blocks using a flag.

# Bitstream parsing with DIMD

## DIMD flag

A CU-level DIMD flag is sent before the multi-reference line (MRL) and Intra Sub-Partitioning (ISP) indexes.

Coding with DIMD assumes that the MRL index is 0 (always use neighboring pixels for intra prediction), and thus, the MRL index is not signaled in case the DIMD flag equals 1 (MRL index is inferred to be 0).

The proposed DIMD is integrated with ISP and can benefit from its sub-partitioning with no modification in its processing order. In other words, depending on the transmitted ISP index, a DIMD block might or might not partition its prediction block.

Furthermore, when the DIMD flag equals 1, all syntax elements used to code the IPM (MPM flag, MPM index or truncated binary codes for non-MPM) are not transmitted.

Figure 1 illustrates the steps of a VTM4 decoder integrated with the proposed DIMD. As can be seen, depending on the parsed DIMD flag, the decoding process follows two possible paths:

- Regular path that parses MRL index, ISP index, MPM flag and IPM index (flag = 0)

- DIMD path that only parses the ISP index. The latter path is followed by a mode derivation module in the beginning of the decompression phase, which is explained in the next section. (flag = 1)

Figure 1 Proposed intra block decoding process

## Integration with MPM list construction of MRL blocks

As DIMD algorithm does not provide access to the IPM during the parsing phase, DIMD blocks must provide a virtual mode to their non-DIMD neighbors during their MPM list construction. In the previous versions of DIMD, the Planar mode was used as the virtual mode.

However, since VTM3, the MPM list construction have gone through major changes. More precisely, in the current specification of MRL, the MPM list must only consist of angular modes when the MRL index is greater than zero. To integrate with this modification, proposed DIMD blocks are forced to provide an angular virtual mode to their non-DIMD neighbors with MRL index greater than zero. To choose this angular virtual mode, DIMD blocks construct their MPM list and use the first angular mode that appears in it.

Figure 2 demonstrates the proposed integration of DIMD with the MPM list construction of blocks with MRL index greater than zero.

(a) (b) (c). Figure 2 (a) A DIMD block next to the current non-DIMD block with MRL index greater that zero. (b) Building the MPM list of the DIMD block. (c) Picking the first angular mode to provide as the virtual mode to the mode coding of the current block.

# Pixel prediction with DIMD

## Texture gradient analysis

To implicitly derive the IPM of a DIMD blocks, a texture gradient analysis is performed at both encoder and decoder sides. This process starts with an empty Histogram of Gradient (HoG) with 65 entries, corresponding to the angular modes of VTM4. Amplitudes of these entries are determined during the texture gradient analysis.

In the first step, DIMD picks a template of T=3 columns and lines from respectively left and above current block, as shown in Figure 3-(a). This area will be used as the reference for the gradient-based IPM derivation.

In the second step, the horizontal and vertical Sobel filters are applied on all 3×3 window positions, centered on the pixels of the middle line of the template (Figure 3-(b)). On each window position, Sobel filters calculate the intensity of pure horizontal and vertical directions as Ghor and Gver, respectively. Then, the texture angle of the window is calculated as:

angle=arctan⁡(Ghor/Ghor),                                                         (1)

which can be converted into one of 65 angular IPMs of VTM4. Once the IPM index of current window is derived as idx, the amplitude of its entry in the HoG[idx] is updated by addition of:

ampl = Ghor+Ghor                                                            (2)

Figure 3-(c) shows an example of HoG, calculated after applying the above operations on all pixel positions in the template.

Figure 3

(a) Template selection from the reconstructed area with T lines of pixels (b) applying horizontal and vertical Sobel filter with T=3 on all pixels in the middle row (c) building the HoG for angular IPMs.

## Prediction fusion

Unlike the previous version of DIMD, where one single IPM corresponding to tallest spike of HoG was used, here DIMD can use up to three IPMs.

Figure 4 demonstrates the fusion algorithm of DIMD. As can be seen, three IPMs corresponding to the three tallest spikes of the HoG are detected as M1, M2 and M3. After applying the pixel prediction by these three IPMs and obtaining Pred1, Pred2 and Pred3, their fusion is computed as the final predictor of the block. For this purpose, a weighted average of the three predictors are calculated with respect to the amplitude of their spikes in the HoG.

Figure 4 Prediction fusion by the proposed DIMD, using three angular modes corresponding to the first three tallest spikes of the HoG.

# Experiments

## Test environment

According to the common test conditions [3], the performance of the proposed DIMD method is evaluated for intra-only (AI) configuration on top of VTM-4.0 [4]. The corresponding simulations were conducted on an Intel Xeon Platinum 8180 machine (AVX2 on, turbo boost on, hyperthreading on) with CentOS Linux 7 and a GCC 4.8.5 compiler.

## Results

Table 1-3 show the BD-Rate performance of the proposed tests on top of VTM4.

Table 1: Results of the proposed DIMD with prediction fusion in the AI configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.53% | -0.35% | -0.35% |  |  |
| Class A2 | -0.29% | -0.09% | -0.09% |  |  |
| Class B | -0.34% | -0.18% | -0.18% |  |  |
| Class C | -0.40% | -0.33% | -0.33% |  |  |
| Class E | -0.51% | -0.27% | -0.26% |  |  |
| Overall | -0.41% | -0.24% | -0.23% |  |  |
| Class D | -0.48% | -0.44% | -0.47% |  |  |
| Class F |  |  |  |  |  |

# Cross-checks

Cross-check for the above test have been performed by Technicolor [5].

# Conclusions

The proposed DIMD algorithm with prediction fusion brings 0.41% BD-Rate gain on top of VTM4. The complexity at the encoder and decoder sides are 125% and 110%, respectively.

The integration of DIMD with respect to MPM list coding is tuned. This integration may be refined depending on the outcome of the currently ongoing MRL/ISP unification.

- References

- E. Mora, A. Nasrallah, M. Abdoli, M. Raulet, CE3: Decoder-side Intra Mode Derivation (tests 3.1.1, 3.1.2, 3.1.3 and 3.1.4), JVET-M0094, Marrakech, January 2019

- E. Mora, A. Nasrallah, and M. Raulet, “CE3-related : Decoder-side Intra Mode Derivation”, JVET-L0164, Macao, October 2018

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-M1010, Marrakech, January 2019

- VTM-4.0 software, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM.git, February 2019

- JVET-Nxxxx - Cross-check by Technicolor (TBA)

# Patent rights declaration(s)

ATEME may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).