| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0344 |
| --- | --- |

| Title: | CE8: Palette Mode in HEVC (CE8-2.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yung-Hsuan Chao, Vadim Seregin, Marta Karczewicz (Qualcomm) Yu-Chen Sun, Tsui-Shan Chang, Jian Lou (Alibaba) | Tel: Email: | yunghsua@qti.qualcomm.com +1-858-349-2818 yuchen.s@alibaba-inc.com |
| Source: | Qualcomm Incorporated Alibaba Cloud Computing Ltd. |  |  |

# Abstract

This document reports the results of CE8-2.1 on top of VTM-4.0 when IBC is enabled and disabled. The syntax of the proposed palette mode is the same as in HEVC SCC. Both QP range specified in common test condition and low QP range are tested. The CE results show that for CTC QP range, the proposed method provides 6.85% (AI), 3.30% (RA), and 0.89% (LDB) luma BD-rate gains for class TGM and 2.38% (AI), 1.44% (RA), and 0.85% (LDB) luma BD-rate gains for class F when IBC is enabled. For IBC disabled, the proposed method provides 27.12% (AI), 13.17% (RA), and 3.43% (LDB) luma BD-rate gains for class TGM and 8.58% (AI), 6.72% (RA), and 3.11% (LDB) luma BD-rate gains for class F.

Additional low QP results are also provided in this proposal. When IBC is enabled, the proposed method provides 10.24% (AI), 7.04% (RA), and 4.96% (LDB) luma BD-rate gains for class TGM and 1.06% (AI), 1.04% (RA), and 0.37% (LDB) luma BD-rate gains for class F. For IBC disabled, the proposed method provides 32.58% (AI), 16.98% (RA), and 12.46% (LDB) luma BD-rate gains for class TGM and 4.27% (AI), 3.55% (RA), and 2.21% (LDB) luma BD-rate gains for class F

# Proposal

The decoder syntax in this CE test is the same as in HEVC palette. A flag is transmitted for each CU to signal whether the palette mode is used in the current CU. If the palette mode is utilized, the pixels values in the CU are represented by a small set of representative colour values. The set is referred to as the palette. For pixels with values close to the palette colors, the palette indices are signalled. For pixels with values outside the palette, the pixel is denoted with an escape symbol and the quantized pixel values are signaled directly.

To decode a palette encoded block, the decoder needs to decode palette colors and indices. Palette colors are described by a palette table and encoded by palette table coding tools. An escape flag is signaled for each CU to indicate if escape symbols are present in the current CU. If escape symbols are present, the palette table is augmented by one and the last index is assigned to the escape mode. Palette indices of all pixels in a CU form a palette index map and are encoded by palette index map coding tools.

For coding of the palette table, a palette predictor is maintained. The predictor is initialized at the beginning of each slice where predictor is reset to 0. For each entry in the palette predictor, a reuse flag is signalled to indicate whether it is part of the current palette. The reuse flags are sent using run-length coding of zeros. After this, the number of new palette entries are signalled using exponential Golomb code of order 0. Finally, the component values for the new palette entries are signalled. After encoding the current CU, the palette predictor will be updated using the current palette, and entries from the previous palette predictor which are not reused in the current palette will be added at the end of new palette predictor until the maximum size allowed is reached (palette stuffing).

For coding the palette index map, the indices are coded using horizontal and vertical traverse scans as shown in 1. The scan order is explicitly signalled in the bitstream using the palette_transpose_flag.

Figure 1: Horizontal and vertical traverse scans

The palette indices are coded using two main palette sample modes: 'INDEX' and 'COPY_ABOVE'. The mode is signalled using a flag except for the top row when horizontal scan is used, the first column when the vertical scan is used, or when the previous mode was 'COPY_ABOVE'. In the 'COPY_ABOVE' mode, the palette index of the sample in the row above is copied. In the 'INDEX' mode, the palette index is explicitly signalled. For both 'INDEX' and 'COPY_ABOVE' modes, a run value is signalled which specifies the number pixels that are coded using the same mode.

The encoding order for index map is as follows: First, the number of index values for the CU is signalled. This is followed by signalling of the actual index values for the entire CU using truncated binary coding. Both the number of indices as well as the index values are coded in bypass mode. This groups the index-related bypass bins together. Then the palette mode (INDEX or COPY_ABOVE) and run are signalled in an interleaved manner. Finally, the component escape values corresponding to the escape samples for the entire CU are grouped together and coded in bypass mode. An additional syntax element, last_run_type_flag, is signalled after signalling the index values. This syntax element, in conjunction with the number of indices, eliminates the need to signal the run value corresponding to the last run in the block.

In VTM4.0, dual tree is enabled for I slice which separate the coding unit partitioning for Luma and Chroma. In this case, two separated palettes are applied and signaled for luma (Y component) and chroma (Cb and Cr components). If dual tree is disabled (i.e. non-I slice or when dual tree is disabled in SPS), one palette will be applied on Y, Cb, Cr components jointly, same as in HEVC palette.

# Encoder Speedup

In this CE test, encoder speedup approaches proposed in [2] and [3] are applied:

- Palette mode may be bypassed on the encoder side based on the history RD cost information. Specifically, on the first visit of a CU, the RD cost of palette mode is stored. On the successive visit of the same CU, palette mode will be bypassed if the stored RD cost is larger than the cost of the best mode (among the modes checked before palette mode)

- For non-intra slice, intra mode will be bypassed if the RD cost of palette is the smallest among all the modes checked before intra mode.

- For intra slice, palette mode is checked before intra mode on encoder side. The RD cost of palette is compared with the first intra mode (in the loop of full RD checking). If palette mode has lower cost than the first intra mode, the rest of the intra modes will be skipped. Otherwise, intra mode checking will be continued until all intra modes selected for full RD check are processed.

- For AI configuration, the ratio of area using palette mode is checked in the first I slice. If the area using palette mode is smaller than 1% of the total area, palette will be disabled for the rest of the sequence. For RA and LD configuration, the ratio of area using palette mode will be checked for each IRAP frame.  If the area using palette mode is smaller than 1% of the total area in the IRAP frame, palette mode will be disabled until the next IRAP frame is processed.

# Simulation results

The proposed method is evaluated according to the CE8 description [1] for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations with the VTM4.0 software. Besides the QP range specified for CTC, additional low QP results are also provided (encoding/decoding time may not be accurate).

The proposed method is tested with/without IBC. Table 1 and 2 shows the results of CE8.2.1 compared with VTM3.0 when IBC is turned on and off in the setting for CTC QP range. Table 3 and 4 shows the results of CE8.2.1 compared with VTM3.0 when IBC is turned on and off in the setting for low QP range

Table 1:Results of the CE8.2.1 for QP range in CTC (IBC is turned on)

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM4.0 (IBC=1) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | 0.00% | 0.02% | 104% | 108% |
| Class A2 | 0.06% | 0.02% | 0.01% | 103% | 102% |
| Class B | 0.05% | 0.05% | 0.03% | 102% | 102% |
| Class C | 0.06% | 0.03% | 0.01% | 103% | 107% |
| Class E | 0.07% | -0.02% | 0.18% | 103% | 104% |
| Overall | 0.06% | 0.02% | 0.05% | 103% | 104% |
| Class D | 0.09% | 0.06% | 0.05% | 104% | 106% |
| Class F | -2.38% | -2.20% | -2.28% | 100% | 102% |
| TGM | -6.85% | -5.66% | -5.29% | 91% | 94% |
|  | Random Access |  |  |  |  |
|  | Over VTM4.0 (IBC=1) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | -0.23% | 0.02% | 102% | 101% |
| Class A2 | 0.04% | -0.01% | 0.06% | 104% | 103% |
| Class B | 0.06% | -0.16% | -0.39% | 104% | 102% |
| Class C | 0.04% | 0.31% | 0.27% | 104% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | -0.02% | -0.04% | 103% | 102% |
| Class D | 0.07% | 0.24% | -0.16% | 105% | 100% |
| Class F | -1.44% | -2.13% | -2.07% | 103% | 101% |
| TGM | -3.30% | -3.16% | -3.03% | 100% | 100% |
|  | Low delay B |  |  |  |  |
|  | Over VTM4.0 (IBC=1) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | 0.31% | -0.02% | 104% | 99% |
| Class C | 0.00% | -0.04% | 0.08% | 106% | 105% |
| Class E | -0.03% | 0.14% | 0.27% | 103% | 100% |
| Overall | 0.01% | 0.15% | 0.08% | 104% | 101% |
| Class D | -0.03% | 0.81% | 0.02% | 104% | 103% |
| Class F | -0.85% | -2.34% | -2.75% | 108% | 103% |
| TGM | -0.89% | -0.55% | -0.46% | 101% | 102% |

Table 2:Results of the CE8.2.1 for QP range in CTC (IBC is turned off)

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM4.0 (IBC=0) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | 0.05% | 0.06% | 102% | 102% |
| Class A2 | 0.05% | 0.13% | 0.03% | 101% | 100% |
| Class B | 0.05% | 0.04% | 0.04% | 103% | 105% |
| Class C | 0.08% | 0.03% | 0.03% | 102% | 110% |
| Class E | 0.09% | 0.07% | 0.06% | 102% | 109% |
| Overall | 0.06% | 0.06% | 0.04% | 102% | 105% |
| Class D | 0.08% | -0.07% | -0.08% | 103% | 105% |
| Class F | -8.58% | -7.71% | -7.86% | 94% | 99% |
| TGM | -27.12% | -25.34% | -25.16% | 75% | 81% |
|  | Random Access |  |  |  |  |
|  | Over VTM4.0 (IBC=0) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.02% | 0.04% | 104% | 100% |
| Class A2 | 0.01% | -0.08% | -0.02% | 104% | 102% |
| Class B | 0.05% | -0.03% | -0.15% | 103% | 103% |
| Class C | 0.08% | -0.06% | 0.06% | 105% | 110% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | -0.04% | -0.03% | 104% | 104% |
| Class D | 0.12% | -0.16% | -0.32% | 105% | 106% |
| Class F | -6.72% | -7.66% | -7.47% | 103% | 103% |
| TGM | -13.17% | -13.41% | -13.18% | 99% | 100% |
|  | Low delay B |  |  |  |  |
|  | Over VTM4.0 (IBC=0) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.39% | -0.15% | 100% | 99% |
| Class C | 0.05% | 0.41% | 0.11% | 102% | 102% |
| Class E | 0.12% | 0.93% | -0.58% | 102% | 102% |
| Overall | 0.05% | 0.21% | -0.17% | 102% | 101% |
| Class D | 0.02% | 0.21% | -0.79% | 104% | 103% |
| Class F | -3.11% | -3.91% | -3.84% | 103% | 100% |
| TGM | -3.43% | -2.99% | -2.96% | 103% | 101% |

Table 3: Results of the CE8.2.1 for low QP (IBC is turned on)

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM4.0 (IBC=1) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.04% | -0.05% | 103% | 104% |
| Class A2 | 0.01% | 0.01% | 0.01% | 101% | 105% |
| Class B | -0.17% | -0.13% | -0.13% | 105% | 103% |
| Class C | -0.11% | -0.14% | -0.11% | 105% | 105% |
| Class E | -1.00% | -1.81% | -2.12% | 101% | 96% |
| Overall | -0.24% | -0.37% | -0.42% | 103% | 102% |
| Class D | -0.03% | -0.20% | -0.15% | 105% | 106% |
| Class F | -1.06% | -1.13% | -1.19% | 104% | 101% |
| TGM | -10.24% | -8.16% | -7.98% | 94% | 91% |
|  | Random Access |  |  |  |  |
|  | Over VTM4.0 (IBC=1) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.01% | -0.02% | 102% | 101% |
| Class A2 | 0.02% | 0.01% | 0.01% | 102% | 102% |
| Class B | -0.03% | -0.03% | 0.00% | 104% | 104% |
| Class C | -0.04% | -0.07% | -0.02% | 104% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.03% | -0.01% | 103% | 102% |
| Class D | -0.04% | -0.15% | -0.06% | 104% | 101% |
| Class F | -1.04% | -0.90% | -0.92% | 104% | 98% |
| TGM | -7.04% | -4.05% | -4.04% | 97% | 97% |
|  | Low delay B |  |  |  |  |
|  | Over VTM4.0 (IBC=1) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 96% | 98% |
| Class C | -0.02% | -0.04% | -0.06% | 106% | #NUM! |
| Class E | -0.04% | -0.16% | -0.25% | 106% | #NUM! |
| Overall | -0.02% | -0.05% | -0.08% | 102% | #NUM! |
| Class D | 0.01% | 0.01% | 0.00% | 105% | 102% |
| Class F | -0.37% | -0.19% | -0.27% | 108% | #NUM! |
| TGM | -4.96% | -2.78% | -2.73% | 104% | #NUM! |

Table 4: Results of the CE8.2.1 for low QP (IBC is turned off)

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM4.0 (IBC=0) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.02% | -0.06% | #DIV/0! | #DIV/0! |
| Class A2 | 0.01% | 0.01% | 0.01% | #DIV/0! | #DIV/0! |
| Class B | -0.18% | -0.12% | -0.13% | #DIV/0! | #DIV/0! |
| Class C | -0.11% | -0.15% | -0.12% | #DIV/0! | #DIV/0! |
| Class E | -1.01% | -1.82% | -2.08% | #DIV/0! | #DIV/0! |
| Overall | -0.25% | -0.37% | -0.42% | #DIV/0! | #DIV/0! |
| Class D | -0.03% | -0.22% | -0.22% | #DIV/0! | #DIV/0! |
| Class F | -4.27% | -6.02% | -6.02% | #DIV/0! | #DIV/0! |
| TGM | -32.58% | -32.45% | -32.50% | #DIV/0! | #DIV/0! |
|  | Random Access |  |  |  |  |
|  | Over VTM4.0 (IBC=0) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.02% | -0.01% | #DIV/0! | #DIV/0! |
| Class A2 | 0.02% | 0.00% | 0.01% | #DIV/0! | #DIV/0! |
| Class B | -0.03% | -0.02% | 0.00% | #DIV/0! | #DIV/0! |
| Class C | -0.04% | -0.11% | -0.10% | #DIV/0! | #DIV/0! |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.04% | -0.03% | #DIV/0! | #DIV/0! |
| Class D | -0.06% | -0.08% | -0.13% | #DIV/0! | #DIV/0! |
| Class F | -3.55% | -4.61% | -4.63% | #DIV/0! | #DIV/0! |
| TGM | -16.98% | -15.95% | -15.80% | #DIV/0! | #DIV/0! |
|  | Low delay B |  |  |  |  |
|  | Over VTM4.0 (IBC=0) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.02% | 0.00% | #DIV/0! | #DIV/0! |
| Class C | -0.01% | 0.00% | 0.03% | #DIV/0! | #DIV/0! |
| Class E | -0.02% | -0.12% | -0.11% | #DIV/0! | #DIV/0! |
| Overall | -0.01% | -0.02% | -0.02% | #DIV/0! | #DIV/0! |
| Class D | -0.01% | -0.01% | -0.04% | #DIV/0! | #DIV/0! |
| Class F | -2.21% | -2.64% | -2.80% | #DIV/0! | #DIV/0! |
| TGM | -12.46% | -11.01% | -10.91% | #DIV/0! | #DIV/0! |

# Reference

- X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu, “Description of Core Experiment 8 (CE8): Screen Content Coding Tools”, JVET-M1028, 13th meeting, Marrakech, MA, January 2019.

- Y.-H. Chao, H. Wang, V. Seregin, M. Karczewicz, Y.-C. Sun, J. An, J. Lou, “CE15-2: Palette mode of HEVC SCC”, JVET-L0336, 12th meeting, Macao, CN, October 2018.

- H. Wang, Y.-H Chao, V. Seregin, M. Karczewicz, “Non-CE8: Encoder optimization for palette mode”, JVET-M0822, 13th meeting, Marrakech, MA, January 2019.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Alibaba Cloud Computing Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).