| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0355 |
| --- | --- |

| Title: | CE12-related: Normative temporally independent tile group coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Robert Skupin Yago Sanchez Valeri George Karsten Sühring Thomas Schierl | Email: | {forename.surname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document proposes normative changes to allow temporally independent coding of tile groups in a coded video. The proposed changes include that when MVs point to disallowed sample positions near or beyond the boundaries of temporally independent tile groups, the following is carried out:

- MV clipping

- MV sub-pel component rounding

- Sub-block division

- MV predictor clipping and/or rounding for AMVP

- Disabling BDOF and DMVR next to boundaries of temporally independent tile groups

- TMVP and ATMVP handle boundaries of temporally independent tile groups similar to picture boundaries.

The document also reports coding results of the proposed scheme following the test conditions used in CE12.

Revision 1 of the document includes corrected results for the proposal and a correct the CE12 anchor (i.e. with BIO enabled).

# Introduction

CE12 investigates proposals for enabling temporally independent tile group coding in a coded video. Such coding modes are a key enabler in RoI based applications such as viewport-dependent streaming of 360° video. Tiles coded in such a way can be separated and re-combined depending on the application needs.

In AVC and HEVC, the selected techniques for temporally independent tile group coding are encoder-side constraints wherein the encoder is prohibited to choose coding modes (predictor usage, motion vectors, etc.) that introduce dependencies between different temporally independent; in HEVC referred to as MCTS. CE12-2 tests such encoder-side restrictions for VVC.

Another option is to introduce normative decoder changes to enable more efficient coding in such scenarios. In test CE12-1, an example of such an approach is investigated. Boundaries of temporally independent tile groups are to be padded with perpendicular padding similar to the treatment of picture boundaries. Motion compensated inter-prediction may then use the area outside a temporally independent tile group and reach into the boundary padding area. However, in many software implementations of AVC and HEVC, particularly also in VTM, padding samples are generated in a static fashion by increasing the DPB memory. For traditional padding of the picture boundary, the increase per picture is equal to the maximum block size (128 samples for VVC) plus the interpolation filter kernel reach (8 samples for VVC) in each direction and clipping further reaching Motion Vectors. As the number of padded boundaries for temporally independent tile groups increases significantly (e.g. 96 tiles in CE12 test conditions) such an approach incurs an infeasible implementation overhead.

# Proposal

This document proposes normative changes for enabling temporally independent tile group coding in different way than CE12-1 (boundary padding around tile groups). The proposed changes can be characterized as changing inter-prediction to be aware of tile group boundary location and introduce

- MV clipping and/or rounding

- Sub-block division

- MV predictor clipping and/or rounding

- Disabling BDOF and DMVR next to boundaries of temporally independent tile groups

In addition, TMVP and ATMVP handle boundaries of temporally independent tile groups similar to picture boundaries.

In more detail, for this purpose, a restricted area for sub-pel and integer pel sample positions is defined as follows:

- Sub-pel sample position inside the temporally independent tile group that require samples from outside the tile group for sub-pel interpolation are disallowed as illustrated in dark blue in Figure 1.

- All sample positions that are located beyond the boundaries of a temporally independent tile group as illustrated in light blue in Figure 1 are disallowed.

Figure 1: Restricted area at temporally independent tile group boundaries.

When MVs point to sample positions of the restricted area the following restrictions are enabled:

MV clipping

MV components are clipped when the MV points into the restricted area in order to only use values of samples at allowed sample positions. When an MV points to a sample position outside the tile group, the MV is clipped to the nearest allowed integer-pel position as follows:

MVX=Clip3((TileGroupBoundaryLeft–Posx) << 4,(TileGroupBoundaryRight–Posx–(BW–1)) << 4,MVX)

MVY=Clip3((TileGroupBoundaryTop–Posy) << 4,(TileGroupBoundaryBottom–Posy–(BH–1)) << 4,MVY)

MV sub-pel component rounding

When an MV points to a disallowed sub-pel sample position in the temporally independent tile group, the sub-pel MV component is rounded to the nearest allowed integer-pel position inside the tile group.

Sub-block division

When the MV associated with a CU points into the restricted area and the CU has a certain size (e.g. larger than 8x8 samples), the CU is further divided into sub-blocks of 4x4 samples which are individually processed (e.g. MV clipping and rounding).

MV predictor clipping and/or rounding

In case of AMVP, when the chosen MV predictor points into the restricted area, the MV predictor is clipped as discussed above and for blocks <8x8, the sub-pel component is rounded to the nearest integer-pel position in addition.

Disabling tools in restricted area

BDOF and DMVR are disabled when MVs point into the restricted area.

In addition, another part of the proposal is that CTUs on boundaries of temporally independent tile groups behave similar to CTUs on picture boundary, i.e. for TMVP, the center-collocated candidate instead of the right-bottom collocated is chosen and for ATMVP, the collocated sub-block is restricted to be in the same CTU.

# Experimental results

The test conditions are equal to CE12. In detail, the 360° video test sequences SkateboardInLot, ChairLift, KiteFlite, Harbor, Trolley and GasLamp are used in CMP format with a face size of 1536x1536 and a total resolution of 4608x3072 equivalent to 6k. Two tiling setups were investigated, namely 6x4 and 12x8 uniformly sized tiles, which results in tile sizes of 768x768 (i.e. 24 tiles) or 384x384 (i.e. 96 tiles) respectively. ALF and reshaper are disabled for the tests. Furthermore, for the test, each of the tiles is packaged into an individual tile group while for the anchor, all tiles are packaged into a single tile group per picture.

Revision 1 of the document includes corrected results for the proposal and a correct the CE12 anchor (i.e. with BIO enabled).

6x4 Tiles, RA

| Test sequence | Y-PSNR | U-PSNR | V-PSNR |
| --- | --- | --- | --- |
| Chairlift | 5,67% | 6,91% | 6,37% |
| GasLamp | 2,62% | 1,95% | 2,48% |
| Harbor | 1,52% | 1,70% | 0,79% |
| KiteFlite | 1,01% | 1,63% | 1,57% |
| SkateboardInLot | 8,68% | 10,95% | 12,13% |
| Trolley | 0,85% | 1,00% | 1,42% |
| Average | 3,39% | 4,02% | 4,13% |

12x8 Tiles, RA

| Test sequence | Y-PSNR | U-PSNR | V-PSNR |
| --- | --- | --- | --- |
| Chairlift | 19,26% | 23,17% | 21,44% |
| GasLamp | 11,10% | 10,98% | 11,40% |
| Harbor | 8,33% | 9,13% | 8,72% |
| KiteFlite | 3,83% | 5,11% | 5,07% |
| SkateboardInLot | 20,67% | 26,13% | 29,48% |
| Trolley | 4,40% | 5,12% | 5,93% |
| Average | 11,27% | 13,27% | 13,67% |

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).