| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0357-v2 |
| --- | --- |

| Title: | CE8-related: Context Modelling of Sign for TS Residual Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | B. Bross T. Nguyen H. Schwarz D. Marpe T. Wiegand Einsteinufer 37 10587 Berlin Germany | Tel: Email: | - benjamin.bross@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

The compression efficiency of screen content can be improved when using a modified residual coding (transform coefficient level coding) for the transform skip mode (TSM) as proposed in the input document JVET-M0464. One part of the design is the coding of the sign syntax element using context models instead of the bypass coding mode as can be found in the residual coding for regular transformed signals. This document describes a simple context modelling for the sign syntax element of the approach in the input document JVET-M0464. An improvement in terms of BD-rate relative to the approach in JVET-M0464 is about -0.26 for Class F and -0.42 for TGM for the AI configuration, and -0.25 for Class F and -0.20 for TGM in the RA configuration, respectively, in the IBC enabled configuration (CTC) without a context coded bins limitation. A similar performance can be observed when a context coded bins limitation is enabled.

# Introduction

The residual coding for the transform skip mode proposed in the input document JVET-M0464 employs a single context model for the coding of the sign syntax element. In contrast to transformed signals, the statistics of the sign syntax element is locally biased, and thus, suitable for the coding with context models. This document proposes to extend the context modelling so that three context models are employed in total for the sign syntax element as follows.

# Proposed Context Modelling

The proposed approach employs three context models as follows. For each transform block using TSM, the number of sign syntax elements equal to one and the total number of sign syntax element are tracked. The relative ratio r between the two is updated and evaluated resulting in three context models: the first context model is employed for the case r < a, the second context model is employed for the case r > b, and the third (and initial) context model is employed for the case where a ≤ r ≤ b, with a = 0.05 and b = 0.95, respectively.

# Experimental Results

The results of the residual coding for transform skip mode including the sign context modelling are as follows in the IBC=1 configuration.

Class F: 	−3.07 % (AI), −2.23 % (RA), −1.69 % (LB); 
TGM: 		−4.88 % (AI), −3.25 % (RA), −2.61 % (LB);

The results for the residual coding for transform skip mode without the sign context modelling are as follows in the IBC=1 configuration.

Class F: 	−2.81 % (AI), −1.98 % (RA), −1.61 % (LB); 
TGM: 		−4.46 % (AI), −3.05 % (RA), −2.43 % (LB);

Comparing the two results, the proposed context modeling improves the compression efficiency by -0.26 for Class F and -0.42 for TGM for the AI configuration, and -0.25 for Class F and -0.20 for TGM in the RA configuration.

Similar observation can be found for the case with IBC=0. The results with and without the proposed modifications are as follows.

Class F: 	−5.89 % (AI), −4.00 % (RA), −2.68 % (LB); 
TGM: 		−15.2 % (AI), −7.79 % (RA), −3.85 % (LB);

Class F: 	−5.59 % (AI), −3.79 % (RA), −2.47 % (LB); 
TGM: 		−14.6 % (AI), −7.54 % (RA), −3.70 % (LB);

Additional results for the configuration with a context coded bins limitation equal to two and three, respectively, are as follows, using the bitstream restriction approach.

Class F: 	−2.81 % (AI), −1.92 % (RA), −1.75 % (LB); 
TGM: 		−4.26 % (AI), −2.78 % (RA), −2.44 % (LB);

Class F: 	−3.03 % (AI), −2.14 % (RA), −1.66 % (LB); 
TGM: 		−4.78 % (AI), −3.15 % (RA), −2.64 % (LB);

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).