| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0373_v2 |
| --- | --- |

| Title: | Non-CE4: HMVP unification between the Merge and MVP list |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hahyun Lee, Jungwon Kang, Sung-Chang Lim, Jinho Lee, Hui Yong Kim 218 Gajeong-ro, Yuseong-gu, Daejeon, 34129, KOREA | Tel: Email: | +82-42-860-6138 {hanilee, jungwon, sclim, jinosoul, hykim5}@etri.re.kr |
| Source: | ETRI (Electronics and Telecommunications Research Institute) |  |  |

_____________________________

- Abstract

This contribution proposes two unification methods of using HMVP candidates in the both merge candidates list and MPV candidates list construction. 1) Same as AMVP, the HMVP candidates, from the first entry to the last entry in the HMVP table, are added in the merge list without redundant check. Also, 2) at most 5 HMVP candidates can be checked in MVP list construction. The proposal 1) achieved 0.04% and 0.01% loss in terms of average Luma BD-rate in RA and LDB, respectively. And the proposal 2) achieved 0.00% and -0.02% gain in terms of average Luma BD-rate in RA and LDB, respectively. Both proposal 1) and proposal 2) achieved 0.04% and 0.01% loss in terms of average Luma BD-rate in RA and LDB, respectively. It is asserted by the proponent that the methods have marginal impact on the BD-rate performance while a unification of using HMVP candidates in the merge and MVP list is achieved.

Introduction

At the 13th JVET meeting, regarding History-based Motion Vector Prediction (HMVP), the followings are adopted in the Versatile Video Coding (VVC) Working Draft (WD) 4 [1].

Firstly, it was adopted that a partial pruning is used when adding a new HMVP candidate in the merge list. This means that the first 2 HMVP candidates, from the last entry (H4) to the first entry (H0) in the HMVP table, are pruned to the left and above spatial candidates as shown in the Figure 1. At most 5 HMVP candidates can be available in the merge list.

- HMVP candidates pre-defined order (VVC WD4)

- The first 2 HMVP candidates are compared with the spatial candidates A1 and B1

Figure 1. HMVP candidate pre-defined order and pruned with spatial candidates in the merge list construction

Secondly, it was also adopted that if the MVP list is still not full, the HMVP candidates, from the first entry (H0) to the last entry (H4) in the HMVP table as shown in the Figure 2, are added in the MVP list without redundancy check. The adding of HMVP candidates stops when the maximum number of checked HMVP candidates reaches a threshold (= 4 in VTM-4) or the MVP list is full.

Figure 2. HMVP candidate pre-defined order and maximum threshold in MPV list construction (VVC WD4)

Proposals

This contribution proposes the unification methods of using HMVP candidate in the both merge and MPV candidate list.

Change the scan order of HMVP candidates and remove a partial pruning with spatial candidates in merge list

The HMVP candidates, from the first entry to the last entry in the HMVP table (i.e. the most least HMVP candidate is checked first), are added in the merge list as shown in the Figure 3. Since the motivation of changing the scan order of HMVP candidate is to first check candidates that are more likely to be different from the existing candidate in the merge list, a partial pruning (the maximum number of pruning is 4) is removed.

Figure 2. HMVP candidate predefined order (H0 → H4) in merge list

Remove the maximum HMVP candidate threshold in AMVP mode

In MVP list construction, the maximum HMVP candidate threshold (=4 in VTM-4.0) is removed. This means that if the MVP is still not full, at most 5 HMVP candidates can be checked in MVP list construction.

Experimental results

The proposed methods were integrated on VTM-4.0 and the experiments were performed under JVET common test conditions [2]. The experiments were conducted on the platform of Intel Xeon E5-2690 v2 (3.0GHz) and 64GB RAM with GCC 7.3.1 compiler under CentOS Linux.

Proposal 2.1

Simulation results as below, the anchor is VTM-4.0 with default setting and the HMVP candidates, from the first entry to the last entry in the HMVP table, are added in the merge list and a partial pruning with A1 and B1 is removed.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.17% | 0.09% | 100% | 100% |
| Class A2 | 0.01% | -0.10% | 0.07% | 100% | 100% |
| Class B | 0.04% | 0.05% | 0.03% | 100% | 100% |
| Class C | 0.06% | 0.06% | 0.02% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | 0.05% | 0.05% | 100% | 100% |
| Class D | 0.04% | -0.05% | 0.04% | 100% | 100% |
| Class F | 0.07% | -0.03% | -0.06% | 100% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.03% | -0.32% | 100% | 100% |
| Class C | -0.02% | 0.47% | 0.08% | 100% | 100% |
| Class E | 0.04% | 0.54% | 0.12% | 100% | 100% |
| Overall | 0.01% | 0.28% | -0.08% | 100% | 100% |
| Class D | -0.02% | 0.06% | -0.06% | 100% | 100% |
| Class F | -0.08% | 0.25% | -0.17% | 101% | 100% |

Proposal 2.2

Simulation results as below, the anchor is VTM-4.0 with default setting and the maximum HMVP candidate threshold (=4 in VTM-4.0) is removed in MPV list construction.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.01% | 0.01% | 99% | 100% |
| Class A2 | -0.01% | 0.09% | 0.02% | 99% | 100% |
| Class B | 0.00% | 0.02% | 0.08% | 100% | 100% |
| Class C | 0.00% | 0.00% | -0.06% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.02% | 0.02% | 100% | 100% |
| Class D | 0.00% | -0.07% | -0.06% | 100% | 100% |
| Class F | 0.00% | 0.04% | -0.01% | 100% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.08% | -0.24% | 100% | 100% |
| Class C | -0.03% | 0.28% | -0.27% | 100% | 100% |
| Class E | -0.02% | 0.06% | -0.24% | 100% | 100% |
| Overall | -0.02% | 0.07% | -0.25% | 100% | 100% |
| Class D | -0.02% | 0.16% | -0.15% | 100% | 100% |
| Class F | 0.03% | 0.05% | -0.34% | 100% | 101% |

Proposal 2.1 + Proposal 2.2

Simulation results as below, the anchor is VTM-4.0 with default setting and the combination of proposal 2.1 and proposal 2.2 is tested.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.18% | 0.07% | 100% | 100% |
| Class A2 | 0.02% | -0.01% | 0.11% | 100% | 100% |
| Class B | 0.05% | 0.02% | 0.00% | 100% | 100% |
| Class C | 0.05% | -0.04% | -0.01% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | 0.03% | 0.03% | 100% | 100% |
| Class D | 0.03% | -0.11% | -0.05% | 100% | 100% |
| Class F | 0.07% | -0.05% | -0.07% | 100% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.00% | -0.34% | 100% | 100% |
| Class C | 0.00% | 0.23% | 0.13% | 100% | 100% |
| Class E | 0.02% | 0.47% | -0.13% | 100% | 100% |
| Overall | 0.01% | 0.19% | -0.13% | 100% | 100% |
| Class D | -0.03% | 0.38% | -0.52% | 100% | 100% |
| Class F | 0.02% | -0.12% | 0.22% | 100% | 101% |

Supplemental result

Simulation results as below, the anchor is VTM-4.0 with default setting and only a partial pruning with A1 and B1 is removed without changing scan order of HMVP candidate in merge list.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.13% | 0.11% | 0.22% | 99% | 99% |
| Class A2 | 0.19% | 0.18% | 0.26% | 100% | 99% |
| Class B | 0.20% | 0.14% | 0.16% | 100% | 99% |
| Class C | 0.16% | 0.14% | 0.03% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.17% | 0.14% | 0.16% | 100% | 99% |
| Class D | 0.12% | 0.11% | 0.15% | 100% | 99% |
| Class F | 0.17% | 0.15% | 0.03% | 101% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.21% | -0.04% | 0.09% | 100% | 100% |
| Class C | 0.11% | 0.19% | 0.26% | 100% | 100% |
| Class E | 0.17% | 0.55% | -0.23% | 100% | 100% |
| Overall | 0.17% | 0.18% | 0.06% | 100% | 100% |
| Class D | 0.12% | 0.28% | 0.23% | 100% | 100% |
| Class F | 0.01% | -0.16% | -0.30% | 101% | 100% |

Conclusion

This contribution proposes two unification methods of using HMVP candidate in the both merge list and MPV list construction. 1) Same as AMVP, the HMVP candidates, from the first entry to the last entry in the HMVP table, are added in the merge list without redundant check. Also, 2) at most 5 HMVP candidates can be check in MVP list construction. The experimental results show that the proposed methods have marginal impact on the BD-rate performance while a unification of using HMVP candidates in the merge and MVP list is achieved. Therefore, it is recommended to adopt the proposed methods to the VVC WD5 and VTM-5.0.

References

[1] B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 4),” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-M1001, Jan. 2019.

[2] F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-M1010, Jan. 2019.

Patent rights declaration(s)

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

VVC WD4 (JVET-M1001-v6) changes

Derivation process for history-based merging candidates

Inputs to this process are:

- a merge candidate list mergeCandList,

- a variable isInSmr specifying whether the current coding unit is inside a shared mergign candidate region,

- the number of available merging candidates in the list numCurrMergeCand.

Outputs to this process are:

- the modified merging candidate list mergeCandList,

- the modified number of merging candidates in the list numCurrMergeCand.

The variables isPrunedA1 and isPrunedB1 are both set equal to FALSE.

The array smrHmvpCandList and the variable smrNumHmvpCand are derived as follows:

smrHmvpCandList = isInSmr  ?  HmvpSmrCandList  :  HmvpCandList	(8-353)

smrNumHmvpCand = isInSmr  ?  NumHmvpSmrCand  :  NumHmvpCand	(8-354)

For each candidate in smrHmvpCandList[ hMvpIdx ] with index hMvpIdx = 1..smrNumHmvpCand, the following ordered steps are repeated until numCurrMergeCand is equal to ( MaxNumMergeCand − 1):

- The variable sameMotion is derived as follows:

  - If all of the following conditions are true for any merging candidate N with N being A1 or B1, sameMotion and isPrunedN are both set equal to TRUE:

  - hMvpIdx is less than or equal to 2.

  - The candidate smrHmvpCandList[ smrNumHmvpCand − hMvpIdx] is equal to the merging candidate N.

  - isPrunedN is equal to FALSE.

  - Otherwise, sameMotion is set equal to FALSE.

- When sameMotion is equal to FALSE, the candidate smrHmvpCandList[ smrNumHmvpCand − hMvpIdx] is added to the merging candidate list as follows:

mergeCandList[ numCurrMergeCand++ ] = smrHmvpCandList[ smrNumHmvpCand − hMvpIdx hMvpIdx - 1] 	(8-355)

Derivation process for motion vector predictor candidate list

- When numCurrMvpCand is less than 2 and NumHmvpCand is greater than 0, the following applies for i= 1..Min( 4, NumHmvpCand ) until numCurrMvpCand is equal to 2:

- For each reference picture list LY with Y = X..( 1 − X ), the following applies until numCurrMvpCand is equal to 2:

- When the reference picture corresponding to the reference index of the history-based motion vector predictor candidate HmvpCandList[ i − 1 ] in the reference picture list LY is the same as the reference picture corresponding to reference index refIdxLX in the reference picture list LX, the following applies:

- The rounding process for motion vectors as specified in clause 8.5.2.14 is invoked with mvX set equal to the LY motion vector of the candidate HmvpCandList[ i − 1 ], rightShift set equal to MvShift + 2, and leftShift set equal to MvShift + 2 as inputs and the rounded LY motion vector of the candidate HmvpCandList[ i − 1 ] as output is assigned to mvpListLX[ numCurrMvpCand++ ].