| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0376_r1 |
| --- | --- |

| Title: | AHG16/CE3-related: CCLM mode restriction for increasing decoder throughput |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ji-Yeon Jung Yung-Lyul Lee Sejong University, 209 Neungdong-ro, Gwangjin-gu, Seoul, 05006, KOREA Dae-Yeon Kim Wook Je Jeong Chips&Media, 11/12/13F, V&S Tower, 26 Samseong-ro 85 gil, Gangnam-Gu, Seoul, 06194, KOREA Sung-Chang Lim Jungwon Kang ETRI, 218 Gajeong-ro, Yuseong-gu, Daejeon, 34129, KOREA | Tel: Email: | +82-42-860-5772 jyjung@sju.ac.kr yllee@sejong.ac.kr brad.kim@chipsnmedia.com nick.jeong@chipsnmedia.com sclim@etri.re.kr jungwon@etri.re.kr |
| Source: | Sejong University, Chips&Media, and Electronics and Telecommunications Research Institute (ETRI) |  |  |

_____________________________

- Abstract

This contribution proposes to restrict the usage of CCLM modes for small chroma intra blocks. In order to increase the throughput of the small chroma intra blocks in the hardware decoder, the proposed method prevents the small chroma intra blocks that are increasing the worst case processing cycles. In the proposed method, INTRA_LT_CCLM, INTRA_L_CCLM, and INTRA_T_CCLM modes are disallowed for the small chroma blocks having an area less than or equal to a predefined threshold. Compared to VTM-4.0, it is reported that the proposed method shows 0.01%/0.27%/0.27%, 0.00%/0.40%/0.41%, and -0.01%/0.29%/0.09% BD-rate impact on Y/Cb/Cr components for AI, RA, and LDB cases, respectively, in case of the threshold equal to 8. And, the proposed method shows 0.00%/0.03%/0.03%, 0.00%/0.06%/-0.02%, and 0.01%/0.19%/-0.06% BD-rate impact on Y/Cb/Cr components for AI, RA, and LDB cases, respectively, in case of the threshold equal to 4.

Problem Statement

As discussed in AHG16 on implementation studies, the decoding process of the chroma intra prediction for small blocks is not affordable in terms of VVC hardware decoder implementation. And, several proposals for improving the decoder throughput are included in CE3 tests [1]. Especially, the small blocks using CCLM modes could be a serious problem, because it can be estimated that non-CCLM mode block would consume the minimum 2 cycles to handle prediction and reconstruction process, whereas CCLM mode block would consume the minimum 3 cycles due to down-sampling and parameter derivation processes for one chroma block. Therefore, in order to increase the throughput of the small chroma intra blocks in the hardware decoder implementation, it is needed to restrict CCLM modes for the small chroma intra blocks.

Proposed method

To overcome the abovementioned problem, it is proposed that all CCLM modes, such as INTRA_LT_CCLM, INTRA_L_CCLM, and INTRA_T_CCLM are disallowed for the small chroma blocks having an area less than or equal to a predefined threshold. Since the proposed method is applied regardless of tree type, such as SINGLE_TREE or DUAL_TREE_CHROMA, the same binarization of the chroma intra prediction mode can be used for all tree types.

The proposed method can be described with the pseudo-code where the variables nTbW and nTbH mean the width and height of the chroma block, respectively, and MaxCclmRestrictionArea means a predefined threshold value as follows:

if (nTbW * nTbH <= MaxCclmRestrictionArea) or (sps_cclm_enabled_flag = = 0) INTRA_LT_CCLM, INTRA_L_CCLM, and INTRA_T_CCLM modes are disallowed. else INTRA_LT_CCLM, INTRA_L_CCLM, and INTRA_T_CCLM modes are allowed.

Experimental results

The experiment of the proposed method was performed based on VTM-4.0.1 software [2] with VTM configuration files under JVET common test condition [3]. And, the experiment was conducted on the platform of Intel Xeon E5-2690 v2 (3.0GHz) and 64GB RAM with GCC 7.3.1 compiler under CentOS Linux.

Table 1 shows the summary of two tests for the proposed CCLM mode restriction when MaxCclmRestrictionArea is set to 8 and 4, respectively.

Table 1. Summary of tests for the proposed chroma block size restrictions for CCLM modes

| Test # | Description | Disallowed chroma blocks for CCLM modes |
| --- | --- | --- |
| 1 | Disable CCLM modes for chroma blocks with nTbW * nTbH <= 8 (MaxCclmRestrictionArea = 8) | 2x2, 4x2, and 2x4 |
| 2 | Disable CCLM modes for chroma blocks with nTbW * nTbH <= 4 (MaxCclmRestrictionArea = 4) | 2x2 |

The experimental results of the proposed method compared to VTM-4.0 anchor are summarized in Table 2 and Table 3. Table 2 shows the experimental result of the proposed method where MaxCclmRestrictionArea is equal to 8. And, Table 3 shows the experimental result the proposed method where MaxCclmRestrictionArea is equal to 4. And, the detailed results for tests can be found in accompanied Excel files.

In the case of MaxCclmRestrictionArea equal to 8, the proposed method shows 0.01%/0.27%/0.27%, 0.00%/0.40%/0.41%, and -0.01%/0.29%/0.09% BD-rate impact on Y/Cb/Cr components for AI, RA, and LDB cases, respectively, as shown in Table 2.

Table 2. Experimental result of the test 1 (MaxCclmRestrictionArea = 8)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.16% | 0.13% | 100% | 100% |
| Class A2 | 0.02% | 0.27% | 0.16% | 100% | 100% |
| Class B | -0.01% | 0.13% | 0.18% | 100% | 100% |
| Class C | 0.03% | 0.70% | 0.83% | 100% | 100% |
| Class E | 0.00% | 0.04% | -0.06% | 100% | 101% |
| Overall | 0.01% | 0.27% | 0.27% | 100% | 100% |
| Class D | 0.05% | 0.84% | 1.07% | 100% | 99% |
| Class F | 0.09% | 0.79% | 0.99% | 100% | 99% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.01% | 0.15% | 101% | 101% |
| Class A2 | -0.02% | 0.18% | 0.09% | 100% | 101% |
| Class B | -0.01% | 0.26% | 0.20% | 100% | 100% |
| Class C | 0.01% | 1.04% | 1.09% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.40% | 0.41% | 100% | 101% |
| Class D | 0.02% | 1.00% | 1.35% | 100% | 100% |
| Class F | 0.03% | 0.73% | 0.97% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.17% | 0.08% | 100% | 100% |
| Class C | -0.01% | 0.77% | 0.42% | 100% | 100% |
| Class E | 0.01% | 0.41% | -0.33% | 100% | 100% |
| Overall | -0.01% | 0.29% | 0.09% | 100% | 100% |
| Class D | 0.04% | 0.74% | 0.01% | 100% | 100% |
| Class F | -0.03% | 1.13% | 0.66% | 100% | 100% |

In the case of MaxCclmRestrictionArea equal to 4, the proposed method shows 0.00%/0.03%/0.03%, 0.00%/0.06%/-0.02%, and 0.01%/0.19%/-0.06% BD-rate impact on Y/Cb/Cr components for AI, RA, and LDB cases, respectively, as shown in Table 3.

Table 3. Experimental result of the test 2 (MaxCclmRestrictionArea = 4)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.01% | 0.03% | 100% | 101% |
| Class A2 | 0.00% | 0.08% | 0.00% | 100% | 101% |
| Class B | 0.00% | -0.02% | 0.03% | 100% | 100% |
| Class C | 0.00% | 0.11% | 0.09% | 100% | 100% |
| Class E | -0.01% | -0.03% | -0.01% | 100% | 100% |
| Overall | 0.00% | 0.03% | 0.03% | 100% | 101% |
| Class D | 0.00% | 0.22% | 0.13% | 100% | 101% |
| Class F | 0.03% | 0.13% | 0.15% | 100% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.04% | 0.09% | 101% | 103% |
| Class A2 | -0.01% | 0.01% | 0.03% | 100% | 103% |
| Class B | -0.01% | 0.01% | -0.06% | 100% | 100% |
| Class C | 0.01% | 0.16% | -0.09% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.06% | -0.02% | 100% | 101% |
| Class D | 0.05% | 0.07% | 0.02% | 100% | 100% |
| Class F | -0.02% | 0.06% | 0.14% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.08% | -0.15% | 100% | 100% |
| Class C | -0.03% | 0.43% | 0.12% | 100% | 100% |
| Class E | 0.08% | 0.34% | -0.16% | 100% | 100% |
| Overall | 0.01% | 0.19% | -0.06% | 100% | 100% |
| Class D | 0.02% | 0.33% | 0.01% | 100% | 100% |
| Class F | -0.10% | -0.22% | 0.30% | 100% | 100% |

Conclusion

In this contribution, it is proposed that all CCLM modes are restricted for the small chroma blocks to reduce processing cycles at the worse case. From the experimental results, it is observed that the proposed method has a small impact on BD-rate. Therefore, it is suggested to adopt the proposed method into next version of VVC working draft.

References

- G. Van der Auwera, L. Li, and A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-M1023, Jan. 2019

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-4.0

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-M1010, Jan. 2019

Patent rights declaration(s)

Sejong University, Chips&Media, and ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

- Annex: Proposed draft text changes based on Draft 4 v6

- 9.5.3.7	Binarization process for intra_chroma_pred_mode

Input to this process is a request for a binarization for the syntax element intra_chroma_pred_mode.

Output of this process is the binarization of the syntax element.

The binarization for the syntax element intra_chroma_pred_mode is specified in Table 9-7 and Table 9-8.

…

Table 9-8 – Binarization for intra_chroma_pred_mode when sps_cclm_enabled_flag is equal to 1

| Value of intra_chroma_pred_mode | Bin string |  |
| --- | --- | --- |
|  | ( nTbW * nTbH ) > MaxCclmRestrictionArea | ( nTbW * nTbH ) <= MaxCclmRestrictionArea |
| 7 | 0 | 0 |
| 4 | 10 | - |
| 5 | 1110 | - |
| 6 | 1111 | - |
| 0 | 11000 | 100 |
| 1 | 11001 | 101 |
| 2 | 11010 | 110 |
| 3 | 11011 | 111 |

- 9.5.4.2	Derivation process for ctxTable, ctxIdx and bypassFlag

- 9.5.4.2.1	General

Input to this process is the position of the current bin within the bin string, binIdx.

Outputs of this process are ctxTable, ctxIdx and bypassFlag.

The values of ctxTable, ctxIdx and bypassFlag are derived as follows based on the entries for binIdx of the corresponding syntax element in Table 9-10:

…

| Table 9-10 – Assignment of ctxInc to syntax elements with context coded bins |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Syntax element | binIdx |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| end_of_tile_group_flag | terminate | na | na | na | na | na |
| alf_ctb_flag[ ][ ][ ] | 0..8 (clause 9.5.4.2.2) | na | na | na | na | na |
| sao_merge_left_flag | 0 | na | na | na | na | na |
| sao_merge_up_flag | 0 | na | na | na | na | na |
| sao_type_idx_luma | 0 | bypass | na | na | na | na |
| sao_type_idx_chroma | 0 | bypass | na | na | na | na |
| sao_offset_abs[ ][ ][ ][ ] | bypass | bypass | bypass | bypass | bypass | na |
| sao_offset_sign[ ][ ][ ][ ] | bypass | na | na | na | na | na |
| sao_band_position[ ][ ][ ] | bypass | bypass | bypass | bypass | bypass | bypass |
| sao_eo_class_luma | bypass | bypass | na | na | na | na |
| sao_eo_class_chroma | bypass | bypass | na | na | na | na |
| qt_split_cu_flag[ ][ ] | 0..5 (clause 9.5.4.2.2) | na | na | na | na | na |
| mtt_split_cu_flag | 0..12 (clause 9.5.4.2.2) | na | na | na | na | na |
| mtt_split_cu_vertical_flag | ( cbWidth = = cbHeight ) ? 0 : ( (cbWidth > cbHeight ) ? 1 : 2 ) | na | na | na | na | na |
| mtt_split_cu_binary_flag | 0 | na | na | na | na | na |
| cu_skip_flag[ ][ ] | 0,1,2 (clause 9.5.4.2.2) | na | na | na | na | na |
| pred_mode_flag | 0 | na | na | na | na | na |
| pcm_flag[ ][ ] | terminate | na | na | na | na | na |
| intra_luma_ref_idx[ ][ ] | 0 | 1 | 2 | na | na | na |
| intra_subpartitions_mode_flag | 0 | na | na | na | na | na |
| intra_subpartition_split_flag | 0 | na | na | na | na | na |
| intra_luma_mpm_flag[ ][ ] | 0 | na | na | na | na | na |
| intra_luma_mpm_idx[ ][ ] | bypass | bypass | bypass | bypass | bypass | na |
| intra_luma_mpm_remainder[ ][ ] | bypass | bypass | bypass | bypass | bypass | bypass |
| intra_chroma_pred_mode[ ][ ] sps_cclm_enabled_flag = = 0 | 0 | bypass | bypass | na | na | na |
| intra_chroma_pred_mode[ ][ ] sps_cclm_enabled_flag = = 1 && ( nTbW * nTbH ) <= MaxCclmRestrictionArea | 0 | bypass | bypass | na | na | na |
| intra_chroma_pred_mode[ ][ ] sps_cclm_enabled_flag = = 1 && bin at binIdx equal to 2 = = 0 && ( nTbW * nTbH ) > MaxCclmRestrictionArea | 0 | 1 | 2 | bypass | bypass | na |
| intra_chroma_pred_mode[ ][ ] sps_cclm_enabled_flag = = 1 && bin at binIdx equal to 2 = = 1 && ( nTbW * nTbH ) > MaxCclmRestrictionArea | 0 | 1 | 2 | 2 | na | na |
| merge_subblock_flag[ ][ ] | 0,1,2 (clause 9.5.4.2.2) | na | na | na | na | na |
| merge_subblock_idx[ ][ ] sps_sbtmvp_enabled_flag = = 0 | 0 | bypass | bypass | bypass | bypass | bypass |