| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0384-v2 |
| --- | --- |

| Title: | Non-CE8: IBC search range increase for small CTU size |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhong Xu Xiang Li Shan Liu | Tel: Email: | xiaozhongxu@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

In this contribution, a modification is made on intra block copy search range when the CTU size is smaller than 128x128. In this case, the size of local memory for IBC prediction is larger than one CTU size so more reference area can be stored in local memory and accessed for IBC prediction. It is asserted that by allowing fully utilzing the local memory for IBC prediction, coding efficiency improvement can be achieved. In the simulation, both anchor and tested methods are with the CTU size equal to 64x64.

In test A, N-1 left CTUs plus current CTU can be used for IBC prediction. In addition, parital data of left Nth CTU are available, the same availablity logic of VTM-4.0 applies to this CTU. Simulation results report that:

- -4.36%/-3.44% BD rate changes are observed for AI/RA configurations for class F when compared to VTM-4.0.1 anchor.

- -13.37%/-6.72% BD rate changes are observed for AI/RA configurations for class TGM when compared to VTM-4.0 anchor.

In test B, only N-1 left CTUs plus current CTU can be used for IBC prediction. The left Nth (left most) CTU in the reference area is marked not available. Simulation results report that:

- -4.34%/-3.33% BD rate changes are observed for AI/RA configurations for class F when compared to VTM-4.0.1 anchor.

- -13.16%/-6.55% BD rate changes are observed for AI/RA configurations for class TGM when compared to VTM-4.0 anchor.

# Problem Statement

Currently, intra block copy mode utilizes a 128x128 size local memory to store previously coded samples for IBC prediction. When the CTU size is equal to 128x128, this memory is one CTU size. In current VVC specification, the refernece area for IBC can be the already reconstructed part in current CTU plus some non-over written part of left CTU. This setup in the specification is fixed for 128x128 CTU size.

When the CTU size is smaller than 128x128, the same memory can store more than one CTU size of previously coded samples. However, the current VVC specification does not consider this fact and still alows the same range (current CTU plus some part of left CTU) to be used.

# Proposed method

In this proposal, the search range of IBC is adjusted for non 128x128 CTU cases such that the local memory can be fully utilized.

In particular, in addition to current CTU, the left N CTUs may be used as the reference area for IBC, where N is equal to maximum CTU area / current CTU area.

Proposed method 1: (test A), the left most CTU in this N (N>1) CTUs are partially available (only the non-overwritten part), according to the conformance conditions in the current specification.

Proposed method 2: ( test B), the left most CTU in this N (N>1) CTUs are not available for IBC prediction.

In both methods, the remaining N-1 CTUs to the left are full available given that they are inside the picture and in the same tile as current CTU.

In particular, if the CTU size is set to be 64x64, then 3 left CTUs are fully available in both tests. In test A, the 4th left CTU are partially available. In test B, the 4th left CTU are completely not available.

Fig. 1: 64x64 CTU and 4 left CTUs in search range for IBC prediction (Test A case)

# Simulation results

The proposed method is implemented on top of VTM-4.0. Simulation is performed using the common test condition as defined in JVET-M1010. One change made is that the CTU size is set to be 64x64. Table I summarize the results:

Table I: Test A: proposed method 1 vs anchor (CTU size=64x64)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM4.0-64CTU |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -4.36% | -4.36% | -4.20% | 106% | 100% |
| Class SCC 1080p | -13.37% | -13.47% | -13.52% | 96% | 93% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM4.0-64CTU |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -3.44% | -3.41% | -3.52% | 101% | 101% |
| Class SCC 1080p | -6.72% | -6.96% | -7.01% | 99% | 99% |

Table II: Test B: proposed method 2 vs anchor (CTU size=64x64)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM4.0-64CTU |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -4.34% | -4.35% | -4.24% | 103% | 99% |
| Class SCC 1080p | -13.16% | -13.24% | -13.24% | 96% | 94% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM4.0-64CTU |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -3.33% | -3.40% | -3.40% | 101% | 100% |
| Class SCC 1080p | -6.55% | -6.78% | -6.89% | 99% | 99% |

# Conclusion

In this proposal, the search range of IBC is adjusted to fully utilize the local memory for IBC prediction. In a non-CTC case where CTU size is equal to 64x64, coding performance improvement is observed. It is proposed to include this modification to the next VVC specification and VTM release.

# Specification Text

Based on JVET-M1001-v6.

### 8.6.2 Derivation process for motion vector components for IBC blocks

#### 8.6.2.1 General

Test A:

……

- The following conditions shall be true:

( yCb + ( mvL[ 1 ] >> 4 ) ) >> CtbLog2SizeY = yCb >> CtbLog2SizeY	(8-972)

( yCb + ( mvL[ 1 ] >> 4 ) + cbHeight − 1) >> CtbLog2SizeY = yCb >> CtbLog2SizeY	(8-973)

( xCb + ( mvL[ 0 ] >> 4 ) ) >> CtbLog2SizeY >= ( xCb >> CtbLog2SizeY ) – ( 1 << ( (7 – CtbLog2SizeY ) << 1) ) )	(8-974)

( xCb + ( mvL[ 0 ] >> 4 ) + cbWidth − 1) >> CtbLog2SizeY <= ( xCb >> CtbLog2SizeY )	 (8-975)

- When ( xCb + ( mvL[ 0 ] >> 4 ) ) >> CtbLog2SizeY is equal to ( xCb >> CtbLog2SizeY ) –( 1 << ( (7 – CtbLog2SizeY ) << 1) ) ), the derivation process for block availability as specified in clause 6.4.X is invoked with the current luma location( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( ( ( xCb + ( mvL[ 0 ] >> 4 ) + CtbSizeY ) >> ( CtbLog2SizeY − 1 ) ) << ( CtbLog2SizeY − 1 ), ( ( yCb + ( mvL[ 1 ] >> 4 ) ) >> ( CtbLog2SizeY − 1 ) ) << ( CtbLog2SizeY − 1 ) ) as inputs, and the output shall be equal to FALSE.

Test B:

……

- The following conditions shall be true:

( yCb + ( mvL[ 1 ] >> 4 ) ) >> CtbLog2SizeY = yCb >> CtbLog2SizeY	(8-972)

( yCb + ( mvL[ 1 ] >> 4 ) + cbHeight − 1) >> CtbLog2SizeY = yCb >> CtbLog2SizeY	(8-973)

( xCb + ( mvL[ 0 ] >> 4 ) ) >> CtbLog2SizeY >= ( xCb >> CtbLog2SizeY ) – ( 1 << ( (7 – CtbLog2SizeY ) << 1) ) )+ Min( 1, 7 – CtbLog2SizeY ) 	(8-974)

( xCb + ( mvL[ 0 ] >> 4 ) + cbWidth − 1) >> CtbLog2SizeY <= ( xCb >> CtbLog2SizeY )	 (8-975)

- When ( xCb + ( mvL[ 0 ] >> 4 ) ) >> CtbLog2SizeY is equal to ( xCb >> CtbLog2SizeY ) − 1 and CtbLog2SizeY is equal to 7, the derivation process for block availability as specified in clause 6.4.X is invoked with the current luma location( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( ( ( xCb + ( mvL[ 0 ] >> 4 ) + CtbSizeY ) >> ( CtbLog2SizeY − 1 ) ) << ( CtbLog2SizeY − 1 ), ( ( yCb + ( mvL[ 1 ] >> 4 ) ) >> ( CtbLog2SizeY − 1 ) ) << ( CtbLog2SizeY − 1 ) ) as inputs, and the output shall be equal to FALSE.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).