| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0385-v1 |
| --- | --- |

| Title: | CE4-related: modification of HMVP update process inside the shared merging candidate list region |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhong Xu Xiang Li Guichun Li Shan Liu | Tel: Email: | xiaozhongxu@tencent.com xlxiangli@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

In this contribution, the HMVP buffer update process is modified such that the HMVP buffer will not update with decoded motion or block vectors when the current block is inside a shared merging candidate list region. In other words, the decoded motion or block vector will not be put into the corresponding HMVP buffer for update if it is inside a shared merging candidate list region.

Simulation results report that:

- 0.00%/0.01%/0.01% BD rate changes are observed in AI/RA/LB configurations, for CTC average.

- -0.02%/0.02%/0.00% BD rate changes are observed in AI/RA/LB configurations, for class F.

# Problem Statement

In current VVC specification, when a region meets certain criteria, all the children nodes within this region will be considered as in a shared merging candidate list region, for which only one merge candidate list will be constructed.

As part of the merge candidate list construction process, entries in the HMVP buffer will be used to form the shared merge candidate list for all blocks inside this region. This property requires that the HMVP buffer status should be the same for all blocks to make sure the same merge candidate list can be constructed.

On the other hand, for blocks in the shared merging candidate list region but coded in non-merge mode, the decoded vectors will continue to update the HMVP buffer, changing the buffer status from block to block.

To solve the requirements of both coding pathes, a copy of HMVP buffer is generated at the beginning of each shared merging candidate list region so that blocks coded in merge mode will use this separate, unchanged HMVP buffer without interfering with the normal HMVP buffer. However, this doubles the memory size for storing the HMVP entries and complicates the decoding process.

# Proposed method

In this proposal, the HMVP buffer update process is modified such that the buffer will be updated only when a block is not inside a shared merging candidate list region. In this way, the duplicated copy of HMVP buffer is no longer needed and the motion vector derivation from HMVP buffer is done consistantly inside and outside the shared merging candidate list region.

# Simulation results

The proposed method is implemented on top of VTM-4.0.1. Simulation is performed using the common test condition as defined in JVET-M1010. Table I summarize the results:

Table I: disable HMVP update process inside the shared merging candidate region

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 102% | 102% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 102% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 104% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 99% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 104% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 102% |
| Class D | 0.00% | 0.00% | 0.00% | 98% | 93% |
| Class F | -0.02% | -0.21% | 0.00% | 101% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-4.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.05% | 0.04% | 101% | 101% |
| Class A2 | 0.00% | 0.10% | 0.00% | 101% | 102% |
| Class B | 0.01% | 0.04% | 0.08% | 101% | 103% |
| Class C | 0.01% | 0.03% | 0.15% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.05% | 0.07% | 100% | 101% |
| Class D | 0.00% | 0.09% | -0.32% | 99% | 97% |
| Class F | 0.02% | 0.03% | 0.09% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.23% | 0.25% | 101% | 103% |
| Class C | -0.02% | 0.10% | 0.03% | 102% | 106% |
| Class E | 0.04% | -0.37% | 0.28% | 101% | 106% |
| Overall | 0.01% | 0.03% | 0.18% | 101% | 104% |
| Class D | 0.01% | 0.16% | 0.37% | 99% | 101% |
| Class F | 0.00% | 0.31% | -0.12% | 100% | 102% |

# Conclusion

In this proposal, a simplified HMVP management method is proposed such that for the HMVP buffer does not update with vectors of decoded blocks from the shared merging candidate regions. By doing this, the HMVP update process is simplified and the extra copy for HMVP is unnecessary. Simulation results show no coding performance loss for the proposed method. It is recommended that this proposal be adopted into next VVC working draft and VTM software release.

# Specification Text

Based on JVET-M1001-v7.

#### 7.4.7.6 Coding unit semantics

When all the following conditions are true, the history-based motion vector predictor lists for the shared merging candidate list region are updated by setting NumHmvpSmrCand equal to the NumHmvpCand, and setting HmvpSmrCandList[ i ] equal to HmvpCandList[ i ] for i = 0..NumHmvpCand − 1 and by setting NumHmvpSmrIbcCand equal to the NumHmvpIbcCand, and setting HmvpSmrIbcCandList[ i ] equal to HmvpIbcCandList[ i ] for i = 0..NumHmvpIbcCand − 1:

- IsInSmr[ x0 ][ y0 ] is equal to TRUE

- SmrX[ x0 ][ y0 ]  is equal to x0

- SmrY[ x0 ][ y0 ]  is equal to y0

### 8.5.2	Derivation process for motion vector components and reference indices

#### 8.5.2.1	General

…

When IsInSmr[ xCb ][ yCb ] is equal to 0, The updating process for the history-based motion vector predictor list as specified in clause 8.5.2.16 is invoked with luma motion vectors mvL0[ 0 ][ 0 ] and mvL1[ 0 ][ 0 ], reference indices refIdxL0 and refIdxL1, prediction list utilization flags predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ], and bi-prediction weight index gbiIdx.

#### 8.5.2.2 Derivation process for luma motion vectors for merge mode

…

The variables xSmr, ySmr, smrWidth, smrHeight, and smrNumHmvpCand are derived as follows:

xSmr = IsInSmr[ xCb ][ yCb ]  ?  SmrX[ xCb ][ yCb ]  :  xCb	(8-276)

ySmr = IsInSmr[ xCb ][ yCb ]  ?  SmrY[ xCb ][ yCb ]  :  yCb	(8-277)

smrWidth = IsInSmr[ xCb ][ yCb ]  ?  SmrW[ xCb ][ yCb ]  :  cbWidth	(8-278)

smrHeight = IsInSmr[ xCb ][ yCb ]  ?  SmrH[ xCb ][ yCb ]  :  cbHeight	(8-279)

smrNumHmvpCand = IsInSmr[ xCb ][ yCb ]  ?  NumHmvpSmrCand  :  NumHmvpCand	(8-280)

…

- When numCurrMergeCand is less than (MaxNumMergeCand − 1) and smrNumHmvpCand is greater than 0, the following applies:

- The derivation process of history-based merging candidates as specified in 8.5.2.6 is invoked with mergeCandList, isInSmr set equal to IsInSmr[ xCb ][ yCb ], and numCurrMergeCand as inputs, and modified mergeCandList and numCurrMergeCand as outputs.

#### 8.5.2.6	Derivation process for history-based merging candidates

Inputs to this process are:

- a merge candidate list mergeCandList,

- a variable isInSmr specifying whether the current coding unit is inside a shared mergign candidate region,

- the number of available merging candidates in the list numCurrMergeCand.

Outputs to this process are:

- the modified merging candidate list mergeCandList,

- the modified number of merging candidates in the list numCurrMergeCand.

The variables isPrunedA1 and isPrunedB1 are both set equal to FALSE.

The array smrHmvpCandList and the variable smrNumHmvpCand are derived as follows:

smrHmvpCandList = isInSmr  ?  HmvpSmrCandList  :  HmvpCandList	(8-353)

smrNumHmvpCand = isInSmr  ?  NumHmvpSmrCand  :  NumHmvpCand	(8-354)

For each candidate in smrHmvpCandList[ hMvpIdx ] with index hMvpIdx = 1..smrNumHmvpCand, the following ordered steps are repeated until numCurrMergeCand is equal to ( MaxNumMergeCand − 1):

- The variable sameMotion is derived as follows:

  - If all of the following conditions are true for any merging candidate N with N being A1 or B1, sameMotion and isPrunedN are both set equal to TRUE:

  - hMvpIdx is less than or equal to 2.

  - The candidate smrHmvpCandList[ smrNumHmvpCand − hMvpIdx] is equal to the merging candidate N.

  - isPrunedN is equal to FALSE.

  - Otherwise, sameMotion is set equal to FALSE.

- When sameMotion is equal to FALSE, the candidate smrHmvpCandList[ smrNumHmvpCand − hMvpIdx] is added to the merging candidate list as follows:

mergeCandList[ numCurrMergeCand++ ] = smrHmvpCandList[ smrNumHmvpCand − hMvpIdx ] 	(8-355)

### 8.6.2 Derivation process for motion vector components for IBC blocks

#### 8.6.2.1	General

…

The luma motion vector mvL is derived as follows:

- If merge_flag[ xCb ][ yCb ] is equal to 1, t…

- …

…

When IsInSmr[ xCb ][ yCb ] is equal to 0, The updating process for the history-based motion vector predictor list as specified in clause 8.6.2.10 is invoked with luma motion vector mvL.

#### 8.6.2.2 Derivation process for luma motion vector for merge mode

…

The variables xSmr, ySmr, smrWidth, smrHeight, and smrNumHmvpIbcCand are derived as follows:

xSmr = IsInSmr[ xCb ][ yCb ]  ?  SmrX[ xCb ][ yCb ]  :  xCb	(8-276)

ySmr = IsInSmr[ xCb ][ yCb ]  ?  SmrY[ xCb ][ yCb ]  :  yCb	(8-277)

smrWidth = IsInSmr[ xCb ][ yCb ]  ?  SmrW[ xCb ][ yCb ]  :  cbWidth	(8-278)

smrHeight = IsInSmr[ xCb ][ yCb ]  ?  SmrH[ xCb ][ yCb ]  :  cbHeight	(8-279)

smrNumHmvpIbcCand = IsInSmr[ xCb ][ yCb ]  ?  NumHmvpSmrIbcCand  :  NumHmvpIbcCand	(8-280)

…

- When numCurrMergeCand is less than (MaxNumMergeCand − 1) and smrNumHmvpIbcCand is greater than 0, the derivation process of history-based merging candidates as specified in 8.6.2.5 is invoked with mergeMvCandList, isInSmr set equal to IsInSmr[ xCb ][ yCb ], and numCurrMergeCand as inputs, and modified mergeMvCandList and numCurrMergeCand as outputs.

#### 8.6.2.5 Derivation process for history-based merging candidates

Inputs to this process are:

- a merge candidate list mergeCandList,

- a variable isInSmr specifying whether the current coding unit is inside a shared mergign candidate region,

- the number of available merging candidates in the list numCurrMergeCand.

Outputs to this process are:

- the modified merging candidate list mergeCandList,

- the modified number of merging candidates in the list numCurrMergeCand.

The variables isPrunedA1 and isPrunedB1 are set both equal to FALSE.

The array smrHmvpIbcCandList and the variable smrNumHmvpIbcCand are derived as follows:

smrHmvpIbcCandList = isInSmr  ?  HmvpSmrIbcCandList  :  HmvpIbcCandList	(8-353)

smrNumHmvpIbcCand = isInSmr  ?  NumHmvpSmrIbcCand  :  NumHmvpIbcCand	(8-354)

For each candidate in smrHmvpIbcCandList[ hMvpIdx ] with index hMvpIdx = 1..smrNumHmvpIbcCand, the following ordered steps are repeated until numCurrMergeCand is equal to ( MaxNumMergeCand − 1):

- The variable sameMotion is derived as follows:

  - If all of the following conditions are true for any merging candidate N with N being A1 or B1, sameMotion and isPrunedN are both set equal to TRUE:

  - hMvpIdx is less than or equal to 2.

  - The candidate smrHmvpIbcCandList[smrNumHmvpIbcCand − hMvpIdx] is equal to the merging candidate N.

  - isPrunedN is equal to FALSE.

  - Otherwise, sameMotion is set equal to FALSE.

- When sameMotion is equal to FALSE, the candidate smrHmvpIbcCandList[smrNumHmvpIbcCand − hMvpIdx] is added to the merging candidate list as follows:

mergeCandList[ numCurrMergeCand++ ] = smrHmvpIbcCandList[ smrNumHmvpIbcCand − hMvpIdx ]

(8-987)

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).