| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 Mar. 2019 | Document: JVET-N0401-v4 |
| --- | --- |

| Title: | Non-CE3/Non-CE8: Enable Transform Skip in CUs using ISP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Santiago De-Luxán-Hernández, Tung Nguyen, Benjamin Bross, Heiko Schwarz, Detlev Marpe, Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49 30 31002-511 santiago.de.luxan@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution enables the transform skip mode on CUs using ISP at the TU and CU level. The experimental results are the following:

- Test 1: gains of 0.01 (CTC), 0.29% (class F) and 0.44% (TGM) for the AI configuration. The corresponding encoder run-times are 101% (CTC), 102% (class F) and 103% (TGM). In the case of RA, the gains are 0.02% (CTC), 0.20% (class F) and 0.22% (TGM) and the associated encoder run-times are 100% (CTC), 100% (class F) and 101% (TGM).

- Test 2: gains of 0.00% (CTC), 0.26% (class F) and 0.52% (TGM) for the AI configuration. The corresponding encoder run-times are 100% (CTC), 101% (class F) and 104% (TGM). In the case of RA, the gains are 0.00% (CTC), 0.17% (class F) and 0.33% (TGM) and the associated encoder run-times are 100% (CTC), 100% (class F) and 101% (TGM).

# Introduction

In the current design of the Intra Sub-Partitions (ISP) coding mode [1] Transform Skip (TS) is disabled in all CUs using ISP. The scope of this contribution consists in enabling TS in these CUs. Two tests are provided:

- Test 1: TS is enabled at the TU level. This means that there is a TS flag for each TU with a non-zero CBF within the CU using ISP. If the TS flag is 1, then that particular TU will use TS. Otherwise it will use the normal implicit transform of ISP.

- Test 2: TS in enabled at the CU level. This means that there is a single TS flag for the whole CU using ISP. If the TS flag is one, then all TUs having a non-zero CBF will use TS.

In both Test 1 and Test 2 the context use to signal the TS CU/TU level flag is the same as the one used by non-ISP blocks.

# Experimental results

According to the common test conditions [2], the proposed method is evaluated for intra-only (AI) and random-access (RA) configuration with the VTM-4.0.1 software. The corresponding simulations were conducted on an Intel Xeon cluster (E5-2697A v4, AVX2 on, turbo boost off) with Linux OS and a GCC 7.2.1 compiler. SCC TGM sequences have also been simulated.

## Test 1

Table I: Results for Test 1 using the AI configuration

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.01% | 0.06% | 101% | 99% |
| Class A2 | -0,01% | 0,01% | 0,00% | 100% | 100% |
| Class B | 0,00% | -0,01% | -0,05% | 100% | 101% |
| Class C | -0.04% | -0.13% | 0.07% | 101% | 100% |
| Class E | -0,01% | 0,01% | -0,02% | 100% | 100% |
| Overall | -0,01% | -0,03% | 0,01% | 101% | 100% |
| Class D | -0.03% | -0.03% | -0.01% | 102% | 102% |
| Class F | -0.29% | -0.35% | -0.19% | 102% | 100% |
| Class TGM | -0,44% | -0,31% | -0,33% | 103% | 101% |

Table II: Results for Test 1 using the RA configuration

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.10% | -0.09% | 100% | 100% |
| Class A2 | 0.00% | -0.05% | 0.00% | 100% | 101% |
| Class B | 0,01% | 0,11% | 0,00% | 100% | 100% |
| Class C | -0.04% | 0.14% | 0.22% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | 0.04% | 0.04% | 100% | 101% |
| Class D | -0.03% | -0.24% | -0.21% | 100% | 101% |
| Class F | -0,20% | -0,31% | -0,28% | 100% | 100% |
| Class TGM | -0,22% | -0,21% | -0,16% | 101% | 100% |

## Test 2

Table III: Results for Test 2 using the AI configuration

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,01% | 0,06% | 0,01% | 101% | 100% |
| Class A2 | -0,01% | 0,01% | -0,05% | 101% | 100% |
| Class B | 0,01% | -0,02% | -0,05% | 100% | 100% |
| Class C | 0,01% | -0,05% | -0,05% | 101% | 100% |
| Class E | -0,01% | -0,03% | 0,00% | 100% | 99% |
| Overall | 0,00% | -0,01% | -0,03% | 100% | 100% |
| Class D | -0,02% | -0,20% | -0,11% | 101% | 100% |
| Class F | -0,26% | -0,24% | -0,12% | 101% | 102% |
| Class TGM | -0.52% | -0.38% | -0.35% | 104% | 100% |

Table IV: Results for Test 2 using the RA configuration

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.01% | -0.11% | 100% | 100% |
| Class A2 | 0.00% | -0.01% | 0.00% | 100% | 101% |
| Class B | 0.02% | 0.23% | -0.06% | 101% | 100% |
| Class C | -0,02% | 0,18% | 0,13% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.12% | -0.01% | 100% | 100% |
| Class D | -0,02% | -0,01% | -0,33% | 100% | 101% |
| Class F | -0,17% | -0,22% | -0,21% | 100% | 100% |
| Class TGM | -0,33% | -0,24% | -0,23% | 101% | 100% |

# Conclusion

It is observed that both Test 1 and Test 2 are able to obtain a significant gain in classes F and TGM without having a negative impact on natural content sequences.

# References

| [1] | S. De-Luxán-Hernández, V. George, J. Ma, T. Nguyen, H. Schwarz, D. Marpe and T. Wiegand, "CE3: Intra Sub-Partitions Coding Mode (Tests 1.1.1 and 1.1.2)," in Document JVET-M0102, Marrakech, MA, 2019. |
| --- | --- |
| [2] | F. Bossen, J. Boyce, X. Li, V. Seregin and K. (. Sühring, "JVET common test conditions and software reference configurations for SDR video," Document JVET-M1010, Marrakech, MA, 2019. |

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).