| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0420-v2 |
| --- | --- |

| Title: | Non-CE6: A Simplification of Implicit Transform Selection in Intra-subblock Partitioning |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hilmi E. Egilmez, Adarsh K. Ramasubramonian, Amir Said, Nan Hu, Vadim Seregin, Geert Van der Auwera, Marta Karczewicz 5775 Morehouse Drive San Diego, CA, U.S.A. 92121 | Tel: Email: | hegilmez@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution document proposes to remove the mode dependency in the transform selection of intra-subblock partition (ISP) in VTM-4.0. Specifically, when ISP is applied, DST-7 is used to transform any row or column if the number of samples is less than 32 in a (luma) block, otherwise DCT-2 is used. A 2-point DST-7 transform is also introduced, since ISP subblocks may have rows/columns with 2 (luma) samples. It is asserted that the proposed changes simplify the transform selection process of ISP blocks. The experimental results over VTM-4.0 show that:

(i) the proposed method without 2-point DST-7 provides no coding loss, and

(ii) by further enabling 2-point DST-7, 0.04% AI, 0.02% RA and 0.02% LDB luma coding gains are achieved on average.

# Introduction

In VTM-4.0, when intra-subblock partitioning is used to code luma blocks, a mode-dependent transform selection is made, where the horizontal and vertical transforms (trTypeHor and trTypeVer) are derived based on the following table in the VVC specification draft, JVET-M1001-v6.

Table 8-15 – Specification of trTypeHor and trTypeVer depending on predModeIntra

| predModeIntra | trTypeHor | trTypeVer |
| --- | --- | --- |
| INTRA_PLANAR, INTRA_ANGULAR31, INTRA_ANGULAR32, INTRA_ANGULAR34, INTRA_ANGULAR36, INTRA_ANGULAR37 | ( nTbW >= 4 && nTbW <= 16 ) ? 1 : 0 | ( nTbH >= 4 && nTbH <= 16 ) ? 1 : 0 |
| INTRA_ANGULAR33, INTRA_ANGULAR35 | 0 | 0 |
| INTRA_ANGULAR2, INTRA_ANGULAR4,…,INTRA_ANGULAR28, INTRA_ANGULAR30, INTRA_ANGULAR39, INTRA_ANGULAR41,…,INTRA_ANGULAR63, INTRA_ANGULAR65 | ( nTbW >= 4 && nTbW <= 16 ) ? 1 : 0 | 0 |
| INTRA_ANGULAR3, INTRA_ANGULAR5,…, INTRA_ANGULAR27, INTRA_ANGULAR29, INTRA_ANGULAR38, INTRA_ANGULAR40,…,INTRA_ANGULAR64, INTRA_ANGULAR66 | 0 | ( nTbH >= 4 && nTbH <= 16 ) ? 1 : 0 |

This contribution proposes a simplification by removing the intra-mode dependent transform selection in ISP.

# Proposed method

The proposed implicit transform selection is derived based on the following simpler condition:

- For any row or column with less than or equal to 16 (luma) samples, DST-7 is used.

- Otherwise, DCT-2 is used to transform any row or column with number of (luma) samples greater than 16.

Based on above conditions, the proposed change can be reflected in JVET-M1001-v6 by replacing the Table 8-15 with the following:

trTypeHor = ( nTbW >= 2  &&  nTbW <= 16) ? 1 : 0

trTypeVer = ( nTbH >= 2  &&  nTbH <= 16 ) ? 1 : 0

where “0” and “1” denote DCT-2 and DST-7 respectively.

Since ISP can have rows/columns with two samples, 2-point DST-7 is also introduced. The entries of the 2-point DST-7 matrix is as follows:

{  48   77  }

{  77  -48  }

which only introduces 4-bytes of additonal memory.

# Experimental Results

The proposed method is tested over VTM-4.0 under CTC [1], and the results are presented in Table 1.

Table 1: Results of the proposed method over VTM-4.0 under CTC.

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.02% | -0.04% | 100% | 102% |
| Class A2 | -0.06% | -0.01% | -0.10% | 100% | 102% |
| Class B | -0.04% | -0.08% | -0.05% | 100% | 100% |
| Class C | -0.01% | -0.12% | -0.10% | 100% | 100% |
| Class E | -0.11% | -0.15% | -0.23% | 100% | 98% |
| Overall | -0.04% | -0.08% | -0.10% | 100% | 100% |
| Class D | -0.01% | -0.18% | -0.22% | 100% | 103% |
| Class F | 0.03% | -0.13% | -0.05% | 101% | 103% |
|  | Random Access |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.12% | -0.11% | 101% | 102% |
| Class A2 | -0.04% | 0.01% | 0.01% | 100% | 100% |
| Class B | 0.00% | -0.10% | -0.14% | 100% | 100% |
| Class C | -0.02% | -0.01% | -0.19% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.01% | -0.12% | 101% | 101% |
| Class D | 0.02% | -0.16% | -0.21% | 101% | 103% |
| Class F | 0.00% | -0.18% | -0.19% | 100% | 101% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | 0.04% | -0.21% | 100% | 102% |
| Class C | -0.01% | 0.28% | -0.06% | 100% | 103% |
| Class E | -0.02% | 0.99% | 1.07% | 100% | 103% |
| Overall | -0.02% | 0.36% | 0.16% | 100% | 102% |
| Class D | -0.02% | 1.10% | 0.03% | 100% | 99% |
| Class F | -0.10% | -0.25% | -0.29% | 100% | 101% |

Table 2 shows the results of applying the proposed simplification for rows/columns on more than 4 samples, so that 2-point DCT-2 is used for rows/columns with 2 samples as in the current ISP. The following results show that the proposed simplification does not diminish the coding efficiency.

Table 2: Results of the proposed simplification method without 2-point DST-7 over VTM-4.0 under CTC.

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.03% | 0.00% | 100% | 100% |
| Class A2 | -0.03% | 0.07% | -0.08% | 100% | 102% |
| Class B | -0.01% | 0.00% | 0.05% | 100% | 98% |
| Class C | 0.04% | -0.11% | -0.04% | 100% | 101% |
| Class E | -0.06% | -0.13% | -0.12% | 100% | 96% |
| Overall | -0.01% | -0.04% | -0.03% | 100% | 99% |
| Class D | 0.01% | -0.03% | -0.19% | 100% | 101% |
| Class F | 0.05% | -0.05% | -0.19% | 100% | 99% |
|  | Random Access |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.04% | 0.04% | 100% | 100% |
| Class A2 | -0.02% | 0.04% | -0.02% | 100% | 99% |
| Class B | 0.01% | -0.07% | -0.23% | 101% | 101% |
| Class C | 0.01% | -0.04% | -0.11% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.03% | -0.10% | 100% | 100% |
| Class D | 0.00% | -0.36% | -0.16% | 101% | 101% |
| Class F | 0.04% | -0.17% | -0.17% | 101% | 101% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.32% | -0.76% | 100% | 99% |
| Class C | -0.05% | 0.27% | -0.10% | 101% | 102% |
| Class E | 0.06% | -0.81% | 1.53% | 102% | 105% |
| Overall | -0.01% | -0.25% | 0.04% | 101% | 102% |
| Class D | 0.02% | 0.28% | 0.39% | 101% | 98% |
| Class F | -0.05% | -0.38% | 0.32% | 100% | 102% |

The proposed method is also tested over VTM-4.0.1 where a fast encoder method for ISP (i.e., ISPFast option) is enabled by default. Table 3 presents the AI coding results tested on VTM-4.0.1 and shows that the average AI coding gain difference be is 0.01% as compared to Table 1.

Table 3: All-intra results of the proposed method over VTM-4.0.1 under CTC.

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.04% | -0.12% | 100% | 101% |
| Class A2 | -0.06% | -0.06% | -0.11% | 100% | 100% |
| Class B | -0.03% | -0.05% | -0.07% | 100% | 98% |
| Class C | -0.02% | -0.10% | -0.11% | 100% | 101% |
| Class E | -0.11% | -0.18% | -0.26% | 101% | 100% |
| Overall | -0.05% | -0.08% | -0.13% | 100% | 100% |
| Class D | -0.04% | -0.04% | -0.02% | 101% | 101% |
| Class F | 0.01% | -0.14% | -0.08% | 100% | 100% |

# Proposed text for specification document

The proposed syntax changes for JVET-M1001 version 6 are as follows, where the changes are colored in red and the removed syntax elements are crossed out as red.

….

#### General

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable nTbW specifying the width of the current transform block,

- a variable nTbH specifying the height of the current transform block,

- a variable cIdx specifying the colour component of the current block,

- an (nTbW)x(nTbH) array d[ x ][ y ] of scaled transform coefficients with x = 0..nTbW − 1, y = 0..nTbH − 1.

Output of this process is the (nTbW)x(nTbH) array r[ x ][ y ] of residual samples with x = 0..nTbW − 1, y = 0..nTbH − 1.

The variable implicitMtsEnabled is derived as follows:

- If sps_mts_enabled_flag is equal to 1 and one of the following conditions is true, implicitMtsEnabled is set equal to 1:

- IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT

- cu_sbt_flag is equal to 1 and Max( nTbW, nTbH ) is less than or equal to 32

- sps_explicit_mts_intra_enabled_flag and sps_explicit_mts_inter_enabled_flag are both equal to 0 and CuPredMode[ xTbY ][ yTbY ] is equal to MODE_INTRA

- Otherwise, implicitMtsEnabled is set equal to 0.

The variable trTypeHor specifying the horizontal transform kernel and the variable trTypeVer specifying the vertical transform kernel are derived as follows:

- If cIdx is greater than 0, trTypeHor and trTypeVer are set equal to 0.

- Otherwise, if implicitMtsEnabled is equal to 1, the following applies:

- If IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT, trTypeHor and trTypeVer are derived as follows: specified in Table 8-15 depending on intraPredMode.

trTypeHor = ( nTbW >= 2   &&  nTbW <= 16 ) ? 1 : 0	(8-1029)

trTypeVer = ( nTbH >= 2   &&  nTbH <= 16 ) ?  1 : 0	(8-1030)

- Otherwise, if cu_sbt_flag is equal to 1, trTypeHor and trTypeVer are specified in Table 8-14 depending on cu_sbt_horizontal_flag and cu_sbt_pos_flag.

….

Table 8-15 – Specification of trTypeHor and trTypeVer depending on predModeIntra

| predModeIntra | trTypeHor | trTypeVer |
| --- | --- | --- |
| INTRA_PLANAR, INTRA_ANGULAR31, INTRA_ANGULAR32, INTRA_ANGULAR34, INTRA_ANGULAR36, INTRA_ANGULAR37 | ( nTbW >= 4 && nTbW <= 16 ) ? 1 : 0 | ( nTbH >= 4 && nTbH <= 16 ) ? 1 : 0 |
| INTRA_ANGULAR33, INTRA_ANGULAR35 | 0 | 0 |
| INTRA_ANGULAR2, INTRA_ANGULAR4,…,INTRA_ANGULAR28, INTRA_ANGULAR30, INTRA_ANGULAR39, INTRA_ANGULAR41,…,INTRA_ANGULAR63, INTRA_ANGULAR65 | ( nTbW >= 4 && nTbW <= 16 ) ? 1 : 0 | 0 |
| INTRA_ANGULAR3, INTRA_ANGULAR5,…, INTRA_ANGULAR27, INTRA_ANGULAR29, INTRA_ANGULAR38, INTRA_ANGULAR40,…,INTRA_ANGULAR64, INTRA_ANGULAR66 | 0 | ( nTbH >= 4 && nTbH <= 16 ) ? 1 : 0 |

…

- Otherwise, if trType is equal to 1 and nTbs is equal to 2, the following applies:

transMatrix[ m ][ n ] = 					(8-1042)

{

{  48   77  }

{  77  -48  }

},

- Otherwise, if trType is equal to 1 and nTbs is equal to 4, the following applies:

transMatrix[ m ][ n ] = 					(8-1043)

{

{  29   55   74   84 }

{  74   74    0  −74 }

{  84  −29  −74   55 }

{  55  −84   74  −29 }

},

- Otherwise, if trType is equal to 1 and nTbs is equal to 8, the following applies:

…

# Conclusions

In this contribution document, a simplification of the implicit transform selection in ISP is proposed. The proposed simplification removes mode dependency and provides about 0.04% all-intra coding gain. With the JVET-N0419, proposing the same implicit transform selection for MTS, this proposal can be viewed as a unification of both implicit transform selection schemes.

It is recommended to adopt the proposed method in VVC.

# Acknowledgements

Authors of this contribution document thank S. De-Luxán-Hernández (HHI) for cross-checking the experimental results in Table 1.

# References

- F. Bossen, J.Boyce, X. Li, V. Seregin and K. Suehring,  “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, 13th meeting, Marrakech, MA, Jan. 2019.

- S. De-Luxán-Hernández, V. George, J. Ma, T. Nguyen, H. Schwarz, D. Marpe, T. Wiegand, “CE3: Intra Sub-Partitions Coding Mode (Tests 1.1.1 and 1.1.2)” JVET-M0102, 13th meeting, Marrakech, MA, 9–18 Jan. 2019.

- J. Lainema, “CE6: Shape adaptive transform selection (Test 3.1)” JVET-M0303, 13th meeting, Marrakech, MA, 9–18 Jan. 2019.

- A. Said and X. Zhao, “Description of Core Experiment 6: Transforms and Transform signaling,” JVET-L1026, 12th meeting, Macao, CN, Oct. 2018.

- X. Zhao and H. E. Egilmez, “Description of Core Experiment 6: Transforms and Transform signaling,” JVET-M1026, 13th meeting, Marrakech, MA, Jan. 2019.

# Patent rights declaration

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).